OV3660

datasheet

PRELIMINARY SPECIFICATION

1/5" color CMOS QSXGA (3 megapixel) image sensor

with OmniBSI™ technology

Copyright © 2011 OmniVision Technologies, Inc. All rights reserved.

This document is provided “as is” with no warranties whatsoever, including any warranty of merchantability,

non-infringement, fitness for any particular purpose, or any warranty otherwise arising out of any proposal, specification, or sample.

OmniVision Technologies, Inc. and all its affiliates disclaim all liability, including liability for infringement of any proprietary rights, relating to the use of information in this document. No license, expressed or implied, by estoppel or otherwise, to any intellectual property rights is granted herein.

The information contained in this document is considered proprietary to OmniVision Technologies, Inc. and all its affiliates. This information may be distributed to individuals or organizations authorized by OmniVision Technologies, Inc. to receive said information. Individuals and/or organizations are not allowed to re-distribute said information.

###### Trademark Information

OmniVision and the OmniVision logo are registered trademarks of OmniVision Technologies, Inc. OmniBSI is a trademark of OmniVision Technologies, Inc.

All other trademarks used herein are the property of their respective owners.

color CMOS QSXGA (3 megapixel) image sensor with OmniBSI™ technology

datasheet (CSP3) PRELIMINARY SPECIFICATION

version 1.3

may 2011

To learn more about OmniVision Technologies, visit www.ovt.com.

OmniVision Technologies is publicly traded on NASDAQ under the symbol OVTI.

ordering information

 OV03660-A51A (color, lead-free) 51-pin CSP3

applications

  cellular phones

  toys

  PC multimedia

 digital still cameras

features

 1.4 µm x 1.4 µm pixel with OmniBSI technology for high performance (high sensitivity, low crosstalk, low noise, improved quantum efficiency)

 optical size of 1/5"

 automatic image control functions: automatic exposure control (AEC), automatic white balance (AWB), automatic flicker detection, and automatic black level calibration (ABLC)

 programmable controls for frame rate, AEC/AGC 16-zone size/position/weight control, mirror and flip, cropping, windowing, and panning

 image quality controls: color saturation, hue, gamma, sharpness (edge enhancement), lens correction, defective pixel canceling, and noise canceling

 support for output formats: RAW RGB, RGB565/555/444, CCIR656, YCbCr422, and

compression

 support for LED and flash strobe mode

key specifications (typical)

 active array size: 2048 x 1536

  power supply:

core: 1.5V ± 5% (with embedded 1.5V regulator) analog: 2.6 ~ 3.0V (2.8V typical)

I/O: 1.8V / 2.8V (1.8V recommended)

  power requirements:

active: 98 mA

standby: 20 µA

 temperature range:

operating: -20°C to 70°C junction temperature (see table 8-2)

stable image: 0°C to 50°C junction temperature (see table 8-2)

 output formats: 8-/10-bit RAW, RGB and YCbCr output, compression

  lens size: 1/5"

  lens chief ray angle: 27.6° (see figure 10-2)

  input clock frequency: 6~27 MHz

 support for horizontal and vertical sub-sampling, binning

 support 2x2 binning with binning filter to minimize binning artifacts

 support for data compression output

 standard serial SCCB interface (see section 2.10 for details)

 digital video port (DVP) parallel output interface

  embedded 1.5V regulator for core power

 programmable I/O drive capability, I/O tri-state configurability

 support for black sun cancellation

 support for images sizes: 3 megapixel and any arbitrary size scaling down from 3 megapixel

 suitable for module size of 6.5 x 6.5 x <6mm

note	pixel performance shown are target values. These values are subject to

change based on real measurements.

 max S/N ratio: 34 dB

 dynamic range: 70 dB @ 8x gain

 maximum image transfer rate:

2048x1536: 15 fps

1080p: 20 fps

720p: 45 fps

XGA (1024x768): 45 fps VGA (640x480): 60 fps QVGA (320x240): 120 fps

  sensitivity: 670 mV/Lux-sec

 shutter: rolling shutter

 maximum exposure interval: 1560 x tROW

 pixel size: 1.4 µm x 1.4 µm

 dark current: TBD

 image area: 2912 µm x 2167.2 µm

  package dimensions: 5010 µm x 4960 µm

table of contents

| 1 | signal descriptions | 1-1 |
| --- | --- | --- |
| 2 | system level description | 2-1 |
|  | 2.1 overview | 2-1 |
|  | 2.2 architecture | 2-1 |
|  | 2.3 pixel array structure | 2-4 |
|  | 2.3.1 binning | 2-5 |
|  | 2.4 power up sequence | 2-5 |
|  | 2.4.1 power up with internal DVDD | 2-6 |
|  | 2.4.2 power up with external DVDD source | 2-7 |
|  | 2.5 reset | 2-8 |
|  | 2.5.1 power ON reset generation | 2-8 |
|  | 2.6 hardware and software standby | 2-8 |
|  | 2.6.1 hardware standby | 2-8 |
|  | 2.6.2 software standby | 2-8 |
|  | 2.7 format and frame rate | 2-9 |
|  | 2.7.1 output format control | 2-9 |
|  | 2.8 I/O control | 2-14 |
|  | 2.9 system control | 2-16 |
|  | 2.9.1 system clock control | 2-19 |
|  | 2.10 group write | 2-22 |
| 3 | image sensor core digital functions | 3-1 |
|  | 3.1 mirror and flip | 3-1 |
|  | 3.2 image windowing and scaling | 3-2 |
|  | 3.2.1 image windowing | 3-2 |
|  | 3.2.2 scaling | 3-4 |
|  | 3.3 test pattern | 3-7 |
|  | 3.4 exposure / gain control | 3-8 |
|  | 3.4.1 overview | 3-8 |
|  | 3.4.2 automatic exposure/gain control | 3-8 |
|  | 3.4.3 50Hz/60Hz detection | 3-12 |
|  | 3.4.4 anti-flicker | 3-12 |
|  | 3.4.5 extending the exposure time under dark conditions | 3-13 |
|  | 3.4.6 sub-row exposure time under extremely bright conditions | 3-14 |

|  | 3.5 black level calibration (BLC) | 3-15 |
| --- | --- | --- |
|  | 3.6 strobe flash | 3-16 |
|  | 3.6.1 strobe flash control | 3-16 |
|  | 3.7 one time programmable (OTP) memory | 3-19 |
|  | 3.7.1 OTP write sequence | 3-19 |
|  | 3.7.2 OTP read sequence | 3-19 |
|  | 3.8 temperature sensor | 3-21 |
| 4 | image sensor processor digital functions | 5-1 |
|  | 4.1 ISP general controls | 5-1 |
|  | 4.2 lens correction (LENC) | 5-4 |
|  | 4.3 white balance | 5-6 |
|  | 4.3.1 simple AWB | 5-6 |
|  | 4.3.2 advanced AWB | 5-6 |
|  | 4.3.3 AWB stable range and gain range | 5-11 |
|  | 4.4 gamma | 5-12 |
|  | 4.5 defect pixel cancellation (DPC) | 5-14 |
|  | 4.6 sharpness control and de-noise | 5-14 |
|  | 4.7 color matrix (CMX) | 5-18 |
|  | 4.8 auto color saturation adjust | 5-19 |
|  | 4.8.1 manual mode | 5-19 |
|  | 4.8.2 auto mode | 5-20 |
|  | 4.9 special digital effects (SDE) | 5-21 |
| 5 | image sensor output interface digital functions | 6-1 |
|  | 5.1 compression engine | 6-1 |
|  | 5.1.1 compression mode 1 timing | 6-1 |
|  | 5.1.2 compression mode 2 timing | 6-1 |
|  | 5.1.3 compression mode 3 timing | 6-2 |
|  | 5.1.4 compression mode 4 timing | 6-2 |
|  | 5.1.5 compression mode 5 timing | 6-3 |
|  | 5.1.6 compression mode 6 timing | 6-3 |
|  | 5.1.7 compression mode control | 6-4 |
|  | 5.2 parallel digital video port (DVP) | 6-6 |
|  | 5.2.1 overview | 6-6 |
|  | 5.2.2 DVP timing | 6-9 |
| 6 | serial camera control bus (SCCB) interface | 7-1 |
|  | 6.1 data transfer protocol | 7-1 |

|  | 6.2 message format | 7-1 |
| --- | --- | --- |
|  | 6.3 read / write operation | 7-1 |
|  | 6.4 SCCB timing | 7-4 |
| 7 | register tables | 8-1 |
|  | 7.1 system and IO pad control [0x3000 ~ 0x3052] | 8-1 |
|  | 7.2 SCCB control [0x3100 ~ 0x3108] | 8-4 |
|  | 7.3 SRB control [0x3200 ~ 0x3213] | 8-5 |
|  | 7.4 AWB gain control [0x3400 ~ 0x3406] | 8-5 |
|  | 7.5 AEC/AGC control [0x3500 ~ 0x3513] | 8-6 |
|  | 7.6 ANA control registers [0x3600 ~ 0x3634] | 8-7 |
|  | 7.7 sensor control [0x3700 ~ 0x373C] | 8-8 |
|  | 7.8 timing control [0x3800 ~ 0x3836] | 8-8 |
|  | 7.9 AEC/AGC power down domain control [0x3A00 ~ 0x3A25] | 8-10 |
|  | 7.10 strobe control [0x3B00] | 8-13 |
|  | 7.11 sigmadelta/5060Hz detector [0x3C00 ~ 0x3C1E] | 8-13 |
|  | 7.12 OTP control [0x3D00 ~ 0x3D21] | 8-15 |
|  | 7.13 BLC control [0x4000 ~ 0x4033] | 8-16 |
|  | 7.14 frame control [0x4201 ~ 0x4202] | 8-18 |
|  | 7.15 format control [0x4300 ~ 0x430D] | 8-19 |
|  | 7.16 compression control [0x4400 ~ 0x4458] | 8-23 |
|  | 7.17 VFIFO control [0x4600 ~ 0x460D] | 8-26 |
|  | 7.18 DVP control [0x4709 ~ 0x4745] | 8-27 |
|  | 7.19 ISP frame control [0x4901 ~ 0x4902] | 8-30 |
|  | 7.20 ISP top control [0x5000 ~ 0x5063] | 8-30 |
|  | 7.21 AWB control [0x5180 ~ 0x51D0] | 8-33 |
|  | 7.22 CIP control [0x5300 ~ 0x530F] | 8-35 |
|  | 7.23 CMX control [0x5380 ~ 0x538B] | 8-36 |
|  | 7.24 gamma control [0x5480 ~ 0x5490] | 8-37 |
|  | 7.25 SDE control [0x5580 ~ 0x558C] | 8-38 |
|  | 7.26 scale control [0x5600 ~ 0x5606] | 8-40 |
|  | 7.27 average control [0x5680 ~ 0x56A2] | 8-41 |
|  | 7.28 DPC control [0x5780 ~ 0x5794] | 8-43 |
|  | 7.29 LENC control [0x5800 ~ 0x5849] | 8-44 |
|  | 7.30 temperature sensor control [0x6700 ~ 0x6721] | 8-49 |
| 8 | operating specifications | 9-1 |

  - absolute maximum ratings	9-1

  - functional temperature	9-1

  - DC characteristics	9-2

  - timing characteristics	9-4

- mechanical specifications	10-1

  - physical specifications	10-1

  - IR reflow specifications	10-2

- optical specifications	11-1

  - lens chief ray angle (CRA)

  - spectrum response

sensor array center	11-1

###### 11-2

###### 11-3

list of figures

| figure 1-1 | pin diagram | 1-4 |
| --- | --- | --- |
| figure 2-1 | OV3660 block diagram | 2-2 |
| figure 2-2 | reference design schematic | 2-3 |
| figure 2-3 | sensor array region color filter layout | 2-4 |
| figure 2-4 | example of 2x2 binning | 2-5 |
| figure 2-5 | power up timing with internal DVDD | 2-6 |
| figure 2-6 | power up timing with external DVDD source | 2-7 |
| figure 2-7 | PLL block diagram | 2-20 |
| figure 3-1 | mirror and flip samples | 3-1 |
| figure 3-2 | image windowing | 3-2 |
| figure 3-3 | image windowing configuration | 3-3 |
| figure 3-4 | scaling function | 3-4 |
| figure 3-5 | test pattern | 3-7 |
| figure 3-6 | average-based window definition | 3-9 |
| figure 3-7 | xenon flash mode | 3-16 |
| figure 3-8 | LED 1 & 2 mode - one pulse output | 3-17 |
| figure 3-9 | LED 1 & 2 mode - multiple pulse output | 3-17 |
| figure 3-10 | LED 3 mode | 3-18 |
| figure 4-1 | control points of luminance and color channels | 5-4 |
| figure 4-2 | luminance compensation level calculation | 5-4 |
| figure 4-3 | gamma curve | 5-12 |
| figure 4-4 | auto color saturation adjust graph | 5-20 |
| figure 5-1 | compression mode 1 timing | 6-1 |
| figure 5-2 | compression mode 2 timing | 6-1 |
| figure 5-3 | compression mode 3 timing | 6-2 |
| figure 5-4 | compression mode 4 timing | 6-2 |
| figure 5-5 | compression mode 5 timing | 6-3 |
| figure 5-6 | compression mode 6 timing | 6-3 |
| figure 5-7 | DVP frame timing diagram | 6-9 |
| figure 6-1 | message type | 7-1 |
| figure 6-2 | SCCB single read from random location | 7-2 |
| figure 6-3 | SCCB single read from current location | 7-2 |

| figure 6-4 | SCCB sequential read from random location | 7-2 |
| --- | --- | --- |
| figure 6-5 | SCCB sequential read from current location | 7-3 |
| figure 6-6 | SCCB single write to random location | 7-3 |
| figure 6-7 | SCCB sequential write to random location | 7-3 |
| figure 6-8 | SCCB interface timing | 7-4 |
| figure 9-1 | package specifications | 10-1 |
| figure 9-2 | IR reflow ramp rate requirements | 10-2 |
| figure 10-1 | sensor array center | 11-1 |
| figure 10-2 | chief ray angle (CRA) | 11-2 |
| figure 10-3 | spectrum response graph | 11-3 |

list of tables

| table 1-1 | signal descriptions | 1-1 |
| --- | --- | --- |
| table 1-2 | configuration under various conditions | 1-3 |
| table 1-3 | pad symbol and equivalent circuit | 1-5 |
| table 2-1 | binning-related registers | 2-5 |
| table 2-2 | format and frame rate | 2-9 |
| table 2-3 | output format control registers | 2-9 |
| table 2-4 | format control registers | 2-10 |
| table 2-5 | driving capability and direction control for I/O pads | 2-14 |
| table 2-6 | system control registers | 2-16 |
| table 2-7 | PCLK divider ratio | 2-19 |
| table 2-8 | PLL configurations | 2-20 |
| table 2-9 | PLL control registers | 2-21 |
| table 2-10 | group write registers | 2-22 |
| table 3-1 | mirror and flip registers | 3-1 |
| table 3-2 | image windowing registers | 3-3 |
| table 3-3 | scaling control registers | 3-5 |
| table 3-4 | test pattern selection control | 3-7 |
| table 3-5 | timing control functions | 3-9 |
| table 3-6 | night mode control registers | 3-14 |
| table 3-7 | BLC control functions | 3-15 |
| table 3-8 | flashlight modes | 3-16 |
| table 3-9 | strobe control functions | 3-18 |
| table 3-10 | OTP control functions | 3-20 |
| table 3-11 | temperature sensor function | 3-21 |
| table 4-1 | ISP general control registers | 5-1 |
| table 4-2 | LENC control registers | 5-5 |
| table 4-3 | white balance registers | 5-6 |
| table 4-4 | AWB calibration registers | 5-7 |
| table 4-5 | AWB control | 5-9 |
| table 4-6 | AWB stable range registers | 5-11 |
| table 4-7 | gamma parameters and corresponding registers | 5-12 |
| table 4-8 | gamma control registers | 5-13 |

| table 4-9 | DPC control registers | 5-14 |
| --- | --- | --- |
| table 4-10 | sharpness and de-noise control registers | 5-17 |
| table 4-11 | CMX control registers | 5-19 |
| table 4-12 | SDE image effects | 5-21 |
| table 4-13 | image contrast/brightness/saturation/hue control | 5-23 |
| table 4-14 | SDE control registers | 5-24 |
| table 5-1 | compression control registers | 6-4 |
| table 5-2 | parallel DVP control registers | 6-6 |
| table 5-3 | DVP timing specifications | 6-9 |
| table 6-1 | SCCB interface timing specifications | 7-4 |
| table 7-1 | system and IO pad control registers | 8-1 |
| table 7-2 | SCCB control registers | 8-4 |
| table 7-3 | SRB control registers | 8-5 |
| table 7-4 | AWB gain control registers | 8-5 |
| table 7-5 | AEC/AGC control functions | 8-6 |
| table 7-6 | ANA control registers | 8-7 |
| table 7-7 | sensor control registers | 8-8 |
| table 7-8 | timing control registers | 8-8 |
| table 7-9 | AEC/AGC power down domain control registers | 8-10 |
| table 7-10 | strobe registers | 8-13 |
| table 7-11 | sigmadelta/5060HZ detector registers | 8-13 |
| table 7-12 | OTP control functions | 8-15 |
| table 7-13 | BLC registers | 8-16 |
| table 7-14 | frame control registers | 8-18 |
| table 7-15 | format control registers | 8-19 |
| table 7-16 | compression control registers | 8-23 |
| table 7-17 | VFIFO registers | 8-26 |
| table 7-18 | DVP control registers | 8-27 |
| table 7-19 | ISP frame control registers | 8-30 |
| table 7-20 | ISP top control registers | 8-30 |
| table 7-21 | AWB registers | 8-33 |
| table 7-22 | CIP control registers | 8-35 |
| table 7-23 | CMX control registers | 8-36 |
| table 7-24 | gamma control registers | 8-37 |
| table 7-25 | SDE control registers | 8-38 |

| table 7-26 | scale control registers | 8-40 |
| --- | --- | --- |
| table 7-27 | AVG registers | 8-41 |
| table 7-28 | DPC control registers | 8-43 |
| table 7-29 | LENC control registers | 8-44 |
| table 7-30 | TPM control registers | 8-49 |
| table 8-1 | absolute maximum ratings | 9-1 |
| table 8-2 | functional temperature | 9-1 |
| table 8-3 | DC characteristics (-20°C < TJ < 70°C) | 9-2 |
| table 8-4 | timing characteristics | 9-4 |
| table 9-1 | package dimensions | 10-1 |
| table 9-2 | reflow conditions | 10-2 |
| table 10-1 | CRA versus image height plot | 11-2 |

- signal descriptions

table 1-1 lists the signal descriptions and their corresponding pin numbers for the OV3660 image sensor. The package information is shown in section 9.

##### table 1-1	signal descriptions (sheet 1 of 2)

| pin number | signal name | pin type | description |
| --- | --- | --- | --- |

| A1 | DVDD | power | power for digital circuit |
| --- | --- | --- | --- |
| A2 | DOGND | ground | ground for digital circuit |
| A3 | NC | – | no connect |
| A4 | NC | – | no connect |
| A5 | NC | – | no connect |
| A6 | NC | – | no connect |
| A7 | VNH | reference | internal analog reference |
| A8 | VH | reference | internal analog reference |
| B1 | HREF | I/O | video output horizontal sync signal |
| B2 | XVCLK | input | system input clock/scan clock input |
| B3 | DOVDD | power | power for I/O circuit |
| B6 | VNL | reference | internal analog reference |
| B7 | AGND | ground | ground for analog circuit |
| B8 | AGND | ground | ground for analog circuit |
| C1 | D8 | I/O | image data output 8 |
| C2 | D9 | I/O | image data output 9 |
| C3 | NC | – | no connect |
| C6 | RESETB | input | reset (active low with internal pull up resistor) |
| C7 | AVDD | power | power for analog circuit |
| C8 | AVDD | power | power for analog circuit |
| D1 | D6 | I/O | image data output 6 |
| D2 | D7 | I/O | image data output 7 |
| D3 | DOGND | ground | ground for digital circuit |
| D6 | DOGND | ground | ground for digital circuit |
| D7 | DOGND | ground | ground for digital circuit |
| D8 | DOGND | ground | ground for digital circuit |

##### table 1-1	signal descriptions (sheet 2 of 2)

| pin number | signal name | pin type | description |
| --- | --- | --- | --- |

| E1 | DVDD | power | power for digital circuit |
| --- | --- | --- | --- |
| E2 | D5 | I/O | image data output 5 |
| E3 | FSIN | I/O | frame sync |
| E6 | DOVDD | power | power for I/O circuit |
| E7 | DOVDD | power | power for I/O circuit |
| E8 | DOVDD | power | power for I/O circuit |
| F1 | PCLK | I/O | image output clock |
| F2 | D4 | I/O | image data output 4 |
| F3 | DOVDD | power | power for I/O circuit |
| F6 | PWDN | input | power down (active high with pull down resistor) |
| F7 | DVDD | power | power for digital circuit |
| F8 | DVDD | power | power for digital circuit |
| G1 | D2 | I/O | image data output 2 |
| G2 | D3 | I/O | image data output 3 |
| G3 | DVDD | power | power for digital circuit |
| G6 | STROBE | I/O | LED/flash control |
| G7 | TM | input | test mode (active high with pull down resistor) |
| G8 | VSYNC | I/O | video output vertical sync signal |
| H1 | D0 | I/O | image data output 0 |
| H2 | D1 | I/O | image data output 1 |
| H3 | DOGND | ground | ground for digital circuit |
| H4 | NC | – | no connect |
| H6 | NC | – | no connect |
| H7 | SIOC | input | SCCB input clock |
| H8 | SIOD | I/O | SCCB data |

##### table 1-2	configuration under various conditions

| pin | signal name | RESET | after RESET release | software standby | hardware standby (PWDNB = 0) |
| --- | --- | --- | --- | --- | --- |

| B1 | HREF | high-z |
| --- | --- | --- |
| B2 | XVCLK | input |
| C1 | D8 | high-z |
| C2 | D9 | high-z |
| C6 | RESETB | input |
| D1 | D6 | high-z |
| D2 | D7 | high-z |
| E2 | D5 | high-z |
| E3 | FSIN | input |
| F1 | PCLK | high-z |
| F2 | D4 | high-z |
| F6 | PWDN | input |
| G1 | D2 | high-z |
| G2 | D3 | high-z |
| G6 | STROBE | high-z |
| G7 | TM | input |
| G8 | VSYNC | high-z |
| H1 | D0 | high-z |
| H2 | D1 | high-z |
| H7 | SIOC | input |
| H8 | SIOD | input |

high-z by default (configurable)

high-z by default (configurable)

high-z by default (configurable)

input	input	high-z

| high-z by default | high-z by default | high-z by default |
| --- | --- | --- |
| (configurable) | (configurable) | (configurable) |
| high-z by default | high-z by default | high-z by default |
| (configurable) | (configurable) | (configurable) |
| input | input | input |
| high-z by default | high-z by default | high-z by default |
| (configurable) | (configurable) | (configurable) |
| high-z by default | high-z by default | high-z by default |
| (configurable) | (configurable) | (configurable) |
| high-z by default | high-z by default | high-z by default |
| (configurable) | (configurable) | (configurable) |
| input by default | high-z by default | high-z by default |
| (configurable) | (configurable) | (configurable) |
| high-z by default | high-z by default | high-z by default |
| (configurable) | (configurable) | (configurable) |
| high-z by default | high-z by default | high-z by default |
| (configurable) | (configurable) | (configurable) |
| input | input | input |
| high-z by default | high-z by default | high-z by default |
| (configurable) | (configurable) | (configurable) |
| high-z by default | high-z by default | high-z by default |
| (configurable) | (configurable) | (configurable) |
| high-z by default | high-z by default | high-z by default |
| (configurable) | (configurable) | (configurable) |
| input | input | input |
| high-z by default | high-z by default | high-z by default |
| (configurable) | (configurable) | (configurable) |
| high-z by default | high-z by default | high-z by default |
| (configurable) | (configurable) | (configurable) |
| high-z by default | high-z by default | high-z by default |
| (configurable) | (configurable) | (configurable) |
| input | input | high-z |
| input | input | high-z |

##### figure 1-1	pin diagram

A1

DVDD

A2

DOGND

A3

NC

A4

NC

A5

NC

A6

NC

A7

VNH

A8

VH

B1

HREF

B2

XVCLK

B3

DOVDD

B6

VNL

B7

AGND

B8

AGND

C1

D8

C2

D9

C3

NC

C6

RESETB

C7

AVDD

C8

AVDD

D1

D6

D2

D7

D3

D6

DOGND

OV3660

D7	D8

DOGND	DOGND	DOGND

E1

DVDD

E2

D5

E3

FSIN

E6	E7	E8

DOVDD	DOVDD	DOVDD

F1

PCLK

F2

D4

F3

DOVDD

F6

PWDN

F7

DVDD

F8

DVDD

G1

D2

G2

D3

G3

DVDD

G6

STROBE

G7

TM

G8

VSYNC

H1

D0

H2

D1

H3

DOGND

H4

NC

H6

NC

H7

SIOC

H8 SIOD

3660_CSP_DS_1_1

top view

##### table 1-3	pad symbol and equivalent circuit

equivalent circuit

symbol

EN

PAD

XVCLK

DOGND

PAD

to core

SIOD

from core

PD

DOGND

open-drain

PAD

SIOC

PD

DOGND

DOVDD

DOUT

D9, D8, D7, D6, D5, D4, D3, D2, D1, D0, VSYNC, STROBE, FSIN

PAD

EN

DIN

DOGND

PD

PAD

AGND, DOGND, DGND

DOGND

PAD

AVDD, DVDD, DOVDD

DOGND

DOVDD DOVDD

DOGND

RESETB

PAD

DOGND

DOVDD

PAD

TM, PWDN

DOGND

- system level description

  - overview

The OV3660 (color) image sensor is a low voltage, high-performance, 1/5-inch 3 megapixel CMOS image sensor that provides the full functionality of a single chip 3 megapixel (2048x1536) camera using OmniBSI™ technology in a small footprint package. It provides full-frame, sub-sampled, windowed or arbitrarily scaled 8-bit/10-bit images in various formats via the control of the Serial Camera Control Bus (SCCB) interface.

The OV3660 has an image array capable of operating at up to 15 frames per second (fps) in 3 megapixel resolution with complete user control over image quality, formatting and output data transfer. All required image processing functions, including exposure control, gamma, white balance, color saturation, hue control, defective pixel canceling, noise canceling, etc., are programmable through the SCCB interface. The OV3660 also includes a compression engine for increased processing power. In addition, OmniVision image sensors use proprietary sensor technology to improve image quality by reducing or eliminating common lighting/electrical sources of image contamination, such as fixed pattern noise, smearing, etc., to produce a clean, fully stable, color image.

For identification and storage purposes, the OV3660 includes a one-time programmable (OTP) memory. The OV3660 supports a digital video parallel port.

  - architecture

The OV3660 sensor core generates streaming pixel data at a constant frame rate, indicated by HREF and VSYNC.

figure 2-1 shows the functional block diagram of the OV3660 image sensor.

The timing generator outputs signals to access the rows of the image array, precharging and sampling the rows of the array in series. In the time between pre-charging and sampling a row, the charge in the pixels decreases with the time exposed to the incident light. This is known as exposure time.

The exposure time is controlled by adjusting the time interval between precharging and sampling. After the data of the pixels in the row has been sampled, it is processed through analog circuitry to correct the offset and multiply the data with corresponding gain. Following analog processing is the ADC which outputs 10-bit data for each pixel in the array.

XVCLK

PWDN RESETB STROBE

FSIN PCLK HREF

VSYNC

SIOC

SIOD

##### figure 2-1	OV3660 block diagram

OV3660

image sensor core

column sample/hold

image sensor	image output processor		interface

image array

AMP

10-bit ADC

50/60 Hz

auto detection

control register bank

PLL

timing generator

and

system control logic

SCCB interface

3660_DS_2_1

gain control

row select

ISP

compression engine

FIFO

DVP

D[9:0]

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

DVDD DOVDD  AVDD

figure 2-2	reference design schematic

AVDD

AGND VH VNH

C8 AVDD C7 AVDD B8 AGND B7 AGND A8 VH

A7 VNH

NC A3 NC A4 NC A5 NC A6 NC C3 NC H4 NC H6

DOVDD B3 DOVDD E6 DOVDD E7 DOVDD E8 DOVDD F3 DOGND D6 DOGND D7

DOGND D8 DGND

DOVDD

JP2 flex cable to Molex 52437-2491

STROBE AGND SIOD AVDD SIOC RESETB VSYNC PWDN HREF

DVDD

VNL RESETB

DGND

B6 VNL

F6 PWDN G6 STROBE G8 VSYNC G7 TM

H7 SIOC H8 SIOD B1 HREF B2 XVCLK

C6 RESETB G3 DVDD A1 DVDD E1 DVDD F7 DVDD F8 DVDD A2 DOGND D3 DOGND H3 DOGND

U6

OV3660 CSP BSI

FSIN E3 PCLK F1 D9 C2 D8 C1 D7 D2 D6 D1 D5 E2 D4 F2 D3 G2 D2 G1 D1 H2 D0 H1

FSIN PCLK D9 D8 D7 D6 D5 D4 D3 D2 D1 D0

FSIN DOVDD D9 XVCLK D8 DGND D7 PCLK D6

D2 D5 D3 D4 D1 D0

PWDN STROBE VSYNC

SIOC SIOD HREF XVCLK

C28 0.1μF-0201

C24 0.1μF-0201  VH

C25 0.1μF-0201 VNH

C23 0.1μF-0201 VNL

note 7

C26 0.1μF-0201

C27 0.1μF-0201

note 1 PWDN should not be left floating. Please refer to power up sequence section for more details on controlling the PWDN pin.

note 2 RESETB is mandatory during power up reset and should be connected out (please refer to section 2.9 for more details)

note 3 AVDD is 2.8 V of sensor analog power (clean). During OTP programming, an AVDD voltage range of 2.5 V ±5% is required.

OTP read may use normal AVDD voltage range.

note 4 DOVDD is 1.7 ~ 3 V of sensor digital IO power (clean). 1.8 V is recommended.

note 5 DVDD is 1.5 V sensor core power (clean).

DVDD can be provided by internal regulator (recommended).

note 6 sensor AGND and DGND should be separated and connected to a single point outside module (do not connect inside module).

note 7 capacitors should be close to the related sensor pins.

note 8 if more space is available, use a capacitor of 1 μF-0402 between DVDD and DGND

note 9 D[9:0] (D9:MSB, D0:LSB) is sensor RGB raw 10-bit output D[9:2] (D9:MSB, D2:LSB) is 8-bit output.

3660_CSP_DS_2_2

rows

...

  - pixel array structure

The OV3660 sensor has an image array of 2080 columns by 1548 rows (3,219,840 pixels). figure 2-3 shows a cross-section of the image sensor array.

The color filters are arranged in a Bayer pattern. The primary color BG/GR array is arranged in line-alternating fashion. Of the 3,219,840 pixels, 3,145,728 (2048x1536) are active pixels and can be output. The other pixels are used for black level calibration and interpolation.

0

1

14

15

16

17

2062

2063

2064

2065

2078

2079

The sensor array design is based on a field integration readout system with line-by-line transfer and an electronic shutter with a synchronous pixel readout scheme.

figure 2-3

sensor array region color filter layout

columns

...	...	...

- B Gb B Gb B Gb	B Gb B Gb B Gb	B Gb B Gb B Gb

- Gr R Gr R Gr R	Gr R Gr R Gr R	Gr R Gr R Gr R

- B Gb B Gb B Gb	B Gb B Gb B Gb	B Gb B Gb B Gb

- Gr R Gr R Gr R	Gr R Gr R Gr R	Gr R Gr R Gr R

- B Gb B Gb B Gb	B Gb B Gb B Gb	B Gb B Gb B Gb

- Gr R Gr R Gr R	Gr R Gr R Gr R	Gr R Gr R Gr R

- B Gb B Gb B Gb  B Gb B Gb B Gb  B Gb B Gb B Gb

- Gr R Gr R Gr R  Gr R Gr R Gr R  Gr R Gr R Gr R B Gb B Gb B Gb  B Gb B Gb B Gb  B Gb B Gb B Gb Gr R Gr R Gr R  Gr R Gr R Gr R  Gr R Gr R Gr R

1540 B Gb B Gb B Gb  B Gb B Gb B Gb  B Gb B Gb B Gb

1541 Gr R Gr R Gr R  Gr R Gr R Gr R  Gr R Gr R Gr R

1542 B Gb B Gb B Gb	B Gb B Gb B Gb	B Gb B Gb B Gb

1543 Gr R Gr R Gr R	Gr R Gr R Gr R	Gr R Gr R Gr R

1544 B Gb B Gb B Gb	B Gb B Gb B Gb	B Gb B Gb B Gb

1545 Gr R Gr R Gr R	Gr R Gr R Gr R	Gr R Gr R Gr R

1546 B Gb B Gb B Gb	B Gb B Gb B Gb	B Gb B Gb B Gb

1547 Gr R Gr R Gr R	Gr R Gr R Gr R	Gr R Gr R Gr R

dummy column

active column

dummy column

dummy line

active line

dummy line

3660_DS_2_3

###### binning

Binning mode is usually used for low resolution. When the binning function is ON, voltage levels of adjacent pixels are averaged before sent to the ADC. If the binning function is OFF, the pixels, which are not output, are merely skipped. The OV3660 supports 2x2 binning, where the voltage levels of two horizontal (2x1) adjacent same-color pixels are averaged before entering the ADC.

figure 2-4 illustrates 2x2 binning, where the voltage levels of four (2x2) adjacent same-color pixels are averaged before entering the ADC.

B

G

B

G

figure 2-4	example of 2x2 binning

B

G

B

G

2x2 blue pixels are	2x2 green pixels are binned to 1 blue pixel binned to 1 green pixel

B  G  B  G

G  R  G  R 	B  G

B  G  B  G 	G  R  G  R  G  R

B G B G G R G R B G B G G R G R

B  G  B  G

G  R  G  R

B  G  B  G

G  R G R

B G B G G R G R B G B G G R G R

B  G

G  R

B G

G R

B G

G R

2x2 green pixels are	2x2 red pixels are binned to 1 green pixel  binned to 1 red pixel

3660_DS_2_4

table 2-1	binning-related registers

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

| 0x3820 | TIMING TC REG20 | 0x64 | RW | Bit[0]: | Vertical binning enable |
| --- | --- | --- | --- | --- | --- |
| 0x3821 | TIMING TC REG21 | 0x0C | RW | Bit[0]: | Horizontal binning enable |

  - power up sequence

Based on the system power configuration (1.8V or 2.8V for I/O power, using external DVDD or internal DVDD, the power up sequence will differ. If 1.8V is used for I/O power, using the internal DVDD is preferred. If 2.8V is used for I/O power, due to a high voltage drop at the internal DVDD regulator, there is a potential heat issue. Hence, for a 2.8V power system, OmniVision recommends using an external DVDD source. Due to the higher power down current when using an external DVDD source, OmniVision strongly recommends cutting off all powers, including the external DVDD, when the sensor is not in use in the case of 2.8V I/O and external DVDD.

###### power up with internal DVDD

For powering up with the internal DVDD and I2C access during the power ON period, the following conditions must occur:

- when DOVDD and AVDD are turned ON, make sure DOVDD becomes stable before AVDD becomes stable

- PWDN is active high with an asynchronized design (does not need clock)

- PWDN must go high during the power on period

- for PWDN to go low, power must first become stable (AVDD to PWDNB ≥ 5 ms)

- RESETB is active low with an asynchronized design

- state of RESETB does not matter during power on period once DOVDD is up

- master clock XVCLK should provide at least 2 ms before host accesses the sensor’s registers

- host can access I2C bus (if shared) during entire period. 20 ms after PWDN goes low or 20 ms after RESETB goes high if reset is inserted after PWDN goes low, host can access the sensor’s registers to initialize sensor

##### figure 2-5	power up timing with internal DVDD

DOVDD

power on	power off

t0

t5

AVDD	t2

PWDN	t3

t6

>=0ms t1

RESETB	t4

XVCLK SCCB

note t0  0ms, delay from DOVDD stable to AVDD stable, it is recommended to power up AVDD shortly after DOVDD has been powered up

t1  0ms, delay from XVCLK off to AVDD off

t2  5ms, delay from AVDD stable to sensor power up stable, PWDN can be pulled low after this point,

XVCLK can be turned on after power on

t3  1ms, delay from sensor power up stable to RESETB pull up t4  20ms, delay from RESETB pull high to SCCB initialization t5  0ms, delay from AVDD off to DOVDD off

t6  0ms, delay from RESETB pull low to AVDD off

3660_DS_2_14

###### power up with external DVDD source

For powering up with an external DVDD source and I2C access during the power ON period, the following conditions must occur:

- when DOVDD and AVDD are turned ON, make sure DOVDD becomes stable before AVDD becomes stable

- when AVDD and DVDD are turned ON, make sure AVDD becomes stable before DVDD becomes stable

- PWDN is active high with an asynchronized design (does not need clock)

- for PWDN to go low, power must first become stable (DVDD to PWDNB ≥ 5 ms)

- all powers are cut off when the camera is not in use (power down mode is not recommended

- RESETB is active low with an asynchronized design

- state of RESETB does not matter during power on period once DOVDD is up

- master clock XVCLK should provide at least 2 ms before host accesses the sensor’s registers

- host can access I2C bus (if shared) during entire period. 20 ms after PWDN goes low or 20 ms after RESETB goes high if reset is inserted after PWDN goes high, host can access the sensor’s registers to initialize sensor

figure 2-6	power up timing with external DVDD source

DOVDD first, then AVDD, followed by DVDD, and rising time is less than 5 ms

t0

cut off power

DOVDD	t6

t1

AVDD

t

t2

7

DVDD

power on period

t3

t5

PWDN

RESETB

XVCLK

SCCB

note t0  0 ms: delay from DOVDD stable to AVDD stable, it is recommended to power up AVDD shortly after DOVDD has been powered up

t1  0 ms: delay from AVDD stable to DVDD stable

t2  5 ms: delay from DVDD stable to sensor power up stable

t3  1ms, delay from sensor power up stable to RESETB pull up

t4  20ms, delay from RESETB pull high to SCCB initialization

t5  0ms, delay from AVDD off to DOVDD off

t6  0ms, delay from RESETB pull low to DVDD off

t7  0ms, delay from XVCLK off to DVDD off

3660_DS_2_15

t4

  - reset

The OV3660 sensor includes a RESETB pin (pin C6) that forces a complete hardware reset when it is pulled low (GND). The OV3660 clears all registers and resets them to their default values when a hardware reset occurs. A reset can also be initiated through the SCCB interface by setting register 0x3008[7] to high. Reset requires ~2ms settling time.

###### power ON reset generation

The power on reset can be controlled from the RESETB pin. However, inside this chip, a power on reset is generated after power is stable.

  - hardware and software standby

Two suspend modes are available for the OV3660:

- hardware standby

- software standby

###### hardware standby

To initiate a hardware standby, the PWDN pin (pin F6) must be tied to high. When this occurs, the OV3660 internal device clock is halted and all internal counters are reset and registers are maintained. Majority of the digital circuitry will remain in the power-cut state.

###### software standby

Executing a software standby through the SCCB interface suspends internal circuit activity but does not halt the device clock. All register content is maintained in standby mode.

  - format and frame rate

2.7.1 output format control

table 2-3

output format control registers

0x501F

FORMAT CTRL

0x03

RW

Format Controla

Bit[2:0]: Format select

000: YUV422

001: RGB

010: Dither

011: RAW after DPC 101: RAW after CIP

a.	for details, contact your local OmniVision FAE for the software application note

table 2-2	format and frame rate

| format | resolution | frame rate | scaling method | system clock |
| --- | --- | --- | --- | --- |

| 3 megapixel | 2048x1536 | 15 fps | full resolution | 54 MHz |
| --- | --- | --- | --- | --- |
| 1080p | 1920x1080 | 20 fps | cropping from full resolution | 54 MHz |
| 720p | 1280x720 | 45 fps | cropping from full resolution | 54 MHz |
| XGA | 1024x768 | 45 fps | subsampling in vertical and horizontal | 54 MHz |
| VGA | 640x480 | 60 fps | subsampling from 1280x960 | 27 MHz |
| QVGA | 320x240 | 120 fps | subsampling from 1280x960 | 27 MHz |

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

2.7.1.1 format description

Format control converts the internal data format into the desired output format including YUV, RGB, or RAW. Format setting should also combine with 0x501F.

##### table 2-4	format control registers (sheet 1 of 4)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

Format Control 00

Bit[7:4]: Output format selection 0x0: RAW

0x4300	FORMAT CTRL 00

0xF8

RW

Bit[3:0]: Output sequence

0x0: BGBG... / GRGR...

0x1: GBGB... / RGRG...

0x2: GRGR... / BGBG...

0x3: RGRG... / GBGB...

0x4~0xF: Not allowed

0x1: Y8

Bit[3:0]: Does not matter 0x2: Not used

0x3: YUV422

Bit[3:0]: Output sequence

0x0: YUYV...

0x1: YVYU...

0x2: UYVY...

0x3: VYUY...

0x4~0xE: Not allowed 0xF: UYVY...

0x4~0x5: Not used 0x6: RGB565

Bit[3:0]: Output sequence

0x0: {b[4:0],g[5:3]},

{g[2:0],r[4:0]}

0x1: {r[4:0],g[5:3]},

{g[2:0],b[4:0]}

0x2: {g[4:0],r[5:3]},

{r[2:0],b[4:0]}

0x3: {b[4:0],r[5:3]},

{r[2:0],g[4:0]}

0x4: {g[4:0],b[5:3]},

{b[2:0],r[4:0]}

0x5: {r[4:0],b[5:3]},

{b[2:0],g[4:0]}

0x6~0xE: Not allowed 0xF: {g[2:0],b[4:0]},

{r[4:0],g[5:3]}

0x7: RGB555 format 1

Bit[3:0]: Output sequence

0x0: {b[4:0],g[4:2]},

{g[1:0],1'b0,r[4:0]}

0x1: {r[4:0],g[4:2]},

{g[1:0],1'b0,b[4:0]}

##### table 2-4	format control registers (sheet 2 of 4)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

0x2: {g[4:0],r[4:2]},

{r[1:0],1'b0,b[4:0]}

0x3: {b[4:0],r[4:2]},

{r[1:0],1'b0,g[4:0]}

0x4: {r[4:0],b[4:2]},

{b[1:0],1'b0,g[4:0]}

0x5: {g[4:0],b[4:2]},

{b[1:0],1'b0,r[4:0]}

0x6~0xF: Not allowed 0x8: RGB555 format 2

Bit[3:0]: Output sequence

0x0: {1'b0,b[4:0],g[4:3]},

{g[2:0],r[4:0]}

0x1: {1'b0,r[4:0],g[4:2]},

{g[2:0],b[4:0]}

0x2: {1'b0,g[4:0],r[4:2]},

{r[2:0],b[4:0]}

0x3: {1'b0,b[4:0],r[4:2]},

{r[2:0],g[4:0]}

0x4: {1'b0,r[4:0],b[4:2]},

{b[2:0],g[4:0]}

0x5: {1'b0,g[4:0],b[4:2]},

{b[2:0],r[4:0]}

0x6: {b[4:0],1'b0,g[4:3]},

{g[2:0],r[4:0]}

0x7: {r[4:0],1'b0,g[4:2]},

{g[2:0],b[4:0]}

0x8: {g[4:0],1'b0,r[4:2]},

{r[2:0],b[4:0]}

0x9: {b[4:0],1'b0,r[4:2]},

{r[2:0],g[4:0]}

0xA: {r[4:0],1'b0,b[4:2]},

{b[2:0],g[4:0]}

0xB: {g[4:0],1'b0,b[4:2]},

{b[2:0],r[4:0]}

0xC~0xF: Not allowed 0x9: RGB444 format 1

Bit[3:0]: Output sequence

0x0: {1'b0,b[3:0],2'h0,g[3]},

{g[2:0],1'b0,r[3:0]}

0x1: {1'b0,r[3:0],2'h0,g[3]},

{g[2:0],1'b0,b[3:0]}

0x2: {1'b0,g[3:0],2'h0,r[3]},

{r[2:0],1'b0,b[3:0]}

0x3: {1'b0,b[3:0],2'h0,r[3]},

{r[2:0],1'b0,g[3:0]}

0x4: {1'b0,r[3:0],2'h0,b[3]},

{b[2:0],1'b0,g[3:0]}

0x5: {1'b0,g[3:0],2'h0,b[3]},

{b[2:0],1'h0,r[3:0]}

##### table 2-4	format control registers (sheet 3 of 4)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

0x6: {b[3:0],1'b0,g[3:1]},

{g[0],2'h0,r[3:0],1'b0}

0x7: {r[3:0],1'b0,g[3:1]},

{g[0],2'h0,b[3:0],1'b0}

0x8: {g[3:0],1'b0,r[3:1]},

{r[0],2'h0,b[3:0],1'b0}

0x9: {b[3:0],1'b0,r[3:1]},

{r[0],2'h0,g[3:0],1'b0}

0xA: {r[3:0],1'b0,b[3:1]},

{b[0],2'h0,g[3:0],1'b0}

0xB: {g[3:0],1'b0,b[3:1]},

{b[0],2'h0,r[3:0],1'b0}

0xC~0xF: Not allowed 0xA: RGB444 format 2

Bit[3:0]: Output sequence

0x0: {4'b0,b[3:0]},

{g[3:0],r[3:0]}

0x1: {4'b0,r[3:0]},

{g[3:0],b[3:0]}

0x2: {4'b0,b[3:0]},

{r[3:0],g[3:0]}

0x3: {4'b0,r[3:0]},

{b[3:0],g[3:0]}

0x4: {4'b0,g[3:0]},

{b[3:0],r[3:0]}

0x5: {4'b0,g[3:0]},

{r[3:0],b[3:0]}

0x6: {b[3:0],g[3:0],2'h0},

{r[3:0],b[3:0],2'h0,g[3: 0],r[3:0],2'h0}

0x7: {r[3:0],g[3:0],2'h0},

{b[3:0],r[3:0],2'h0,g[3: 0],b[3:0],2'h0}

0x8: {b[3:0],r[3:0],2'h0},

{g[3:0],b[3:0],2'h0,r[3: 0],g[3:0],2'h0}

0x9: {r[3:0],b[3:0],2'h0},

{g[3:0],r[3:0],2'h0,b[3: 0],g[3:0],2'h0}

0xA: {g[3:0],b[3:0],2'h0},

{r[3:0],g[3:0],2'h0,b[3: 0],r[3:0],2'h0}

0xB: {g[3:0],r[3:0],2'h0},

{b[3:0],g[3:0],2'h0,r[3: 0],b[3:0],2'h0}

0xC~0xF: Not allowed 0xB~0xE: Not allowed

##### table 2-4	format control registers (sheet 4 of 4)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

0xF: Bypass formatter module Bit[3:0]: Output format

0x8: Raw 0x9: YUV422

|  |  |  |  | Bit[1:0]: | YUV422 UV control |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 00: U/V generated from average |
| 0x4301 | FORMAT CTRL 01 | 0x00 | RW |  | 01: U/V generated from first pixel |
|  |  |  |  |  | 10: Not valid |
|  |  |  |  |  | 11: U/V generated from second pixel |
| 0x4302 | YMAX VALUE | 0x03 | RW | Bit[1:0]: | Set y max clip value[9:8] |
| 0x4303 | YMAX VALUE | 0xFF | RW | Bit[7:0]: | Set y max clip value[7:0] |
| 0x4304 | YMIN VALUE | 0x00 | RW | Bit[1:0]: | Set y min clip value[9:8] |
| 0x4305 | YMIN VALUE | 0x00 | RW | Bit[7:0]: | Set y min clip value[7:0] |
| 0x4306 | UMAX VALUE | 0x03 | RW | Bit[1:0]: | Set u max clip value[9:8] |
| 0x4307 | UMAX VALUE | 0xFF | RW | Bit[7:0]: | Set u max clip value[7:0] |
| 0x4308 | UMIN VALUE | 0x00 | RW | Bit[1:0]: | Set u min clip value[9:8] |
| 0x4309 | UMIN VALUE | 0x00 | RW | Bit[7:0]: | Set u min clip value[7:0] |
| 0x430A | VMAX VALUE | 0x03 | RW | Bit[1:0]: | Set v max clip value[9:8] |
| 0x430B | VMAX VALUE | 0xFF | RW | Bit[7:0]: | Set v max clip value[7:0] |
| 0x430C | VMIN VALUE | 0x00 | RW | Bit[1:0]: | Set v min clip value[9:8] |
| 0x430D | VMIN VALUE | 0x00 | RW | Bit[7:0]: | Set v min clip value[7:0] |

  - I/O control

The I/O control allows the user to configure the I/O pad direction to either be an input or output. In addition, the I/O control also allows the user to set the driving capability for the output. Please take note that the driving capability is not independently controlled for each output. Setting the driving capability will apply to all configured outputs. table 2-5 lists the driving capability and direction control registers of the I/O pads.

| function | register | default value | R/W | description |
| --- | --- | --- | --- | --- |

| D[9:0] output select 0x301D[3:0], 0x00 0x301E[7:2] | RW | output selection for the D[9:0] pins 0: normal data path 1: register-controlled value |
| --- | --- | --- |
| D[9:0] output value 0x301A[3:0], 0x00 | RW | D[9:0] output value |
| D[9:0] input value 0x3051[3:0], – | R | D[9:0] input value |

table 2-5

driving capability and direction control for I/O pads (sheet 1 of 2)

Bit[7:6]:

output drive capability control

output drive capability for all I/O type pads (see table 1-2)

0x302C

0x02

RW

00: 1x

01: 2x

10: 3x

11: 4x

D[9:0] I/O control

0x3017[3:0],

0x3018[7:2]

0x00

RW

input/output control for the D[9:0] pins 0:	input

1:	output

0x301B[7:2]

0x3052[7:2]

Bit[6]:

VSYNC I/O control

0x3017

0x00

RW

input/output control for the VSYNC pin

0:	input

1:	output

Bit[6]:	output selection for the VSYNC pin

VSYNC output select	0x301D	0x00	RW	0:	normal data path

1:	register-controlled value

VSYNC output value

0x301A

0x00

RW

Bit[6]:	VSYNC output value

note

To achieve optimal signal quality, for high speed and high capacitance loading designs, OmniVision recommends using a higher drive capability. Conversely, for low speed and low capacitance loading, use a lower driving capability setting.

VSYNC input value	0x3051	–	R	Bit[6]:	VSYNC input value

Bit[5]:	input/output control for

HREF I/O control	0x3017	0x00	RW

the HREF pin 0:	input

1:	output

##### table 2-5	driving capability and direction control for I/O pads (sheet 2 of 2)

| function | register | default value | R/W | description |
| --- | --- | --- | --- | --- |

| HREF output select | 0x301D | 0x00 | RW | Bit[5]: | output selection for the HREF pin 0: normal data path 1: register-controlled value |
| --- | --- | --- | --- | --- | --- |
| HREF output value | 0x301A | 0x00 | RW | Bit[5]: | HREF output value |
| HREF input value | 0x3051 | – | R | Bit[5]: | HREF input value |
| PCLK I/O control | 0x3017 | 0x00 | RW | Bit[4]: | input/output control for the PCLK pin 0: input 1: output |
| PCLK output select | 0x301D | 0x00 | RW | Bit[4]: | output selection for the PCLK pin 0: normal data path 1: register-controlled value |
| PCLK output value | 0x301A | 0x00 | RW | Bit[4]: | PCLK output value |
| PCLK input value | 0x3051 | – | R | Bit[4]: | PCLK input value |

  - system control

System control registers include clock, reset control, and PLL configure. Individual modules can be reset or clock gated by setting the appropriate registers.

##### table 2-6	system control registers (sheet 1 of 3)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

0x3000~

0x3003

0x3004~

0x3007

SYSTEM RESET00~ SYSTEM RESET03

CLOCK ENABLE00~ CLOCK ENABLE03

- RW	Reset for Individual Blocks

- RW	Clock Enable Control for Individual

Blocks

System Control

Bit[7]:	Software reset

Bit[6]:	Software power down

0x3008	SYSTEM CTROL0	0x02	RW

Bit[5]:  Reserved

Bit[4]: SRB clock SYNC enable Bit[3]: Isolation suspend select Bit[2:0]: Not used

0x300A	SENSOR CHIP ID HIGH BYTE	0x36	R	Chip ID High Byte

0x300B	SENSOR CHIP ID LOW BYTE	0x60	R	Chip ID Low Byte

Input/Output Control (0: input; 1: output)

0x3016	PAD OUTPUT ENABLE 00	0x22	RW

0x3017	PAD OUTPUT ENABLE 01	0x00	RW

0x3018	PAD OUTPUT ENABLE 02	0x00	RW

0x3019	PAD OUTPUT VALUE 00	0xF0	RW

Bit[2]:	FSIN output enable Bit[1]:	 STROBE output enable Bit[0]:	Not used

Input/Output Control (0: input; 1: output) Bit[7]:	Not used

Bit[6]:	 VSYNC output enable Bit[5]:	HREF output enable Bit[4]:	 PCLK output enable Bit[3:0]: D[9:6] output enable

Input/Output Control (0: input; 1: output) Bit[7:2]: D[5:0] output enable Bit[1:0]: Not used

PAD Output Value Bit[7:3]: Not used Bit[2]:	FSIN

Bit[1]:	STROBE

Bit[0]:	Not used

##### table 2-6	system control registers (sheet 2 of 3)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

0x301A	PAD OUTPUT VALUE 01	0x00	RW

0x301B	PAD OUTPUT VALUE 02	0x00	RW

0x301C	PAD SEL0	0x00	RW

0x301D	PAD SEL1	0x00	RW

GPIO Output Value Bit[7]:	Reserved

Bit[6]:	VSYNC

Bit[5]:	HREF

Bit[4]:	PCLK Bit[3:0]: D[9:6]

GPIO Output Value Bit[7:2]: D[5:0]

Bit[1:0]: Not used

Pad select control

Bit[2]:	IO FSIN select Bit[1]:	IO STROBE select Bit[0]:	Not used

Pad select control Bit[7]:	Not used

Bit[6]:	IO VSYNC select Bit[5]:	IO HREF select Bit[4]:	IO PCLK select Bit[3:0]: IO D[9:6] select

Pad select control

| 0x301E | PAD SEL2 | 0x00 | RW Bit[7:2]: IO D[5:0] select Bit[1:0]: Not used |
| --- | --- | --- | --- |
| 0x302A | CHIP REVISION | – | Bit[7:4]: Process R 0xB: BSI Bit[3:0]: Chip revision |
| 0x302C | PAD CONTROL | 0x03 | Pad control Bit[7:6]: Pad driving strength 00: 1x 01: 2x RW 10: 3x 11: 4x Bit[5]: pd_dato_en Bit[4:2]: Reserved Bit[1]: FSIN input enable Bit[0]: STROBE input enable |
| 0x303A | SC PLLS CTRL0 | 0x00 | RW Bit[7]: PLLS bypass Bit[6:0]: Reserved |
| 0x303B | SC PLLS CTRL1 | 0x1B | RW Bit[4:0]: PLLS multiplier |
| 0x303C | SC PLLS CTRL2 | 0x11 | RW Bit[6:4]: PLLS charge pump control Bit[3:0]: PLLS system divider |

##### table 2-6	system control registers (sheet 3 of 3)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

0x303D	SC PLLS CTRL3	0x30	RW

0x3050	IO PAD VALUE	–	R

0x3051	IO PAD VALUE	–	R

0x3052	IO PAD VALUE	–	R

Bit[5:4]: PLLS predivider 00: /1

01: /1.5

10: /2

11: /3

Bit[3]:	Not used

Bit[2]:	PLLS root divider 0:	/1

1:	/2

Bit[1:0]: PLLS seld5

00: /1

01: /1

10: /2

11: /2.5

Read pad value Bit[7:4]: Not used Bit[3]:	PWDN

Bit[2]:	Not used

Bit[1]:	SIOC

Read pad value

Bit[7]:	OTP memory out Bit[6]:	VSYNC

Bit[5]:	HREF

Bit[4]:	PCLK Bit[3:0]: D[9:6]

Read pad value Bit[7:2]: D[5:0]

Bit[1:0]: Not used

###### system clock control

The OV3660 PLL allows input clock frequency from 6~27 MHz and has VCO frequency of 150MHz~500MHz. SYS_CLK is for the internal clock of the Image Signal Processing (ISP) block. The PLL can be bypassed by setting register 0x303A[7] to 1.

By default, the PCLK frequency is decided by the horizontal scaling ratio. Refer to table 2-7 for the auto PCLK divider ratio. When 0x460C[1] is set to 1, the PCLK divider ratio is manually set by 0x3824[4:0].

table 2-7

PCLK divider ratio

CF = horizontal input size / horizontal output size

| horizontal scaling factor (CF) | PCLK divider ratio |
| --- | --- |
| CF<2 | 1 |
| 2<=CF<4 | 2 |
| 4<=CF<8 | 4 |
| 8<=CF<16 | 8 |
| CF>16 | 16 |

##### figure 2-7	PLL block diagram

| configuration | register 0x303A | register 0x303B | register 0x303C | register 0x303D | register 0x3824 | register 0x460C |
| --- | --- | --- | --- | --- | --- | --- |

| defaulta | 0x00 | 0x1B | 0x11 | 0x30 | 0x01 | 0x20 |
| --- | --- | --- | --- | --- | --- | --- |
| (sample 1)b | 0x00 | 0x1B | 0x12 | 0x30 | 0x01 | 0x20 |

XVCLK

(6 ~ 27MHz)

REFIN

(4 ~ 13.5MHz)

fmax: 216MHz

MUX

PLL bypass

{0x303A[7], 1'h0}

PLL_CLK

digital core

Sys_Clk

PCLK

3660_DS_2_5

table 2-8

PLL configurations

pdiv (auto control)

/2

system divider ({0x303C[3:0], 4'h1,4'h0} means

divide by

seld5 (0x303D[1:0], 2'h0)

1/1/2/2.5

predivider

{0x303D[5:4], 2'h3}

1/1.5/2/3

VCO (150~500MHz)

multiplier {0x303B[4:0], 5'h1B}

(150MHz< multiplier*(root divider +1)*REFIN <500MHz)

{root divider, 0x303D[2], 1'h0}

PLL

/2

note

Contact your local OmniVision FAE for additional assistance on PLL configuration.

default settings are for 54MHz pixel clock, sensor output of 3 Mpixel at 15 fps, and DVP output

      - sample settings are for 54MHz pixel clock, sensor output of 3 Mpixel at 7.5 fps and DVP output

##### table 2-9	PLL control registers

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

0x303A	SC PLLS CTRL0	0x00	RW	Bit[7]:	PLLS bypass

Bit[6:0]: Reserved

0x303B	SC PLLS CTRL1	0x1B	RW	Bit[4:0]: PLLS multiplier

0x303C	SC PLLS CTRL2	0x11	RW	Bit[6:4]: PLLS charge pump control

Bit[3:0]: PLLS system divider

Bit[5:4]: PLLS predivider 00: /1

01: /1.5

10: /2

11: /3

Bit[2]:	PLLS root divider

0x303D	SC PLLS CTRL3	0x30	RW

0:	/1

1:	/2

Bit[1:0]: PLLS seld5

00: /1

01: /1

10: /2

11: /2.5

0x3824	TIMING TC REG24	0x01	RW	Bit[7:5]: Not used

Bit[4:0]: PCLK ratio manual

Bit[1]:	PCLK manual enable

0:	DVP PCLK divider

0x460C	VFIFO CTRL0C	0x20	RW

control by auto mode 1:	DVP PCLK divider

control by 0x3824[4:0]

  - group write

Group write is supported in order to update a group of registers in the same frame. These registers are guaranteed to be written prior to the internal latch at the frame boundary.

The OV3660 supports up to four groups. These groups share 512 MB RAM and the size of each group is programmable by adjusting the start address.

0x3208

GROUP ACCESS

–

W

table 2-10	group write registers

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

Group Access Bit[7:4]: group_ctrl

0000: Group hold start 0001: Group hold end 1010: Group launch others: Reserved

Bit[3:0]: group_id

0000: Group bank 0, default start from address 0x00

0001: Group bank 1, default start from address 0x40

0010: Group bank 2, default start from address 0x80

0011:  Group bank 3, default start from address 0xB0

others: Reserved

| 0x3209 | GRP0_PERIOD | 0x00 | RW | Frames For | Staying in Group 0 |
| --- | --- | --- | --- | --- | --- |
| 0x320A | GRP1_PERIOD | 0x00 | RW | Frames For | Staying in Group 1 |
| 0x320B | GRP_SWCTRL | 0x01 | RW | Bit[3]: Bit[2]: Bit[1:0]: | group_switch_repeat context_en Second group selection |
| 0x320D | GRP_ACT | – R |  | Indicates Which Group is Active |  |
| 0x320E | FRAME_CNT_GRP0 | – R |  | frame_cnt_grp0 |  |
| 0x320F | FRAME_CNT_GRP1 | – R |  | frame_cnt_grp1 |  |

- image sensor core digital functions

  - mirror and flip

The OV3660 Mirror and Flip readout modes, which respectively reverse the sensor data readout order horizontally and vertically (see figure 3-1).

##### figure 3-1	mirror and flip samples

F

F

original image

mirrored image

flipped image

mirrored and flipped image

3660_DS_3_1

##### table 3-1	mirror and flip registers

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

0x3820	TIMING TC REG20	0x40	RW

0x3821	TIMING TC REG21	0x00	RW

- for full size flip, it is necessary to set register 0x4514 to 0x88

- for full size mirror + flip, also set register 0x4514 to 0xBB

- for bin + mirror, set register 0x4514 to 0xBB

Timing Control Register Bit[2:1]: Vertical flip enablea

00: Normal

11: Vertical flip

Timing Control Register

Bit[2:1]: Horizontal mirror enableb 00: Normal

11: Horizontal mirrorc

Y_OUTPUT_SIZE

{0x380A 0x380B}

Y_ADDR_END

{0x3806, 0x3807}

  - image windowing and scaling

###### image windowing

The OV3660 uses registers 0x3800 ~ 0x3814 for image windowing. figure 3-2 illustrates how the registers define the windowing size. Physical pixel size is the total pixel array size we have in the sensor. The ISP input size is the total pixel data read from pixel array. Typically, the larger ISP input size is, the less maximum frame rate can be reached. The data output size is the image output size of OV3660. This size is windowed from ISP input size and is defined by x_offset and y_offset as figure 3-2 shows.

(0, 0)

Y_ADDR_ST

{0x3802, 0x3803}

Y_OFFSET

{0x3812, 0x3813}

data output size

X_OUTPUT_SIZE

{0x3808, 0x3809}

ISP input size

X_ADDR_END

{0x3804, 0x3805}

physical pixel size

(X_ADDR_MAX, Y_ADDR_MAX)

3660_DS_3_2

X_ADDR_ST

{0x3800, 0x3801}

X_OFFSET

{0x3810, 0x3811}

figure 3-2	image windowing

figure 3-3 shows the windowing configuration when scaling function is enabled. The pre-scaling image size is the ISP input size subtracted by two times of offsets for both horizontal and vertical.

##### figure 3-3	image windowing configuration

Y_ADDR_ST

{0x3802, 0x3803}

Y_OFFSET

{0x3812, 0x3813}

data output size (after scaling)

pre-scaling size

Y_OFFSET

{0x3812, 0x3813}

X_OUTPUT_SIZE

{0x3808, 0x3809}

ISP input size

X_ADDR_END

{0x3804, 0x3805}	physical pixel size

(X_ADDR_MAX, Y_ADDR_MAX)

3660 DS 3 3

table 3-2

image windowing registers (sheet 1 of 2)

X_ADDR_ST

{0x3800, 0x3801}

X_OFFSET

{0x3810, 0x3811}

X_OFFSET

{0x3810, 0x3811}

Y_ADDR_END

{0x3806, 0x3807}

Y_OUTPUT_SIZE

{0x380A, 0x380B}

(0, 0)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

| 0x3800 | TIMING HS | 0x00 | RW | Bit[3:0]: | X address start[11:8] |
| --- | --- | --- | --- | --- | --- |
| 0x3801 | TIMING HS | 0x00 | RW | Bit[7:0]: | X address start[7:0] |
| 0x3802 | TIMING VS | 0x00 | RW | Bit[2:0]: | Y address start[10:8] |
| 0x3803 | TIMING VS | 0x00 | RW | Bit[7:0]: | Y address start[7:0] |
| 0x3804 | TIMING HW | 0x08 | RW | Bit[3:0]: | X address end[11:8] |
| 0x3805 | TIMING HW | 0x1F | RW | Bit[7:0]: | X address end[7:0] |
| 0x3806 | TIMING VH | 0x06 | RW | Bit[2:0]: | Y address end[10:8] |
| 0x3807 | TIMING VH | 0x0B | RW | Bit[7:0]: | Y address end[7:0] |
| 0x3808 | TIMING DVPHO | 0x08 | RW | Bit[3:0]: | DVP output horizontal width[11:8] |
| 0x3809 | TIMING DVPHO | 0x00 | RW | Bit[7:0]: | DVP output horizontal width[7:0] |
| 0x380A | TIMING DVPVO | 0x06 | RW | Bit[2:0]: | DVP output vertical height[10:8] |

##### table 3-2	image windowing registers (sheet 2 of 2)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

| 0x380B | TIMING DVPVO | 0x00 | RW | Bit[7:0]: | DVP output vertical height[7:0] |
| --- | --- | --- | --- | --- | --- |
| 0x380C | TIMING HTS | 0x08 | RW | Bit[3:0]: | Total horizontal size[11:8] |
| 0x380D | TIMING HTS | 0xFC | RW | Bit[7:0]: | Total horizontal size[7:0] |
| 0x380E | TIMING VTS | 0x06 | RW | Bit[7:0]: | Total vertical size[15:8] |
| 0x380F | TIMING VTS | 0x1C | RW | Bit[7:0]: | Total vertical size[7:0] |
| 0x3810 | TIMING HOFFSET | 0x00 | RW | Bit[3:0]: | ISP horizontal offset[11:8] |
| 0x3811 | TIMING HOFFSET | 0x10 | RW | Bit[7:0]: | ISP horizontal offset[7:0] |
| 0x3812 | TIMING VOFFSET | 0x00 | RW | Bit[2:0]: | ISP vertical offset[10:8] |
| 0x3813 | TIMING VOFFSET | 0x06 | RW | Bit[7:0]: | ISP vertical offset[7:0] |

3.2.2 scaling

The OV3660 includes a scalar function that allows the user to arbitrarily set an output image size (width and height) up to 1/32 of the designated array size. The scalar module outputs the specified image size and maintains the field-of-view as the input image to the scalar. Note that the frame rate will not change in scaling mode.

figure 3-4

scaling function

(0, 0)

vertical start point {0x3802, 0x3803}

vertical offset {0x3812, 0x3813}

pre-scaling size

vertical offset {0x3812, 0x3813}

ISP input size

horizontal output size

{0x3808, 0x3809}

physical pixel size

horizontal endpoint {0x3804, 0x3805}

data output size

(after scaling)

horizontal start point {0x3800, 0x3801}

horizontal offset {0x3810, 0x3811}

horizontal offset {0x3810, 0x3811}

vertical endpoint {0x3806, 0x3807}

vertical output size

{0x380A, 0x380B}

3660_DS_3_4

The following steps allow the user to set their required output size:

  - The scaling function is enabled and disabled by register bit 0x5001[5]. The user then needs to set the image input size, output size and offset. The output size < (input size - 2 × offset).

Horizontal output size: {0x3808, 0x3809} Vertical output size: {0x380A, 0x380B} Horizontal offset: {0x3810, 0x3811} Vertical offset: {0x3812, 0x3813}

Horizontal input size:

Horizontal endpoint {0x3804, 0x3805} - Horizontal start point {0x3800, 0x3801} + 1 - 2 × {0x3810, 0x3811}

Vertical input size:

Vertical endpoint {0x3806, 0x3807} - Vertical start point {0x3802, 0x3803} + 1 - 2 × {0x3812, 0x3813}

  - The scaling factor is calculated automatically.

##### table 3-3	scaling control registers (sheet 1 of 2)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

0x5001	ISP CONTROL 01	0x01	RW

0x5601	SCALE CTRL 1	0x00	RW

Bit[7]:	Scale enable

0:	Disable

1:	Enable

Bit[6:4]: HDIV RW

DCW scale times 000: DCW 1 time

001: DCW 2 times

010: DCW 4 times

100: DCW 8 times

101: DCW 16 times Others: DCW 16 times

Bit[2:0]: VDIV RW

DCW scale times 000: DCW 1 time

001: DCW 2 times

010: DCW 4 times

100: DCW 8 times

101: DCW 16 times Others: DCW 16 times

| 0x5602 | SCALE CTRL 2 | 0x02 | RW | XSC High Bits |
| --- | --- | --- | --- | --- |
| 0x5603 | SCALE CTRL 3 | 0x00 | RW | XSC Low Bits |
| 0x5604 | SCALE CTRL 4 | 0x02 | RW | YSC High Bits |
| 0x5605 | SCALE CTRL 5 | 0x00 | RW | YSC Low Bits |
| 0x5606 | SCALE CTRL 6 | 0x00 | RW | Bit[3:0]: Voffset |

0x3804	TIMING HW	0x00	RW	Bit[7:4]: Not used

Bit[3:0]: HREF horizontal endpoint[11:8]

##### table 3-3	scaling control registers (sheet 2 of 2)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

| 0x3805 | TIMING HW | 0xDF | RW Bit[7:0]: HREF horizontal endpoint[7:0] |
| --- | --- | --- | --- |
| 0x3806 | TIMING VH | 0x00 | RW Bit[7:4]: Not used Bit[3:0]: HREF vertical endpoint[11:8] |
| 0x3807 | TIMING VH | 0x9B | RW Bit[7:0]: HREF vertical endpoint[7:0] |
| 0x3808 | TIMING ISPHO | 0x00 | RW Bit[7:4]: Not used Bit[3:0]: ISP output horizontal width[11:8] |
| 0x3809 | TIMING ISPHO | 0xC0 | RW Bit[7:0]: ISP output horizontal width[7:0] |
| 0x380A | TIMING ISPVO | 0x00 | RW Bit[7:4]: Not used Bit[3:0]: ISP output vertical width[11:8] |
| 0x380B | TIMING ISPVO | 0x90 | RW Bit[7:0]: ISP output vertical width[7:0] |
| 0x3810 | TIMING HOFFS HIGH | 0x00 | RW Bit[7:4]: Not used Bit[3:0]: Horizontal offset[11:8] |
| 0x3811 | TIMING HOFFS LOW | 0x10 | RW Bit[7:0]: Horizontal offset[7:0] |
| 0x3812 | TIMING VOFFS HIGH | 0x00 | RW Bit[7:4]: Not used Bit[3:0]: Vertical offset[11:8] |
| 0x3813 | TIMING VOFFS LOW | 0x10 | RW Bit[7:0]: Vertical offset[7:0] |

  - test pattern

For testing purposes, the OV3660 offers one type of test pattern, color bar.

color bar

table 3-4

test pattern selection control

Bit[7]:	Color bar enable

0:	Color bar disable 1:	Color bar enable

Bit[3:2]: Color bar style

0x503D

PRE ISP TEST SETTING 1

0x00

RW

00:

01:

10:

11:

Standard eight color bar Gradual change at vertical mode 1

Gradual change at horizontal Gradual change at vertical mode 2

figure 3-5	test pattern

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

  - exposure / gain control

###### overview

The exposure/gain control mode allows for a companion backend processor to control the exposure and gain of the OV3660 externally. The pixel array is reset and sampled in series on a row-by-row basis (also known as "rolling shutter"). The time between reset and sampling is called exposure time. As each pixel is exposed, the charge in the pixel decreases proportionally with the time exposed to incident light and the light intensity.

In the OV3660, the exposure time is set by registers {0x3500[3:0], 0x3501[7:0], ox3502[7:0]} in units of 1/16 row period.

### exposure = {0x3500[3:0], 0x3501[7:0], 0x3502[7:0]} / 16  tROW

The pixel output can be further amplified by the analog amplifier and/or digital multiplier. The OV3660 supports up to 16x analog gain and 4x digital gain. The gain can be calculated from the register value using the following formula:

### gain = {0x350A[1:0], 0x350B[7:0]} / 16

The first 15.9375x gain ({0x350A[1:0] = 2b’00, 0x350B[7:0] = 0x00~0xFF}) is analog gain and the remaining 4x gain is digital gain. Please note that analog gain is always preferred to reduce image noise; thus, for gain adjustments, it is always advisable to maximize analog gain before moving on to use digital gain.

Under AC lighting conditions (e.g., 50Hz or 60Hz fluorescent light), the light intensity is not constant. Though the exposure time of each row is equal to each other, the amount of photons each row receives may vary with the start point of the exposure. In order for each row to receive the same amount of photons, the exposure time must be a multiple of the flicker period of the light intensity.

exposure = {

n/100 second,	for 50Hz n/120 second,	for 60Hz

If the exposure time does not fall in these steps, the pixel array will not be evenly illuminated and horizontal bands will show up in the image.

###### automatic exposure/gain control

The Automatic Exposure Control (AEC) function allows the OV3660 image sensor to adjust the exposure and gain values without any external command or control to the sensor. The OV3660 has a built-in AEC/AGC (auto exposure control / auto gain control) algorithm to automatically control the exposure and gain to bring the image level to a pre-defined range, namely stable range. The AEC and AGC can be enabled by setting register 0x3503[0] and 0x3503[1] to 1’b0, respectively. Although the AEC and AGC can be enabled/disabled independently, it is highly recommended that both

AEC and AGC are enabled/disabled together; otherwise, the auto control may induce flicker artifacts in the image due to the precision limitation of the exposure time.

The image level is a weighted average over a user-defined area as shown in figure 3-6. The area is specified by the top-left point ({0x5680[3:0], 0x5681[7:0]}, {0x5682[2:0], 0x5683[7:0]}) and the bottom-right point ({0x5684[3:0], 0x5685[7:0]}, {0x5686[2:0], 0x5687[7:0]}). The area is equally divided into 4x4 zones and the weight of each zone can be programmed by registers 0x5688~0x568F as shown in table 3-5. The average value is calculated in the linear domain when register 0x5025[1:0] is 2’b00 or after gamma correction when register 0x5025[1:0] is set to 2b’10.

sensor array size X

X

start

HW

Y

VH

average

window size

sensor array

size Y

end

3660_DS_3_6

table 3-5

timing control functions (sheet 1 of 3)

figure 3-6	average-based window definition

| 1 | 2 | 3 | 4 |
| --- | --- | --- | --- |
| 5 | 6 | 7 | 8 |
| 9 | 10 | 11 | 12 |
| 13 | 14 | 15 | 16 |

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

| 0x3810 | TIMING HOFFSET | 0x00 | RW | Bit[3:0]: | ISP horizontal offset[11:8] |
| --- | --- | --- | --- | --- | --- |
| 0x3811 | TIMING HOFFSET | 0x10 | RW | Bit[7:0]: | ISP Horizontal offset[7:0] |
| 0x3812 | TIMING VOFFSET | 0x00 | RW | Bit[3:0]: | ISP vertical offset[11:8] |
| 0x3813 | TIMING VOFFSET | 0x06 | RW | Bit[7:0]: | ISP vertical offset[7:0] |
| 0x3808 | TIMING DVPHO | 0x08 | RW | Bit[3:0]: | DVP output horizontal width[11:8] |
| 0x3809 | TIMING DVPHO | 0x00 | RW | Bit[7:0]: | DVP output horizontal width[7:0] |
| 0x380A | TIMING DVPVO | 0x06 | RW | Bit[3:0]: | DVP output vertical height[11:8] |

##### table 3-5	timing control functions (sheet 2 of 3)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

| 0x380B | TIMING DVPVO | 0x00 | RW Bit[7:0]: DVP output vertical height[7:0] |
| --- | --- | --- | --- |
| 0x501D | ISP MISC | 0x00 | RW Bit[4]: Average size manual enable |
| 0x5680 | X START | 0x00 | Bit[3:0]: X start[11:8] RW Horizontal start position for average window high byte, valid when 0x501D[4]=1 |
| 0x5681 | X START | 0x00 | Bit[7:0]: X start[7:0] RW Horizontal start position for average window low byte, valid when 0x501D[4]=1 |
| 0x5682 | Y START | 0x00 | Bit[2:0]: Y start[10:8] RW Vertical start position for average window low byte, valid when 0x501D[4]=1 |
| 0x5683 | Y START | 0x00 | Bit[7:0]: Y start[7:0] RW Vertical start position for average window low byte, valid when 0x501D[4]=1 |
| 0x5684 | X WINDOW | 0x10 | Bit[3:0]: Window X[11:8] RW Horizontal end position for average window high byte, valid when 0x501D[4]=1 |
| 0x5685 | X WINDOW | 0xA0 | Bit[7:0]: Window X[7:0] RW Horizontal end position for average window low byte, valid when 0x501D[4]=1. |
| 0x5686 | Y WINDOW | 0x0C | Bit[2:0]: Window Y[10:8] RW Vertical end position for average window high byte, valid when 0x501D[4]=1 |
| 0x5687 | Y WINDOW | 0x78 | Bit[7:0]: Window Y[7:0] RW Vertical end position for average window low byte, valid when 0x501D[4]=1 |
| 0x5688 | WEIGHT00 | 0x11 | RW Bit[7:4]: Window 01 weight Bit[3:0]: Window 00 weight |
| 0x5689 | WEIGHT01 | 0x11 | RW Bit[7:4]: Window 03 weight Bit[3:0]: Window 02 weight |
| 0x568A | WEIGHT02 | 0x11 | RW Bit[7:4]: Window 11 weight Bit[3:0]: Window 10 weight |
| 0x568B | WEIGHT03 | 0x11 | RW Bit[7:4]: Window 13 weight Bit[3:0]: Window 12 weight |
| 0x568C | WEIGHT04 | 0x11 | RW Bit[7:4]: Window 21 weight Bit[3:0]: Window 20 weight |
| 0x568D | WEIGHT05 | 0x11 | RW Bit[7:4]: Window 23 weight Bit[3:0]: Window 22 weight |

##### table 3-5	timing control functions (sheet 3 of 3)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

| 0x568E | WEIGHT06 | 0x11 | RW Bit[7:4]: Window 31 weight Bit[3:0]: Window 30 weight |
| --- | --- | --- | --- |
| 0x568F | WEIGHT07 | 0x11 | RW Bit[7:4]: Window 33 weight Bit[3:0]: Window 32 weight |

The upper limit of the stable range is specified by register 0x3A1B and the lower limit is set by register 0x3A1E.

When register 0x3A05[5] is set to 1, the sensor automatically calculates the exposure/gain step according to the distance between the current image level and the center of the stable range. The ratio of the adjustment step to the distance can be set using register 0x3A05[4:0]:

32

step = 0__x_3__A__0__5_[_4__:_0_]_ |current_image_level	target_image_level| where target_image_level equals (0x3A1B+0x3A1E/2)

When register 0x3A05[5] is set to 0, the exposure/gain adjustment is manually set by registers 0x3A06 and 0x3A07, depending on where the current image level is. Another wider range, specified by (0x3A10, 0x3A0F), is used to control the step of the exposure/gain adjustment. When the image level is greater than the value of register 0x3A0F, the sensor will decrease the exposure/gain by a big step1:

=

step1	0__x__3_A__0__7__[_3_:_0__]  current_exposure_gain 16

When the image level is less than the value of register 0x3A10, the sensor will increase the exposure/gain by a big step 2:

=

step2	0__x__3_A__0__6__[_4_:_0__]  current_exposure_gain 32

When the image level is within the wider range of (0x3A10, 0x3A0F) but outside of the stable range (0x3A1E, 0x3A1B), the sensor will adjust the exposure/gain by a small step 3:

=

step3	0__x__3_A__0__7__[_7_:_4__]  current_exposure_gain 16

###### 50Hz/60Hz detection

The OV3660 has a built-in capability to identify 50Hz and 60Hz flickering frequency. When this function is enabled by setting register 0x3004[2] to 1, the sensor will continuously detect the light frequency. The sensor will keep the previous result whenever it is not able to determine the current AC flickering frequency.

The 50Hz/60Hz detection block needs an input clock at a frequency of approximately 3 MHz. Register 0x300C[3:0] is the divider that generates this input clock frequency from the input clock pin (XVCLK).

Register 0x3C01[7] must be set to 0 for the AEC/AGC algorithm to apply the exposure constraint based on the result of the 50Hz/60Hz detection.

Register 0x3C0C[0] shows the result of the 50Hz/60Hz detection. When the detection result is 60Hz, register 0x3C0C[0] = 0, the AEC/AGC algorithm will set the exposure time to an integer multiple of {0x3A0A[1:0], 0x3A0B[7:0]}. Conversely, when the detection is 50Hz, register 0x3C0C[0] = 1, the AEC/AGC algorithm will set the exposure time to an integer multiple of {0x3A08[1:0], 0x3A09[7:0]}.

###### anti-flicker

Under AC lighting conditions, the user can specify whether to restrict the exposure time to n/100 or n/120 second using register 0x3A00[5]. When the exposure time is restricted to n/100 in 50Hz light or n/120 in 60Hz light, the pixel array will be evenly exposed despite the flickering nature of the AC light source.

In order for the sensor to restrict the exposure time to n/100 or n/120 second, registers {0x3A08, 0x3A09} should be set to 1/100 second in units of row period and registers {0x3A0A, 0x3A0B} should be set to 1/120 second in units of row period. Furthermore, registers 0x3A0E and 0x3A0D should be set to the maximum number of flickering periods in one frame period for 50Hz and 60Hz, respectively.

Given the frame rate (frame period) and the number of row period per frame period, registers {0x3A08, 0x3A09},

{0x3A0A, 0x3A0B}, 0x3A0E, and 0x3A0D can be calculated by the following formula with the result rounded to the closest integer:

{	} =

0x3A08, 0x3A09	_f_r_a__m__e____p__e_r____s__e_c__o_n__d_____r_o__w_____p__e_r____f_r_a__m___e_

### 100

{	} =

0x3A0A, 0x3A0B	_f_r_a__m__e____p__e_r____s__e_c__o_n__d_____r_o__w_____p__e_r	f_r_a__m__e__

### 100

=

0x3A0E	__________1__0__0____________

### frame_per_second

=

0x3A0D  __________1__2_0_____________

### frame_per_second

For example, for a YUV output at 15 fps and full resolution of 2048x1536:

### row_per_frame = register {0x380E, 0x380F} = 0x61C = 1564 (decimal)

### {0x3A08, 0x3A09} = (1564  15)/100 = 234.6 (decimal) = 0xEA

### {0x3A08, 0x3A09} = (1564  15)/120 = 195.5 (decimal) = 0xC3

### 0x3A0E = 100/15 = 6.67 (decimal) = 0x06 0x3A0D = 120/15 = 8 (decimal) = 0x08

The light flickering frequency is either automatically set by the 50Hz/60Hz detection circuit (explained in detail in

section 3.4.3) or manually selected through register 0x3C00[2].

Under very bright AC light conditions, the desired exposure time may be less than 1/100 or 1/120 second. In this case, the sensor can either adjust the exposure time to the desired value or limit the minimum exposure time to 1/100 or 1/120 second. By adjusting the exposure time under such bright AC lighting conditions, the user should expect that there will be horizontal flickering artifacts in the image as the pixels are no longer integrating in a multiple of the flicker periods. Conversely, by limiting the minimum exposure time to 1/100 or 1/120 second, the user can expect an over exposed image.

The trade-off described above can be controlled through register 0x3A00[4]. When this register is set to 1, the exposure time is allowed to go below 1/100 or 1/120 second to avoid over exposure. When this register is set to 0, the minimum exposure is limited to 1/100 or 1/120 second to avoid the flicker.

###### extending the exposure time under dark conditions

From a noise point of view, it is always preferable to extend the exposure time rather than increasing gain. However, the exposure time is limited to the frame period at a given frame rate. Under extremely dark conditions, the image may still be too dark when the exposure reaches its maximum and the gain then increases to a certain level. One common way to achieve a better image is to slow down the frame rate which, in turn, extends the exposure time. The AEC/AGC algorithm of the OV3660 supports this feature and terms this as night mode. Night mode can be enabled by setting register 0x3A00[2] to 1.

The sensor enters night mode when the exposure time reaches the maximum and the gain reaches the threshold defined by register 0x3A17[1:0] (see table 3-6). The sensor starts to extend the frame period by the original frame period, or the flickering period, depending on the value set in register 0x3A05[6]. When this register is set to 1, the sensor will extend the exposure time up to the value set in register 0x3A21[6:4] until the image level falls into the stable range. If the image is still too dark after the exposure has been extended to its maximum exposure time, the sensor will further increase the gain. When register 0x3A05[6] is set to 0, the sensor will extend the exposure time by flickering period {0x3A0A[1:0], 0x3A0B[7:0]} or {0x3A08[1:0], 0x3A09[7:0]}, depending on the light frequency. Note that even when the anti-flickering function is disabled, the sensor will extend the exposure time by the flickering period when register 0x3A05[6] is set to 0.

##### table 3-6	night mode control registers

registers

function

night mode ceiling setting	{0x3A02[7:0], 0x3A03[7:0], 0x3A14[7:0], 0x3A15[7:0]}

night mode disable	0x3A00[2]

0:	disable night mode

increase/decrease step in night mode based on band or frames

0x3A05[6]

0:	in night mode, insert frame disable 1:	in night mode, insert frame enable

###### sub-row exposure time under extremely bright conditions

Under extremely bright conditions, even the exposure time of one row period will result in an over exposed image. In this case, the AEC/AGC algorithm can set the exposure time to sub-row period. This function can be enabled by setting register 0x3A00[6].

Due to the readout timing limitation, the minimum and maximum sub-row exposure times have certain constraints. The maximum and minimum sub-row exposure times are:

=

max_sub_row_exposure	0__x__3_A___0_C___[7__:_4__]    row_period

### 16

=

min_sub_row_exposure	_0_x__3__A__0__C__[_3_:_0__]    row_period

### 16

OmniVision recommends setting registers 0x3A0C to 0xC4.

  - black level calibration (BLC)

The pixel array contains several optically shielded (black) lines. These lines are used as reference for black level calibration. Black level adjustments can be made with registers 0x4000 through 0x4013.

##### table 3-7	BLC control functions

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

| 0x4000 | BLC CTRL00 | 0x0D | RW BLC Control 00 Bit[0]: BLC enable |
| --- | --- | --- | --- |
| 0x4002 | BLC CTRL02 | 0x45 | RW Bit[7]: Format change enable BLC update when format changes |

0x4003	BLC CTRL03	0x01	RW

0x4005	BLC CTRL05	0x10	RW

Bit[7]:	BLC redo enable

Write 1 into it will trigger a BLC redo N frames begin, N is 0x4003[5:0]

Bit[6]:	BLC freeze

Bit[5:0]: Manual frame number

Bit[1]:	BLC always update

0:	Normal freeze

1:	BLC always updated

0x4009	BLACK LEVEL	0x10	RW	Bit[7:0]: BLC black level target at 10-bit range

  - strobe flash

###### strobe flash control

The strobe signal is programmable. It supports both LED and Xenon modes. The polarity of the pulse can be changed. The strobe signal is enabled (turned high/low depending on the pulse’s polarity) by requesting the signal via the SCCB interface. Flash modules are triggered by the rising edge by default or by the falling edge if the signal polarity is changed. It supports the following flashlight modes (see table 3-8).

##### table 3-8	flashlight modes

| mode | output | AEC / AGC | AWB |
| --- | --- | --- | --- |

| xenon | one-pulse | no | no |
| --- | --- | --- | --- |
| LED 1 | pulse | no | no |
| LED 2 | pulse | no | yes |
| LED 3 | continuous | yes | yes |

      - xenon flash control

After a strobe request is submitted, the strobe pulse will be activated at the beginning of the third frame (see figure 3-7). The third frame will be correctly exposed. The pulse width can be changed in Xenon mode between 1H and 4H, depending on register 0x3B00[3:2], where H is one row period.

##### figure 3-7	xenon flash mode

vertical blanking

exposure

time

data out

strobe request

strobe pulse

correctly exposed frame

request here

strobe

pulse

zoomed

1H

3660_DS_3_7

      - LED 1 & 2 mode

Two frames after the strobe request is submitted, the third frame is correctly exposed. The strobe pulse will be activated only one time if the strobe end request is set correctly (see figure 3-8). If end request is not sent, the strobe signal is activated intermittently until the strobe end request is set (see figure 3-9). The number of skipped frames is programmable using registers {0x3A1C, 0x3A1D}.

##### figure 3-8	LED 1 & 2 mode - one pulse output

the number of skipped frames

is programmable

request here

strobe

pulse

start	end

strobe

request

data

out

exposure

time

vertical

blanking

frame in	is skipped

frame in

is skipped

3660_DS_3_9

correctly

exposed frame

request here

strobe

pulse

correctly exposed frame

start

strobe

request

data

out

exposure

time

vertical

blanking

LED 1 & 2 mode - multiple pulse output

figure 3-9

3660_DS_3_8

the number of skipped frames

is programmable

      - LED 3 mode

In LED 3 mode, the strobe signal stays active until the strobe end request is sent (see figure 3-10).

##### figure 3-10	LED 3 mode

vertical blanking

exposure

time

data out

strobe request

start	end

strobe signal

correctly exposed frame

request here	request here

3660_DS_3_10

##### table 3-9	strobe control functions

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

0x3B00	STROBE CTRL	0x00	RW

Strobe Control

Bit[7]:	Strobe request ON/OFF 0:	OFF

1:	ON

Bit[6]:	Strobe pulse reverse Bit[3:2]: width_in_xenon Bit[1:0]: Strobe mode

00: Xenon

01: LED 1

10: LED 2

11: LED 3

  - one time programmable (OTP) memory

The OV3660 supports a maximum of 256 bits of one-time programmable (OTP) memory to store chip identification and manufacturing information. OTP memory occupies registers 0x3D00 to 0x3D1F. Typically, registers 0x3D00 to 0x3D04 is reserved for OmniVision and registers 0x3D05 to 0x3D1F is for customer use. Detailed read/write sequences are shown below:

###### OTP write sequence

Write OTP Reg0x3d05-0x3d1f ¡§xx¡¨ //"xx" means the value that customer intends to program Write Enable:

78 3000 20 ;Set Bit[4], Reset OTP=0

78 3004 df ;Set Bit[4], Enable OTP clk=1

78 3d20 01 ;OTP Program Enable

Wait time 50mS

End Write Sequence:

78 3d20 00

78 3004 cf

78 3000 30

###### OTP read sequence

First, write OTP Reg0x3d05-0x3d1f to be "00" Read Enable:

Wait time 50mS

78 3000 20 ;Set Bit[4], Reset OTP=0

78 3004 df ;Set Bit[4], Enable OTP clk=1

78 3d21 01 ;OTP Read Enable

End Read Sequence:

78 3d21 00

78 3004 cf

78 3000 30

Read Reg0x3d00-0x3d1f back. Now, the values are what the customer programmed.

##### table 3-10	OTP control functions (sheet 1 of 2)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

| 0x3D20 | OTP PROGRAM CTRL | 0x00 | RW | Bit[7]: Bit[1]: Bit[0]: | OTP program busy OTP program speed 0: Fast 1: Slow OTP program enable |
| --- | --- | --- | --- | --- | --- |
| 0x3D21 | OTP READ CTRL | 0x00 | RW | Bit[7]: Bit[1]: Bit[0]: | OTP read busy OTP read speed 0: Fast 1: Slow OTP read enable |
| 0x3D00 | OTP DATA00 | 0x00 | RW | OTP Dump/Load Data00 |  |
| 0x3D01 | OTP DATA01 | 0x00 | RW | OTP Dump/Load Data01 |  |
| 0x3D02 | OTP DATA02 | 0x00 | RW | OTP Dump/Load Data02 |  |
| 0x3D03 | OTP DATA03 | 0x00 | RW | OTP Dump/Load Data03 |  |
| 0x3D04 | OTP DATA04 | 0x00 | RW | OTP Dump/Load Data04 |  |
| 0x3D05 | OTP DATA05 | 0x00 | RW | OTP Dump/Load Data05 |  |
| 0x3D06 | OTP DATA06 | 0x00 | RW | OTP Dump/Load Data06 |  |
| 0x3D07 | OTP DATA07 | 0x00 | RW | OTP Dump/Load Data07 |  |
| 0x3D08 | OTP DATA08 | 0x00 | RW | OTP Dump/Load Data08 |  |
| 0x3D09 | OTP DATA09 | 0x00 | RW | OTP Dump/Load Data09 |  |
| 0x3D0A | OTP DATA0A | 0x00 | RW | OTP Dump/Load Data0a |  |
| 0x3D0B | OTP DATA0B | 0x00 | RW | OTP Dump/Load Data0b |  |
| 0x3D0C | OTP DATA0C | 0x00 | RW | OTP Dump/Load Data0c |  |
| 0x3D0D | OTP DATA0D | 0x00 | RW | OTP Dump/Load Data0d |  |
| 0x3D0E | OTP DATA0E | 0x00 | RW | OTP Dump/Load Data0e |  |
| 0x3D0F | OTP DATA0F | 0x00 | RW | OTP Dump/Load Data0f |  |
| 0x3D10 | OTP DATA10 | 0x00 | RW | OTP Dump/Load Data10 |  |
| 0x3D11 | OTP DATA11 | 0x00 | RW | OTP Dump/Load Data11 |  |
| 0x3D12 | OTP DATA12 | 0x00 | RW | OTP Dump/Load Data12 |  |
| 0x3D13 | OTP DATA13 | 0x00 | RW | OTP Dump/Load Data13 |  |
| 0x3D14 | OTP DATA14 | 0x00 | RW | OTP Dump/Load Data14 |  |
| 0x3D15 | OTP DATA15 | 0x00 | RW | OTP Dump/Load Data15 |  |
| 0x3D16 | OTP DATA16 | 0x00 | RW | OTP Dump/Load Data16 |  |

3.8  temperature sensor

The OV3660 supports an on-chip temperature sensor that covers 0~80°C with an error range of 5°C. It can be controlled through the SCCB interface (see table 3-11). When the temperature is lower than 0°C, the reading will stop at 0°C.

table 3-11	temperature sensor function

0x6719

TPM CTRL19

–

R

Bit[7:0]: Measured temperature

##### table 3-10	OTP control functions (sheet 2 of 2)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

| 0x3D17 | OTP DATA17 | 0x00 | RW | OTP Dump/Load Data17 |
| --- | --- | --- | --- | --- |
| 0x3D18 | OTP DATA18 | 0x00 | RW | OTP Dump/Load Data18 |
| 0x3D19 | OTP DATA19 | 0x00 | RW | OTP Dump/Load Data19 |
| 0x3D1A | OTP DATA1A | 0x00 | RW | OTP Dump/Load Data1a |
| 0x3D1B | OTP DATA1B | 0x00 | RW | OTP Dump/Load Data1b |
| 0x3D1C | OTP DATA1C | 0x00 | RW | OTP Dump/Load Data1c |
| 0x3D1F | OTP DATA1D | 0x00 | RW | OTP Dump/Load Data1d |
| 0x3D1E | OTP DATA1E | 0x00 | RW | OTP Dump/Load Data1e |
| 0x3D1F | OTP DATA1F | 0x00 | RW | OTP Dump/Load Data1f |

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

- image sensor processor digital functions

  - ISP general controls

The ISP module provides the following functions:

- lens correction

- gamma control

- de-noise

- AWB

- color matrix

- scaling

- CIP and sharpening

These functions are enabled by registers 0x501D, 0x501F~0x5020, 0x503D, 0x5061~0x5063.

##### table 4-1	ISP general control registers (sheet 1 of 3)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

0x5000	ISP CONTROL 00	0x06	RW

ISP Control 00

Bit[7]:	LENC correction enable 0:	Disable

1:	Enable Bit[5]:	Gamma enable

0:	Disable

1:	Enable

Bit[2]:	Black pixel cancellation enable 0:	Disable

1:	Enable

Bit[1]:	White pixel cancellation enable 0:	Disable

1:	Enable

Bit[0]:	Color interpolation (CIP) enable 0:	Disable

1:	Enable

##### table 4-1	ISP general control registers (sheet 2 of 3)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

0x5001	ISP CONTROL 01	0x01	RW

0x5003	ISP CONTROL 03	0x08	RW

0x5004	ISP CONTROL 04	0x08	RW

0x5005	ISP CONTROL 05	0x36	RW

ISP Control 01

Bit[7]:	Special Digital Effects (SDE) enable

0:	Disable

1:	Enable Bit[5]:	Scale enable

0:	Disable

1:	Enable

Bit[2]:	UV average enable

0:	Disable

1:	Enable

Bit[1]:	Color matrix enable

0:	Disable

1:	Enable

Bit[0]:	Auto white balance (AWB) enable

0:	Disable

1:	Enable

ISP Control 03

Bit[2]:	Bin enable

0:	Disable

1:	Enable Bit[0]:	Solarize enable

0:	Disable

1:	Enable

Bit[3]:	Auto size control enable 0:	Manual

1:	Auto

ISP Control 05

Bit[6]:	AWB bias manual enable 0:	Disable

1:	Enable

Bit[5]:	AWB bias on enable 0:	Disable

1:	Enable

Bit[4]:	AWB bias plus enable 0:	Disable

1:	Enable

Bit[3]:	Reserved

Bit[2]:	LENC bias on enable 0:	Disable

1:	Enable

Bit[1]:	GMA bias on enable 0:	Disable

1:	Enable

Bit[0]:	LENC bias manual enable 0:	Disable

1:	Enable

##### table 4-1	ISP general control registers (sheet 3 of 3)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

0x501D	ISP MISC	0x00	RW

0x501F	FORMAT MUX CONTROL	0x03	RW

0x5020	DITHER CTRL 0	0x00	RW

0x503D	PRE ISP TEST SETTING 1	0x00	RW

Bit[6]:	 SDE AVG manual enable Bit[5]:	AWB YUV2CBCR enable Bit[4]:	AVG size manual enable Bit[3:0]: Reserved

Format Mux control

Bit[5]:	Enable option Bit[4]:	Reserved Bit[3]:	Format v first Bit[2:0]: Format select

000: ISP YUV422

001: ISP RGB

010: ISP dither

011: ISP RAW(DPC)

100: SNR RAW

101: ISP RAW(CIP)

Bit[6]:	Dither mux Bit[5:4]: R dithering register Bit[3:2]: G dithering register Bit[1:0]: B dithering register

Bit[7]:	Test enable

0:	Test disable

1:	Color bar enable

Bit[6]:	Rolling Bit[5]:	Transparent

Bit[4]:	Square black and white Bit[3:2]: Color bar style

00: Standard 8 color bar

01: Gradual change at vertical mode 1

10: Gradual change at horizontal

11: Gradual change at vertical mode 2

Bit[1:0]: Test select

00: Color bar

01: Random data

10: Square data

11: Black image

| 0x5061 | ISP SENSOR BIAS I | – R | ISP Sensor Bias Input |
| --- | --- | --- | --- |
| 0x5062 | ISP SENSOR GAIN I | – R | ISP Real Gain Input High Byte |
| 0x5063 | ISP SENSOR GAIN I | – R | ISP Real Gain Input Low Byte |

  - lens correction (LENC)

The main purpose of the LENC is to compensate for lens imperfection. According to the area where each pixel is located, the module calculates a gain for the pixel, correcting each pixel with its gain calculated to compensate for the luminance and color distribution due to lens curvature. Also, the LENC supports the subsample function in both horizontal and vertical directions. LENC is performed in the RGB domain. Luminance channel consists of 36 control points while each color channel consists of 25 control points.

##### figure 4-1	control points of luminance and color channels

| * B11/R11 | * | * |
| --- | --- | --- |
| * | * | * |
| * | * | * |

* G00

* G10

* G20

* G30

* G40

* G50

*

*

*

*

*

* G01

* G11

*

*

*

*

B00/R00 B10/R10 B20/R20 B30/R30 B40/R40

* G02

*

*

*

*

*

* B01/R01

*

* G03

*

*

*

*

*

* B02/R02

*

* G04

*

*

*

*

*

* B03/R03

*

*

*

*

*

* G55

* B04/R04

*

*

*

* G05

* B44/R44

3660_DS_4_1

figure 4-2

luminance compensation level calculation

64

min LENC gain

LENC gain

SensorGainThreshold1	SensorGainThreshold2

sensor gain	3660_DS_4_2

##### table 4-2	LENC control registers

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

|  |  |  |  | Bit[7]: | LENC correction enable |
| --- | --- | --- | --- | --- | --- |
| 0x5000 | ISP CONTROL 00 | 0x06 | RW |  | 0: Disable |
|  |  |  |  |  | 1: Enable |
|  |  |  |  | Bit[2:0]: | br h scale[10:8] |
|  |  |  |  |  | Reciprocal of horizontal step for BR |
| 0x5842 | BR HSCALE | 0x00 | RW |  | channel. BR channel in whole image is |
|  |  |  |  |  | divided into 5x5 blocks. The step is used |
|  |  |  |  |  | to point to the border of the adjacent block |
|  |  |  |  | Bit[7:0]: | br h scale[7:0] |
|  |  |  |  |  | Reciprocal of horizontal step for BR |
| 0x5843 | BR HSCAL | 0xBD | RW |  | channel. BR channel in whole image is |
|  |  |  |  |  | divided into 5x5 blocks. The step is used |
|  |  |  |  |  | to point to the border of the adjacent block |
|  |  |  |  | Bit[2:0]: | br v scale[10:8] |
|  |  |  |  |  | Reciprocal of vertical step for BR channel. |
| 0x5844 | BR VSCALE | 0x00 | RW |  | BR channel in whole image is divided into |
|  |  |  |  |  | 5x5 blocks. The step is used to point to |
|  |  |  |  |  | the border of the adjacent block |
|  |  |  |  | Bit[7:0]: | br v scale[7:0] |
|  |  |  |  |  | Reciprocal of vertical step for BR channel. |
| 0x5845 | BR VSCALE | 0xFE | RW |  | BR channel in whole image is divided into |
|  |  |  |  |  | 5x5 blocks. The step is used to point to |
|  |  |  |  |  | the border of the adjacent block |
|  |  |  |  | Bit[2:0]: | g h scale[10:8] |
|  |  |  |  |  | Reciprocal of horizontal step for G |
| 0x5846 | G HSCALE | 0x00 | RW |  | channel. G channel in whole image is |
|  |  |  |  |  | divided into 6x6 blocks. The step is used |
|  |  |  |  |  | to point to the border of the adjacent block |
|  |  |  |  | Bit[7:0]: | g h scale[7:0] |
|  |  |  |  |  | Reciprocal of horizontal step for G |
| 0x5847 | G HSCAL | 0xFC | RW |  | channel. G channel in whole image is |
|  |  |  |  |  | divided into 6x6 blocks. The step is used |
|  |  |  |  |  | to point to the border of the adjacent block |
|  |  |  |  | Bit[2:0]: | g v scale[10:8] |
|  |  |  |  |  | Reciprocal of vertical step for G channel. |
| 0x5848 | G VSCALE | 0x00 | RW |  | G channel in whole image is divided into |
|  |  |  |  |  | 6x6 blocks. The step is used to point to |
|  |  |  |  |  | the border of the adjacent block |
|  |  |  |  | Bit[7:0]: | g v scale[7:0] |
|  |  |  |  |  | Reciprocal of vertical step for G channel. |
| 0x5849 | G VSCALE | 0xA9 | RW |  | G channel in whole image is divided into |
|  |  |  |  |  | 6x6 blocks. The step is used to point to |
|  |  |  |  |  | the border of the adjacent block |

  - white balance

The raw R, G, and B values of a white object detected by the OV3660 vary with the spectrum of the light source. The spectrum of the light source is usually described by "color temperature," which is the surface temperature of a black body radiating the equivalent spectrum. The white balance process applies a different gain on each color channel to make the white object rendered white in the image.

The OV3660 has a built-in automatic white balance (AWB) algorithm that automatically adjusts the gain of each color channel to achieve white balance. The AWB algorithm works in either simple or advanced mode. The on-chip AWB algorithm can also be disabled so a backend processor can control the gain of each channel.

##### table 4-3	white balance registers

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

0x5001	ISP CONTROL 01	1’b1	RW

0x5183	AWB CONTROL 03	0x90	RW

Bit[0]:	Auto white balance (AWB) enable

0:	Disable (manual AWB mode) where AWB is controlled by an external backend processor

1:	Enable, where Rgain, Ggain, and Bgain are updated automatically

Bit[7]:	AWB algorithm select

0:	Select advanced AWB mode 1:	Select simple AWB mode

| 0x3400 0x3401 | RGAIN | 0x400 | RW | Bit[3:0]: Bit[7:0]: | Red channel gain[11:8] Red channel gain[7:0] |
| --- | --- | --- | --- | --- | --- |
| 0x3402 0x3403 | GGAIN | 0x400 | RW | Bit[3:0]: Bit[7:0]: | Green channel gain[11:8] Green channel gain[7:0] |
| 0x3404 0x3405 | BGAIN | 0x400 | RW | Bit[3:0]: Bit[7:0]: | Blue channel gain[11:8] Blue channel gain[7:0] |

###### simple AWB

The simple AWB algorithm is based on the gray world assumption, meaning the sensor will make the R, G and B average of all pixels equal to each other by adjusting the gain of each color channel.

###### advanced AWB

The advanced AWB algorithm adjusts the R, G and B gain based on the color temperature of the ambient light. It will make the R, G and B channel average of gray pixels equal to each other.

      - advanced AWB calibration

To identify the gray pixel over the color temperature range, the characteristics of the gray object must be calibrated first using the target lens.

##### table 4-4	AWB calibration registers (sheet 1 of 3)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

0x5186	AWB_M_RNG	0x10	RW

0x5187	AWB_L_XRNG	0x10	RW

0x5188	AWB_H_YRNG	0x10	RW

0x5189	AWB_M_X	0x40	RW

Bit[7:0]: AWB_M_RNG[7:0]

Tolerance of AWB_M_X and AWB_M_Y in the middle color temperature range.

A tolerance that is too small may result in an unstable AWB, while a tolerance that is too great may result in an inaccurate AWB.

Bit[7:0]: AWB_L_XRNG[7:0]

Tolerance of AWB_L_X in the low color temperature range, where AWB_L_X is the X characteristic of the gray object in the low color temperature range.

A tolerance that is too small may result in an unstable AWB, while a tolerance that is too great may result in an inaccurate AWB. The typical value ranges from 0x08~0x18.

Bit[7:0]: AWB_H_YRNG[7:0]

Tolerance of AWB_H_Y in the high color temperature range, where AWB_H_Y is the Y characteristic of the gray object in the high color temperature range.

A tolerance that is too small may result in an unstable AWB, while a tolerance that is too great may result in an inaccurate AWB. The typical value ranges from 0x08~0x10.

Bit[7:0]: AWB_M_X[7:0]

X characteristics of the gray object in the middle color temperature range.

When AWB_M_X increases, gray will shift towards blue in low color temperature light, or towards red in high color temperature light.

When AWB_M_X decreases, gray will shift towards yellow in low color temperature light or towards cyan in high color temperature light.

If AWB_M_X is too big or too small, the AWB algorithm may fail to identify the gray object and the result may be unstable and unpredictable.

##### table 4-4	AWB calibration registers (sheet 2 of 3)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

0x518A	AWB_M_Y	0x40	RW

0x518B	AWB_L_K	0x00	RW

0x518C	AWB_H_K	0x00	RW

0x518D	AWB_H_LMT	0x00	RW

0x518E	AWB_L_LMT	0x00	RW

Bit[7:0]: AWB_M_Y[7:0]

Y characteristics of the gray object in the middle color temperature range.

When AWB_M_Y increases, gray will shift towards blue in low color temperature light, or towards red in high color temperature light.

When AWB_M_Y decreases, gray will shift towards yellow in low color temperature light or towards cyan in high color temperature light.

If AWB_M_Y is too big or too small, the AWB algorithm may fail to identify the gray object and the result may be unstable and unpredictable.

Bit[7:0]: AWB_L_K

K characteristic of the gray object in the low color temperature range

When AWB_L_K increases/decreases, the gray color will shift slightly toward yellow/blue, respectively, in the low color temperature range.

Bit[7:0]: AWB_H_K

K characteristic of the gray object in the high color temperature range

When AWB_H_K increases/decreases, the gray color will shift slightly toward cyan/red, respectively, in the high color temperature range.

Bit[7:0]: AWB_H_LMT[7:0]

Lower limit of AWB_H_X, where AWB_H_X is the X characteristic of the gray object in the high color temperature range.

A smaller AWB_H_LMT value covers a greater upper limit of the color temperature; however, it also results in a less accurate white balance.

Bit[7:0]: AWB_L_LMT[7:0]

Lower limit of AWB_L_Y, where AWB_L_Y is the Y characteristic of the gray object in the low color temperature range.

A smaller AWB_L_LMT value covers a smaller lower limit of the color temperature; however, it also results in a less accurate white balance.

##### table 4-4	AWB calibration registers (sheet 3 of 3)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

| 0x518F | AWB_CTRL_D | 0x20 | RW Bit[7:0]: AWB_CTRL_D Day split |
| --- | --- | --- | --- |
| 0x5190 | AWB_CTRL_A | 0x20 | RW Bit[7:0]: AWB_CTRL_A A split |

0x5191	AWB_DATA_ULMT	0xFF	RW

0x5192	AWB_DATA_LLMT	0x00	RW

Bit[7:0]: AWB_DATA_ULMT

Pixels with an output value greater than AWB_DATA_ULMT are excluded from the AWB statistics

Bit[7:0]: AWB_DATA_LLMT

Pixels with an output value smaller than AWB_DATA_LLMT are excluded from the AWB statistics

      - AWB control

##### table 4-5	AWB control (sheet 1 of 2)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

0x5181	AWB_STEP	0x58	RW

0x5182	AWB_UPDATE	0x11	RW

Bit[7:4]: AWB_STABLE_RNG_IN[3:0]

AWB will adjust the gain of each color channel until the difference between each channel is no more than AWB_STABLE_RNG_IN[3:0]

Bit[3:0]: AWB_STABLE_RNG_OUT[3:0]

AWB will start to adjust the gain once the difference between each channel is greater than AWB_STABLE_RNG_OUT[3:0]

0:	Disable (manual AWB mode) where AWB is controlled by an external backend processor

1:	Enable, where Rgain, Ggain, and Bgain are updated automatically

Bit[7:0]: Range of red channel gain

{AWB_RGAIN_RNG_[3:0], 4’h0} < RGain <

{AWB_RGAIN_RNG[7:4], 4’hF}

##### table 4-5	AWB control (sheet 2 of 2)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

0x5183	AWB_CTRL02	0x90	RW

0x5184	AWB_CTRL03	0x25	RW

Bit[7]:	AWB mode

0:	Select advanced AWB 1:	Select simple AWB

Bit[5]:	Green channel gain enable

Green channel gain must be enabled to avoid false color of bright objects in reddish or bluish light.

0:	Only adjust red and blue channel gain 1:	Adjust red, green and blue channel

gain

Bit[4]:	Fast adjustment enable in simple AWB mode

0:	Disable, AWB speed is slow

1:	Enable, AWB adjustment is fast for fast scene changes

Bit[3:2]: AWB statistics window selection 00: Full image

01: Exclude 8 rows and columns at each image boundary

10: Exclude 1/8 of the total rows and columns at each image boundary

11: Exclude 1/4 of the total rows and columns at each image boundary

Bit[1:0]: AWB debug control

Changing these register bits is not recommended.

Bit[7:0]: AWB debug control

Changing this register value is not recommended.

      - advanced AWB tuning guidelines

Before starting the calibration, the final lens shading correction (if needed) should be applied.

- Set the camera to full resolution raw format.

- Disable the AWB and set the R, G, and B gain to 1x.

- Apply the final lens shading correction setting.

- Capture an image of a uniform gray scene (e.g., gray 18 chart in raw data format under A (color temperature around 2950K), CWF (color temperature around 4150K), and D65 (color temperature around 6500K) light.

- Run the OVT AWB calibration tool.

- Select the image following the guide and generate the setting.

- Apply the normal YUV setting with the AWB setting and check the AWB performance under different light conditions.

The procedure shown above are the basic steps required to adjust the OVT Advanced AWB. For a more detailed procedure, refer to the OVTATool User Guide.

###### AWB stable range and gain range

From an unstable state, the AWB algorithm will stop to adjust the AWB gain when the difference between each channel is no more than the threshold, AWB_STABLE_RNG_IN[3:0]. From a stable state, the AWB algorithm will start to adjust gain once the difference between each channel is greater than the threshold, AWB_STABLE_RNG_OUT[3:0].

The R, G and B gain can be further limited by register AWB_RGAIN_RNG, AWB_GGAIN_RNG and AWB_BGAIN_RNG, respectively.

##### table 4-6	AWB stable range registers

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

0x5185	AWB_STABLE_RNG	0x24	RW

Bit[7:4]: AWB_STABLE_RNG_IN[3:0]

AWB will adjust the gain of each color channel until the difference between each channel is no more than AWB_STABLE_RNG_IN[3:0]

Bit[3:0]: AWB_STABLE_RNG_OUT[3:0]

AWB will start to adjust the gain once the difference between each channel is greater than AWB_STABLE_RNG_OUT[3:0]

0:	Disable (manual AWB mode) where AWB is controlled by an external backend processor

1:	Enable, where Rgain, Ggain, and Bgain are updated automatically

| 0x5193 | AWB_RGAIN_RNG | 0x00 | RW | Bit[7:0]: | Range of red channel gain {AWB_RGAIN_RNG_[3:0], 4’h0} < RGain < {AWB_RGAIN_RNG[7:4], 4’hF} |
| --- | --- | --- | --- | --- | --- |
| 0x5194 | AWB_GGAIN_RNG | 0x00 | RW | Bit[7:0]: | Range of green channel gain {AWB_GGAIN_RNG_[3:0], 4’h0} < GGain < {AWB_GGAIN_RNG[7:4], 4’hF} |
| 0x5195 | AWB_BGAIN_RNG | 0x00 | RW | Bit[7:0]: | Range of blue channel gain {AWB_BGAIN_RNG_[3:0], 4’h0} < BGain < {AWB_BGAIN_RNG[7:4], 4’hF} |

  - gamma

The main purpose of the Gamma (GMA) function is to accommodate for the non-linear characteristics of the display device. The gamma curve is constructed by 16 linear segments as shown in figure 4-3.

##### figure 4-3	gamma curve

255

GAMST15 GAMST14

SLOP GAMSP15

GAMSP16

GAMST3

GAMST2 GAMST1

GAMSP3 GAMSP2

GAMSP1

XREF1

XREF2

XREF3

XREF14

XREF15

255

3660_DS_4_3

There are sixteen gamma control registers, including fifteen curve starting points and one highest slope. All of these control registers are configurable.

The slope of the highest segment (register 0x5490) is calculated by the following equation:

### SLOPE = register 0x5490[7:0] = (255 – GAM15[7:0] + 1)  40/30

Note that the gamma starting point and the highest slope should be matched; otherwise, there will be a discontinuous point in the gamma curve. refer to table 4-7 for the correspondence between the gamma curve and the control registers.

##### table 4-7	gamma parameters and corresponding registers

| gamma vertical starting point |  | horizontal reference point |  |
| --- | --- | --- | --- |
| name | register | name | value (decimal) |

| GAM1 | 0x5481 | XREF1 | 4 |
| --- | --- | --- | --- |
| GAM2 | 0x5482 | XREF2 | 8 |
| GAM3 | 0x5483 | XREF3 | 16 |

table 4-8

gamma control registers (sheet 1 of 2)

##### table 4-7	gamma parameters and corresponding registers

| gamma vertical starting point |  | horizontal reference point |  |
| --- | --- | --- | --- |
| name | register | name | value (decimal) |

| GAM4 | 0x5484 | XREF4 | 32 |
| --- | --- | --- | --- |
| GAM5 | 0x5485 | XREF5 | 40 |
| GAM6 | 0x5486 | XREF6 | 48 |
| GAM7 | 0x5487 | XREF7 | 56 |
| GAM8 | 0x5488 | XREF8 | 64 |
| GAM9 | 0x5489 | XREF9 | 72 |
| GAM10 | 0x548A | XREF10 | 80 |
| GAM11 | 0x548B | XREF11 | 96 |
| GAM12 | 0x548C | XREF12 | 112 |
| GAM13 | 0x548D | XREF13 | 144 |
| GAM14 | 0x548E | XREF14 | 176 |
| GAM15 | 0x548F | XREF15 | 208 |
| highest segment slope | 0x5490 |  |  |

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

|  |  |  |  | Bit[5]: | Gamma enable |
| --- | --- | --- | --- | --- | --- |
| 0x5000 | ISP CONTROL 00 | 0x06 | RW |  | 0: Disable GMA |
|  |  |  |  |  | 1: Enable GMA |
| 0x5481 | GAM1 | 0x26 | RW | Bit[7:0]: | Gamma1 |
| 0x5482 | GAM2 | 0x35 | RW | Bit[7:0]: | Gamma2 |
| 0x5483 | GAM3 | 0x48 | RW | Bit[7:0]: | Gamma3 |
| 0x5484 | GAM4 | 0x63 | RW | Bit[7:0]: | Gamma4 |
| 0x5485 | GAM5 | 0x6E | RW | Bit[7:0]: | Gamma5 |
| 0x5486 | GAM6 | 0x77 | RW | Bit[7:0]: | Gamma6 |
| 0x5487 | GAM7 | 0x80 | RW | Bit[7:0]: | Gamma7 |
| 0x5488 | GAM8 | 0x88 | RW | Bit[7:0]: | Gamma8 |
| 0x5489 | GAM9 | 0x8F | RW | Bit[7:0]: | Gamma9 |

##### table 4-8	gamma control registers (sheet 2 of 2)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

| 0x548A | GAM10 | 0x96 | RW | Bit[7:0]: | Gamma10 |
| --- | --- | --- | --- | --- | --- |
| 0x548B | GAM11 | 0xA3 | RW | Bit[7:0]: | Gamma11 |
| 0x548C | GAM12 | 0xAF | RW | Bit[7:0]: | Gamma12 |
| 0x548D | GAM13 | 0xC5 | RW | Bit[7:0]: | Gamma13 |
| 0x548E | GAM14 | 0xD7 | RW | Bit[7:0]: | Gamma14 |
| 0x548F | GAM15 | 0xE8 | RW | Bit[7:0]: | Gamma15 |
| 0x5490 | SLOPE | 0x0F | RW | Bit[7:0]: | Slope of the highest segment |

  - defect pixel cancellation (DPC)

Due to processes and other reasons, pixel defects in the sensor array will occur.

The value of the defect pixel usually has an abrupt change compared to normal pixels. The DPC function is designed to recover these white or black pixels while maintaining image quality.

##### table 4-9	DPC control registers

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

0x5000	ISP CONTROL 00	0x06	RW

Bit[2]:	Black pixel cancellation enable 0:	Disable

1:	Enable

Bit[1]:	White pixel cancellation enable 0:	Disable

1:	Enable

  - sharpness control and de-noise

The OV3660 supports built-in sharpness and de-noise control.

For sharpness, the OV3660 allows the user to select the level of edge enhancement. The sensor first detects the edges of an image and through a complex selection process, the OV3660 amplifies the edges to enhance the sharpness of the image.

The OV3660 supports both auto and manual mode control. If auto mode is selected, the sensor monitors the brightness and noise levels of the image and, based on these two statistics, adjusts the magnitude of the sharpness automatically.

If manual mode is selected by setting register bit 0x5308[6] = 1, the sharpness level can be manually controlled by register 0x5302.

The examples below show sample settings for auto sharpness, sharpness off, and five different manual sharpness levels.

@@ Sharpness Auto 78 5308 00 40

78 5300 08 ;8x

78 5301 30

78 5302 10

78 5303 00

78 5309 08

78 530a 30

78 530b 04

78 530c 06

@@ Sharpness OFF 78 5308 40 40

78 5302 00

@@ Sharpness 1

78 5308 40 40

78 5302 02

@@ Sharpness 2

78 5308 40 40

78 5302 04

@@ Sharpness 3

78 5308 40 40

78 5302 08

@@ Sharpness 4

78 5308 40 40

78 5302 0c

@@ Sharpness 5

78 5308 40 40

78 5302 10

To reduce noise, the OV3660 supports both auto and manual noise suppression modes. If auto mode is selected, the register value of de-noise level is automatically updated by the sensor. If manual mode is selected by setting register bit 0x5308[4] = 1, the user can manually adjust the de-noise level using register 0x5306.

The examples below show sample settings for auto de-noise and manual de-noise control with five different de-noise strengths.

@@ De-Noise Auto 78 5308 00 10

78 5304 08 ;8x

78 5305 30

78 5306 08

78 5307 16

@@ De-Noise 0

78 5306 00

78 5308 10 10

@@ De-Noise 1

78 5306 02

78 5308 10 10

@@ De-Noise 2

78 5306 04

78 5308 10 10

@@ De-Noise 3

78 5306 08

78 5308 10 10

@@ De-Noise 4

78 5306 0c

78 5308 10 10

@@ De-Noise 5

78 5306 10

78 5308 10 10

0x530F

SHARPEN THRESHOLD AUTO

–

R

Sharpen Threshold Auto Read

##### table 4-10	sharpness and de-noise control registers

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

| 0x5300 | SHARPENMT THRESHOLD 1 | 0x08 | RW | Sharpen MT Threshold 1 |
| --- | --- | --- | --- | --- |
| 0x5301 | SHARPENMT THRESHOLD 2 | 0x48 | RW | Sharpen MT Threshold 2 |
| 0x5302 | SHARPENMT OFFSET1 | 0x18 | RW | Sharpen MT Offset1 (Y edge mt manual setting when register |
|  |  |  |  | 0x5308[6] = 1) |
| 0x5303 | SHARPENMT OFFSET2 | 0x0E | RW | Sharpen MT Offset2 |
| 0x5304 | DNS THRESHOLD 1 | 0x08 | RW | DNS Threshold 1 |
| 0x5305 | DNS THRESHOLD 2 | 0x48 | RW | DNS Threshold 2 |
| 0x5306 | DNS OFFSET1 | 0x09 | RW | DNS Offset1 (DNS threshold manual setting when |
|  |  |  |  | register 0x5308[4] = 1) |
| 0x5307 | DNS OFFSET2 | 0x16 | RW | DNS Offset2 |
| 0x5308 | EDGE DNS CTRL | 0x25 | RW | Bit[6]: Edge MT manual enable Bit[4]: DNS manual enable |
|  |  |  |  | Bit[2:0]: Threshold for BR sharpen |
| 0x5309 | SHARPENTH THRESHOLD 1 | 0x08 | RW | Sharpen TH Threshold 1 |
| 0x530A | SHARPENTH THRESHOLD 2 | 0x48 | RW | Sharpen TH Threshold 2 |
| 0x530B | SHARPENTH OFFSET1 | 0x04 | RW | Sharpen TH Offset1 (sharpen threshold manual setting when |
|  |  |  |  | register 0x5308[6] = 1) |
| 0x530C | SHARPENTH OFFSET2 | 0x06 | RW | Sharpen TH Offset2 |
| 0x530D | EDGE MT AUTO | – | R | Edge MT Auto Read |
| 0x530E | DNS THRESHOLD AUTO | – | R | DNS Threshold Auto Read |

  - color matrix (CMX)

note

Color matrix tuning tool is available as part of the OVTATool. Please contact your local FAE for the latest version of the tool.

The OV3660 supports a 3x3 color matrix. The general purpose of the color matrix is to convert color space from the RGB domain to the YUV domain. However, for OmniVision, the CMX is not solely for color space conversion, but also for crosstalk reduction. Therefore, during conversion, a color matrix should be applied first to eliminate crosstalk, which is generally induced by the microlens and the color filter. The calculation is shown below:

### [RGB] = [color correction]  [R0G0B0]

where [R0G0B0] is the primitive response from the sensor without any correction and [RGB] is the RGB response after OmniVision’s color correction.

Then, the corrected RGB signal can be converted to YUV color space by the RGB-to-YUV conversion matrix:

### [YUV] = [RGB-to-YUV conversion matrix]  [RGB]

Combining the equation, we get:

### [YUV] = [RGB-to-YUV conversion matrix]  [color correction]  [R0G0B0] [YUV] = [combined matrix]  [R0G0B0]

The combined matrix can be described as:

[combined matrix] =

cmx4	cmx5	cmx6

cmx7	cmx8	cmx9

[cmx1	cmx2	cmx3 ]

=

reg0x5384

reg0x5387

[reg0x5381

reg0x5386

reg0x5389

reg0x5382

reg0x5385

reg0x5388

#### reg0x5383 ]

The sign bit is assigned by registers 0x538B and 0x538A. Register 0x538B[0] → sign bit of CMX1

Register 0x538B[1] → sign bit of CMX2 Register 0x538B[2] → sign bit of CMX3 Register 0x538B[3] → sign bit of CMX4 Register 0x538B[4] → sign bit of CMX5 Register 0x538B[5] → sign bit of CMX6

Register 0x538B[6] → sign bit of CMX7 Register 0x538B[7] → sign bit of CMX8 Register 0x538A[0] → sign bit of CMX9

4.8 auto color saturation adjust

table 4-11	CMX control registers

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

|  |  |  |  | Bit[1]: | Color matrix enable |
| --- | --- | --- | --- | --- | --- |
| 0x5001 | ISP CONTROL 01 | 0x01 | RW |  | 0: Disable |
|  |  |  |  |  | 1: Enable |
| 0x5381 | CMX1 | 0x20 | RW | Bit[7:0]: | CMX1 for Y |
| 0x5382 | CMX2 | 0x64 | RW | Bit[7:0]: | CMX2 for Y |
| 0x5383 | CMX3 | 0x08 | RW | Bit[7:0]: | CMX3 for Y |
| 0x5384 | CMX4 | 0x30 | RW | Bit[7:0]: | CMX4 for U |
| 0x5385 | CMX5 | 0x90 | RW | Bit[7:0]: | CMX5 for U |
| 0x5386 | CMX6 | 0xC0 | RW | Bit[7:0]: | CMX6 for U |
| 0x5387 | CMX7 | 0xA0 | RW | Bit[7:0]: | CMX7 for V |
| 0x5388 | CMX8 | 0x98 | RW | Bit[7:0]: | CMX8 for V |
| 0x5389 | CMX9 | 0x08 | RW | Bit[7:0]: | CMX9 for V |
| 0x538A | CMXSIGN | 0x01 | RW | CMXsign Bit[0]: | CMX9 sign |
|  |  |  |  | CMXsign |  |
|  |  |  |  | Bit[7]: | CMX8 sign |
|  |  |  |  | Bit[6]: | CMX7 sign |
|  |  |  |  | Bit[5]: | CMX6 sign |
| 0x538B | CMXSIGN | 0x98 | RW | Bit[4]: | CMX5 sign |
|  |  |  |  | Bit[3]: | CMX4 sign |
|  |  |  |  | Bit[2]: | CMX3 sign |
|  |  |  |  | Bit[1]: | CMX2 sign |
|  |  |  |  | Bit[0]: | CMX1 sign |

The main function of the auto color saturation adjust is to adjust the U/V channel value according to sensor gain. It supports both manual and auto modes.

###### 4.8.1 manual mode

By setting 0x5580[1] to 1 and 0x5588[6] to 1, auto color saturation adjust is controlled only by register 0x5583[7:0] and 0x5584[7:0] for U and V gains.

auto color saturation adjust in

###### 4.8.2 auto mode

When the auto color saturation adjust is set for auto mode (0x5580[1]=1 and 0x5588[6]=0), the auto color saturation adjust curve parameters (see figure 4-4) should be entered into the corresponding registers. The auto color saturation adjust parameters, auto color saturation adjust threshold1, auto color saturation adjust threshold2, and offset low, offset high should be entered into the registers to set the curve. To get these values, first set the values of auto color saturation adjust threshold1, auto color saturation adjust threshold2, offset low and offset high. Then, calculate the values of a and k as follows:

a = offset high + (offset high – offset low) / (UV adj th2 – UV adj th1)

Registers to be changed:

- auto color saturation adjust threshold1[8:0] = registers 0x5589[7:0]

- auto color saturation adjust threshold2[8:0] = registers {0x558A[0], 0x558B[7:0]}

- offset high = register 0x5583[7:0] (when 0x5580[1]=1 and 0x5588[6]=0)

- offset low = register 0x5584[7:0] (when 0x5580[1]=1 and 0x5588[6]=0)

figure 4-4	auto color saturation adjust graph

offset_high

offset_low

auto color saturation adjust threshold1

auto color saturation adjust threshold2

AGC gain	3660_DS_4_4

y = a - kx

k = (offset high – offset low) / (UV adj th2 – UV adj th1)

4.9 special digital effects (SDE)

The special digital effects (SDE) include b&w, sepia, greenish, bluish, reddish and negative. The SDE of the OV3660 also allows the user to adjust contrast, brightness, saturation, and hue. For normal, b&w, bluish, sepia, reddish, greenish and negative, set register bit 0x5001[7] = 1 to enable these special effects. Set register bit 0x5001[7] = 1 to fix UV values or select negative effect. Then, manually change the values of the two chrominance registers, U and V. See table 4-12 for sample images and register settings.

##### table 4-12	SDE image effects (sheet 1 of 2)

normal

Disable SDE by setting register 0x5001[7]=0

Turn off fixed UV by setting register 0x5580[4]=0 and register 0x5580[3]=0

Set U value by setting register 0x5583=0x40 (default)

Set V value by setting register 0x5583=0x40 (default)

b&w

Enable SDE by setting register 0x5001[7]=1

Turn on fixed UV by setting register 0x5580[4]=1 and register 0x5580[3]=1

Set U value by setting register 0x5583=0x80

Set V value by setting register 0x5583=0x80

bluish

Enable SDE by setting register 0x5001[7]=1

Turn on fixed UV by setting register 0x5580[4]=1 and register 0x5580[3]=1

Set U value by setting register 0x5583=0xF0

Set V value by setting register 0x5583=0x68

register settings

sample image

effect

##### table 4-12	SDE image effects (sheet 2 of 2)

| effect | sample image | register settings |
| --- | --- | --- |

sepia

register 0x5001[7]=1

Turn on fixed UV by setting register 0x5580[4]=1 and register 0x5580[3]=1

Set U value by setting register 0x5583=0x40

Set V value by setting register 0x5583=0xA0

reddish

Enable SDE by setting register 0x5001[7]=1

Turn on fixed UV by setting register 0x5580[4]=1 and register 0x5580[3]=1

Set U value by setting register 0x5583=0x80

Set V value by setting register 0x5583=0xC0

greenish

Enable SDE by setting register 0x5001[7]=1

Turn on fixed UV by setting register 0x5580[4]=1 and register 0x5580[3]=1

Set U value by setting register 0x5583=0x60

Set V value by setting register 0x5583=0x60

negative

Enable SDE by setting register 0x5001[7]=1

Enable negative effect by setting register 0x5580[6]=1

Enable SDE by setting

For contrast, brightness, saturation, and hue, see table 4-13 for applicable register settings.

##### table 4-13	image contrast/brightness/saturation/hue control

| effect | level/ degrees | enable bit | register control |
| --- | --- | --- | --- |

contrast

level -2

level -1

level 0

Enable SDE by setting register 0x5001[7]=1

Enable contrast control by setting register 0x5580[2]=1

register 0x5586=0x10; register 0x5587=0x10 register 0x5586=0x18; register 0x5587=0x18 register 0x5586=0x20; register 0x5587=0x00

level +1		register 0x5586=0x28; register 0x5587=0x18 Set register 0x5588=0x01

level +2	register 0x5586=0x30; register 0x5587=0x20

level -2

level -1

Enable SDE by setting register 0x5001[7]=1

register 0x5587=0x30; register 0x5588=0x08 register 0x5587=0x20; register 0x5588=0x08

brightness

level 0		register 0x5587=0x00; register 0x5588=0x00 Enable contrast control by

level +1

setting register 0x5580[2]=1

register 0x5587=0x20; register 0x5588=0x00

level +2	register 0x5587=0x30; register 0x5588=0x00

level -2

level -1

Enable SDE by setting register 0x5001[7]=1

register 0x5583=0x10; register 0x5588=0x10 register 0x5583=0x20; register 0x5588=0x20

saturation

level 0		register 0x5583=0x40; register 0x5588=0x40 Enable saturation control by

level +1

setting register 0x5580[1]=1

register 0x5583=0x60; register 0x5588=0x60

level +2	register 0x5583=0x70; register 0x5588=0x70

-60 degrees

register 0x5581=0x40; register 0x5582=0x6F; register 0x558A=0x02

-30 degrees	register 0x5581=0x6F; register 0x5582=0x40;

hue

0 degree

30 degrees

Enable SDE by setting register 0x5001[7]=1

Enable hue control by setting register 0x5580[0]=1

register 0x558A=0x02

register 0x5581=0x80; register 0x5582=0x00; register 0x558A=0x01

register 0x5581=0x6F; register 0x5582=0x40; register 0x558A=0x01

60 degrees	register 0x5581=0x40; register 0x5582=0x6F; register 0x558A=0x01

The register control values shown above are presented as examples. The user can determine the appropriate level of control according to their requirements. See table 4-14 for detailed descriptions of SDE-related registers.

##### table 4-14	SDE control registers

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

|  |  |  |  | Bit[7]: | Special digital effect enable |
| --- | --- | --- | --- | --- | --- |
| 0x5001 | ISP CONTROL 01 | 0x01 | RW |  | 0: Disable |
|  |  |  |  |  | 1: Enable |
| 0x5581 | SDE CTRL1 | 0x80 | RW | Bit[7:0]: | Hue cos coefficient |
| 0x5582 | SDE CTRL2 | 0x00 | RW | Bit[7:0]: | Hue sin coefficient |
|  |  |  |  | Bit[7:0]: | Saturation U when 0x5580[1]=1 and |
| 0x5583 | SDE CTRL3 | 0x40 | RW |  | 0x5588[6]=1, max value for auto color saturation adjust when 0x5580[1]=1 and |
|  |  |  |  |  | 0x5588[6]=0; or fixed U when 0x5580[3]=1 |
|  |  |  |  | Bit[7:0]: | Saturation V when 0x5580[1]=1 and |
| 0x5584 | SDE CTRL4 | 0x40 | RW |  | 0x5588[6]=1, min value for auto color saturation adjust when 0x5580[1]=1 and |
|  |  |  |  |  | 0x5588[6]=0; or Vreg when 0x5580[4]=1 |
| 0x5585 | SDE CTRL5 | 0x00 | RW | Bit[7:0]: | Yoffset for contrast when 0x5044[3]=1; or fixed Y when 0x5580[7]=1 |
| 0x5586 | SDE CTRL6 | 0x20 | RW | Bit[7:0]: | Y gain for contrast |
| 0x5587 | SDE CTRL7 | 0x00 | RW | Bit[7:0]: | Y bright for contrast |
|  |  |  |  | Bit[6]: | Auto color saturation adjust manual enable |
|  |  |  |  | Bit[5]: | Sign5 for hue V, cos |
|  |  |  |  | Bit[4]: | Sign4 for hue U, cos |
|  |  |  |  | Bit[3]: | Sign3 Y bright sign for contrast |
|  |  |  |  |  | 0: Keep Y bright sign |
| 0x5588 | SDE CTRL8 | 0x01 | RW | Bit[2]: | 1: Negative Y bright sign Sign2 |
|  |  |  |  |  | Y offset sign for contrast when 0x5044[3]=1 |
|  |  |  |  | Bit[1]: Bit[0]: | 0: Keep Y offset sign 1: Negative Y offset sign Sign1 for hue V, sin Sign0 for hue U, sin |
| 0x5589 | SDE CTRL9 | 0x01 | RW Bit[7:0]: Auto color saturation adjust threshold 1 Valid when 0x5580[1]=1 |  |  |
| 0x558A | SDE CTRL10 | 0x01 | RW Bit[0]: Auto color saturation adjust threshold 2[8] Valid when 0x5580[1]=1 |  |  |
| 0x558B | SDE CTRL11 | 0xFF | RW Bit[7:0]: Auto color saturation adjust threshold 2[7:0] Valid when 0x5580[1]=1 |  |  |
| 0x558C | SDE CTRL12 | – | R Bit[7:0]: Auto color saturation adjust value read out |  |  |

- image sensor output interface digital functions

  - compression engine

###### compression mode 1 timing

The whole frame has only one line. PCLK will be gated when there is no valid image data transmitted.

##### figure 5-1	compression mode 1 timing

HREF

PCLK

3660 DS 5 1

5.1.2 compression mode 2 timing

Compression data is transmitted with programmable line width. PCLK is free running. The last line may contain dummy

data to match the width. By default, the line number varies from frame to frame. The user can set register 4600[5] (0x4600) to ensure every frame has a fixed line number (programmable).

figure 5-2

compression mode 2 timing

HREF

VSYNC

3660_DS_5_2

###### 5.1.3 compression mode 3 timing

Compression data is transmitted with programmable width. The last line width maybe different from the other line (there is no dummy data). In each frame, the line number may be different.

##### figure 5-3	compression mode 3 timing

HREF

VSYNC

3660_DS_5_3

5.1.4 compression mode 4 timing

The width and height are fixed in each frame. The first two bytes are valid data length in every line, followed by valid image data. Dummy data (0xFF) may be used as padding at each line end if the current valid image data is less than the

line width.

figure 5-4

compression mode 4 timing

VSYNC to HREF delay

VSYNC

HREF

PCLK

valid data length

valid data

dummy data

D[9:2]

Len_h  Len_l  data	data	data

data	FF	FF	FF	FF

3660_DS_5_4

###### 5.1.5 compression mode 5 timing

The width and height are fixed in each frame. Every line begins with valid image data. Dummy data may be used as padding at each line end if the current valid image data is less than the line width. The last two bytes of every line is valid data length.

##### figure 5-5	compression mode 5 timing

VSYNC to HREF delay

VSYNC

HREF

PCLK

valid data

dummy data

valid data length

D[9:2]

data  data  data  data	FF

FF	FF	FF	Len_h Len_l

3660_DS_5_5

5.1.6 compression mode 6 timing

The width and height are fixed in each frame. Every line begins with valid image data. Dummy data may be used as padding at each line end if the current valid image data less than the line width.

figure 5-6

compression mode 6 timing

VSYNC to HREF delay

VSYNC

HREF

PCLK

valid data

D[9:2]

data  data  data  data	FF

FF	FF	FF	FF	FF

dummy data

3660_DS_5_6

###### 5.1.7 compression mode control

different in each frame 1:	In compression mode2 Compression height is

fixed in each frame

0x460C

VFIFO CTRL0C

0x20

RW

Bit[7:4]: Compression dummy data pad speed

0x460D

VFIFO CTRL0D

0x00

RW

Compression Pad Dummy Data

Bit[2:0]: Compression mode select

0x4713

COMPRESSION MODE SELECT

0x02

RW

001: Compression mode 1

010: Compression mode 2

011: Compression mode 3

100: Compression mode 4

101: Compression mode 5

110: Compression mode 6

Bit[7]:	input_format

0x4400

COMPRESSION CTRL00

0x81

RW

0:	YUV420

1:	YUV422

Bit[6:0]: JFIFO read speed control

table 5-1	compression control registers (sheet 1 of 2)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

| 0x3821 | COMPRESSION ENABLE | 0x00 | RW | Bit[5]: Compression enable |
| --- | --- | --- | --- | --- |
|  |  |  |  | Bit[5]: Compression output fixed height enable 0: In compression mode2 |
| 0x4600 | VFIFO CTRL00 | 0x80 | RW | Compression height is |

| 0x4602 | VFIFO HSIZE | 0x04 | RW | Compression Output Width High Byte |
| --- | --- | --- | --- | --- |
| 0x4603 | VFIFO HSIZE | 0x00 | RW | Compression Output Width Low Byte |
| 0x4604 | VFIFO VSIZE | 0x03 | RW | Compression Output Height High Byte |
| 0x4605 | VFIFO VSIZE | 0x00 | RW | Compression Output Height Low Byte |

| 0x471F | DVP HREF CTRL | 0x40 | RW HREF Minimum Blanking in Compression Mode23 |
| --- | --- | --- | --- |
| 0x4723 | DVP CTRL23 | 0x00 | RW DVP Compression Mode456 Skip Line Number |

##### table 5-1	compression control registers (sheet 2 of 2)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

0x4401	COMPRESSION CTRL01	0x01	RW

Bit[7:4]: SFIFO output buffer speed control

Bit[3]:	Read SRAM enable when blanking

0:	Disable

1:	Enable

Bit[2]:	Read SRAM at first blanking 0:	Disable

1:	Enable

|  |  |  |  | Bit[1:0]: | SFIFO read speed control |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Bit[7]: | jfifo_pwrdn |
|  |  |  |  | Bit[6]: | SFIFO pwrdn |
|  |  |  |  | Bit[5]: | Header output enable |
|  |  |  |  | Bit[4]: | Enable gated clock |
| 0x4404 | COMPRESSION CTRL04 | 0x24 | RW |  | 0: Disable gated clock |
|  |  |  |  |  | 1: Enable gated clock |
|  |  |  |  | Bit[3]: | Substitute 0xFF to 0xFE in QT |
|  |  |  |  | Bit[2:0]: | Quantization rounding |
|  |  |  |  |  | Bias: set value = Bias/8 |
| 0x4417 | JFIFO OVERFLOW | – | R | Bit[0]: | JFIFO overflow indicator |

  - parallel digital video port (DVP)

###### overview

The parallel DVP provides 10-bit parallel data output in all formats supported and extended features including:

      - compression mode

      - HSYNC mode

      - CCIR656 mode

      - test pattern output

##### table 5-2	parallel DVP control registers (sheet 1 of 3)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

| 0x4709 | DVP VSYNC WIDTH0 | 0x02 | RW | VSYNC WIDTH Line Unit |  |
| --- | --- | --- | --- | --- | --- |
| 0x470A | DVP VSYNC WIDTH1 | 0x00 | RW | VSYNC WIDTH PCLK Unit High Byte |  |
| 0x470B | DVP VSYNC WIDTH2 | 0x01 | RW | VSYNC WIDTH PCLK Unit Low Byte |  |
| 0x4711 | PAD LEFT CTRL | 0x00 | RW HSYNC Mode Left Padding Pixel Count Add padding data at start of a line |  |  |
| 0x4712 | PAD RIGHT CTRL | 0x00 | RW HSYNC Mode Right Padding Pixel Count Add padding data at end of a line |  |  |
| 0x4713 COMPRESSION MODE 0x02 SELECT |  |  | RW | Bit[2:0]: | Compression mode select 001: Compression mode 1 010: Compression mode 2 011: Compression mode 3 100: Compression mode 4 101: Compression mode 5 110: Compression mode 6 |
|  |  |  |  | Bit[3:0]: CCIR656 dummy line |  |
| 0x4715 | 656 DUMMY LINE | 0x00 | RW | number Control dummy line number at beginning of the frame |  |
| 0x4719 | CCIR656 CTRL | 0x01 | RW | Bit[1:0]: CCIR656 EAV/SAV option |  |
| 0x471B | HSYNC CTRL00 | 0x02 | RW | Bit[0]: HSYNC mode enable |  |

##### table 5-2	parallel DVP control registers (sheet 2 of 3)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

0x471D	DVP VSYNC CTRL	0x01	RW

Bit[2]:	vsync_sel1_clr Bit[1:0]: vsync_mode

00: VSYNC positive edge trigger by end of field, negative edge trigger by start of frame

01: VSYNC positive edge trigger by end of frame, the width define by register

10: VSYNC positive edge trigger by start of field, the width define by register

0x471F	DVP HREF CTRL	0x40	RW	HREF Minimum Blanking in Compression

Mode23

0x4721	VERTICAL START OFFSET

0x01	RW	Bit[3:0]: Vertical start delay between

video output and video input

0x4722	VERTICAL END OFFSET	0x00	RW	Bit[3:0]: Vertical end delay between

video output and video input

0x4723	DVP CTRL23	0x00	RW	DVP Compression Mode456 Skip Line

Number

Bit[7]:	SYNC code selection

0:	Auto generate sync code

1:	Sync code from register setting 0x4732~4735

Bit[6]:	F value in CCIR656 SYNC code when fixed f value

Bit[5]:	Fixed f value

0x4730	CCIR656 CTRL00	0x00	RW

Bit[4:3]: Blank toggle data options 00: Toggle data is

1'h040/1'h200

01: Use register setting 0x4736~0x4738

10: Blanking data always keep 0

Bit[2]:	Debug mode Bit[1]:	Clip data disable

Bit[0]:	CCIR656 mode enable

0x4731	CCIR656 CTRL01	0x01	RW	Bit[0]:	Blanking toggle data order

option

| 0x4732 | CCIR656 FS | 0x01 | RW | CCIR656 SYNC Code Frame Start |
| --- | --- | --- | --- | --- |
| 0x4733 | CCIR656 FE | 0x0F | RW | CCIR656 SYNC Code Frame End |

##### table 5-2	parallel DVP control registers (sheet 3 of 3)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

0x4734	CCIR656 LS	0x00	RW	CCIR6656 SYNC Code Line Start

0x4735	CCIR656 LE	0x00	RW	CCIR656 SYNC Code Line End

0x4736	CCIR656 CTRL6	0x00	RW	Bit[3:2]: Toggle data0[9:8]

Bit[1:0]: Toggle data1[9:8]

0x4737	CCIR656 CTRL7	0x00	RW	Bit[7:0]: Toggle data0[7:0]

0x4738	CCIR656 CTRL8	0x00	RW	Bit[7:0]: Toggle data1[7:0]

Bit[5]:	PCLK polarity

0:	Active low

1:	Active high

Bit[4]:	Reserved

Bit[3]:	Gate PCLK under VSYNC

0x4740	POLARITY CTRL00	0x20	RW

0x4741	TEST PATTERN	0x00	RW

0x4745	DATA ORDER	0x00	RW

Bit[2]:	Gate PCLK under HREF Bit[1]:	HREF polarity

0:	Active low

1:	Active high Bit[0]:	VSYNC polarity

0:	Active low

1:	Active high

Bit[2]:	Test pattern enable Bit[1]:	Test pattern select

0:	Output test pattern 0

1:	Output test pattern 1 Bit[0]:	Test pattern 8-bit/10-bit

0:	10-bit test pattern 1:	8-bit test pattern

Bit[2:1]: DVP order option for debug 00: D[9:0]

10: {D[7:0],D[9:8]}

x1:  {D[1:0],D[9:2]}

Bit[0]:	Output data order

0:	Normal output

1:	Reverse output data bit order

###### 5.2.2 DVP timing

##### figure 5-7	DVP frame timing diagram

(1)

VSYNC

(2)

(3)

(4)

(5)

HREF

HSYNC

(8)

(6)

(7)

(9)

D[9:0]

invalid data

3660_DS_5_7

note

The timing values shown in table 5-3 may vary depending upon register settings.

table 5-3	DVP timing specifications (sheet 1 of 2)

mode

timing

3 megapixel 2048x1536

1080p

1920x1080

- 3,599,434 tp

- 4,600 tp

- 32,062 tp

- 2,300 tp

- 27,924 tp

- 2,048 tp

- 252 tp

- 0 tp

- 252 tp

where tp = 1 sclk

- 2,702,760 tp

- 4,788 tp

- 127,396 tp

- 2,394 tp

- 19,476 tp

- 1,920 tp

- 474 tp

- 0 tp

- 474 tp

where tp = 1 sclk

##### table 5-3	DVP timing specifications (sheet 2 of 2)

timing

mode

720p

1280x720

XGA 1024x768

VGA 640x480

QVGA 320x240

  - 1,201,330 tp

  - 3,208 tp

  - 30,410 tp

  - 1,604 tp

  - 13,156 tp

  - 1,280 tp

  - 324 tp

  - 0 tp

  - 324 tp

where tp = 1 sclk

- 1,210,260 tp

- 3,068 tp

- 23,130 tp

- 1,534 tp

- 6,460 tp

- 1,024 tp

- 510 tp

- 0 tp

- 510 tp

where tp = 1 sclk

- 451,010 tp

- 1,808 tp

- 9,798 tp

- 904 tp

- 5,748 tp

- 640 tp

- 264 tp

- 0 tp

- 264 tp

where tp = 1 sclk

- 225,844 tp

- 1,772 tp

- 8,130 tp

- 886 tp

- 7,412 tp

- 320 tp

- 566 tp

- 0 tp

- 566 tp

where tp = 1 sclk

- serial camera control bus (SCCB) interface

The Serial Camera Control Bus (SCCB) interface controls the image sensor operation. Refer to the OmniVision Technologies Serial Camera Control Bus (SCCB) Specification for detailed usage of the serial control port.

  - data transfer protocol

The data transfer of the OV3660 follows the SCCB protocol.

  - message format

The OV3660 supports the message format shown in figure 6-1. The 7-bit address of the OV3660 is 0x3C by default but can be programmed using register 0x3100[7:1] which has a default of 0x78. The repeated START (Sr) condition is not shown in figure 6-2, but is shown in figure 6-3 and figure 6-4.

##### figure 6-1    message type

message type: 16-bit sub-address, 8-bit data, and 7-bit slave address

| S | slave address | R/W | A | sub address [15:8] | A | sub address [7:0] | A | data | A/A | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

index[15:8]	index[7:0]

from slave to master from master to slave

direction depends on operation

S	START condition P	STOP condition

Sr repeated START condition

A	acknowledge

A	negative acknowledge

3660_DS_6_1

  - read / write operation

The OV3660 supports four different read operations and two different write operations:

- a single read from random locations

- a sequential read from random locations

- a single read from current location

- a sequential read from current location

- single write to random locations

- sequential write starting from random location

The sub-address in the sensor automatically increases by one after each read/write operation.

In a single read from random locations, the master does a dummy write operation to desired sub-address, issues a repeated start condition and then addresses the camera again with a read operation. After acknowledging its slave

address, the camera starts to output data onto the SDA line as shown in figure 6-2. The master terminates the read operation by setting a negative acknowledge and stop condition.

index value M

3660_DS_6_2

If the host addresses the camera with read operation directly without the dummy write operation, the camera responds by setting the data from last used sub-address to the SDA line as shown in figure 6-3. The master terminates the read

operation by setting a negative acknowledge and stop condition.

figure 6-3

SCCB single read from current location

previous index value, K

index K + 1

index K + 2

3660_DS_6_3

The sequential read from a random location is illustrated in figure 6-4. The master does a dummy write to the desired sub-address, issues a repeated start condition after acknowledge from slave and addresses the slave again with read operation. If a master issues an acknowledge after receiving data, it acts as a signal to the slave that the read operation shall continue from the next sub-address. When master has read the last data byte, it issues a negative acknowledge

and stop condition.

figure 6-4

SCCB sequential read from random location

previous index value, K

index M

index	index

M + L - 1	M + L

figure 6-2	SCCB single read from random location

previous index value, K

index M

index M + 1

| S | slave address | 0 | A | sub address [15:8] | A | sub address [7:0] | A | Sr | slave address | 1 | A | data | A | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| S | slave address | 1 | A | data | A | P |
| --- | --- | --- | --- | --- | --- | --- |

| S | slave address | 1 | A | data | A | P |
| --- | --- | --- | --- | --- | --- | --- |

| S | slave address | 1 | A | sub address [15:8] | A | sub address [7:0] | A | Sr | slave address | 1 | A | data | A |  | data | A | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

index value M	L bytes of data	3660_DS_6_4

The sequential read from current location is similar to a sequential read from a random location. The only exception is that there is no dummy write operation. as shown in figure 6-5. The master terminates the read operation by setting a negative acknowledge and stop condition.

L bytes of data

3660_DS_6_5

The write operation to a random location is illustrated in figure 6-6. The master issues a write operation to the slave, sets the sub-address and data correspondingly after the slave has acknowledged. The write operation is terminated with a

stop condition from the master.

figure 6-6

SCCB single write to random location

previous index value, K	index M

index M + 1

index value M

3660_DS_6_6

The sequential write is illustrated in figure 6-7. The slave automatically increments the sub-address after each data byte.

The sequential write operation is terminated with stop condition from the master.

figure 6-7

SCCB sequential write to random location

previous index value, K

index M

M + L - 1

index

index

M + L

figure 6-5   SCCB sequential read from current location

previous index value, K

index K + 1

K + L - 1

index

index

K + L

| S | slave address | 1 | A | data | A | data | A |  | data | A | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| S | slave address | 0 | A | sub address [15:8] | A | sub address [7:0] | A | data | A/A | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| S | slave address | 0 | A | sub address [15:8] | A | sub address [7:0] | A | data | A |  | data | A/A | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

index value M	L bytes of data	3660_DS_6_7

  - SCCB timing

tF

tHIGH

tR

SIOC

tLOW

tHD:STA

tSU:DAT

tSU:STO

SIOD (IN)

tSU:STA

tAA

tHD:DAT

tBUF

SIOD (OUT)

3660_DS_6_8

tDH

table 6-1	SCCB interface timing specificationsab

figure 6-8	SCCB interface timing

| symbol | parameter | min | typ | max | unit |
| --- | --- | --- | --- | --- | --- |

| fSIOC | clock frequency |  | 400 | KHz |
| --- | --- | --- | --- | --- |
| tLOW | clock low period | 1.3 |  | µs |
| tHIGH | clock high period | 0.6 |  | µs |
| tAA | SIOC low to data out valid | 0.1 | 0.9 | µs |
| tBUF | bus free time before new start | 1.3 |  | µs |
| tHD:STA | start condition hold time | 0.6 |  | µs |
| tSU:STA | start condition setup time | 0.6 |  | µs |
| tHD:DAT | data in hold time | 0 |  | µs |
| tSU:DAT | data in setup time | 0.1 |  | µs |
| tSU:STO | stop condition setup time | 0.6 |  | µs |
| tR, tF | SCCB rise/fall times |  | 0.3 | µs |
| tDH | data out hold time | 0.05 |  | µs |

- SCCB timing is based on 400KHz mode

- timing measurement shown at the beginning of the rising edge or/and of the falling edge signifies 10%,

timing measurement shown in the middle of the rising/falling edge signifies 50%, timing measurement shown at the beginning of the rising edge or/and of the falling edge signifies 90%

- register tables

The following tables provide descriptions of the device control registers contained in the OV3660. For all registers enable/disable bits, ENABLE = 1 and DISABLE = 0.

  - system and IO pad control [0x3000 ~ 0x3052]

##### table 7-1	system and IO pad control registers (sheet 1 of 3)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

0x3000~

0x3003

SYSTEM RESET00~ SYSTEM RESET03

–	RW	Reset for Individual Blocks

0x3004~

0x3007

CLOCK ENABLE00	–	RW	Clock Enable Control for Individual Blocks

System Control

Bit[7]:	Software reset

Bit[6]:	Software power down

0x3008	SYSTEM CTROL0	0x02	RW

Bit[5]:  Reserve

Bit[4]: SRB clock SYNC enable Bit[3]: Isolation suspend select Bit[2:0]: Not used

0x300A	CHIP ID HIGH BYTE	0x36	R	Chip ID High Byte

0x300B	CHIP ID LOW BYTE	0x60	R	Chip ID Low Byte

Input/Output Control (0: input; 1: output) Bit[7:3]: Not used

0x3016	PAD OUTPUT ENABLE 00	0x22	RW

0x3017	PAD OUTPUT ENABLE 01	0x00	RW

0x3018	PAD OUTPUT ENABLE 02	0x00	RW

0x3019	PAD OUTPUT VALUE 00	0xF0	RW

Bit[2]:	FSIN output enable Bit[1]:	STROBE output enable Bit[0]:	Not used

Input/Output Control (0: input; 1: output) Bit[7]:	Not used

Bit[6]:	 VSYNC output enable Bit[5]:	HREF output enable Bit[4]:	 PCLK output enable Bit[3:0]: D[9:6] output enable

Input/Output Control (0: input; 1: output) Bit[7:2]: D[5:0] output enable Bit[1:0]: GPIO1 output enable

PAD Output Value Bit[7:3]: Not used Bit[2]:	FSIN

Bit[1]:	STROBE

Bit[0]:	Not used

##### table 7-1	system and IO pad control registers (sheet 2 of 3)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

0x301A	PAD OUTPUT VALUE 01	0x00	RW

0x301B	PAD OUTPUT VALUE 02	0x00	RW

0x301C	PAD SELECT 00	0x00	RW

0x301D	PAD SELECT 01	0x00	RW

0x301E	PAD SELECT 02	0x00	RW

0x302A	CHIP REVISION	0xB0	R

0x302C	PAD CONTROL	0x03	RW

GPIO Output Value 01 Bit[7]:	Reserved

Bit[6]:	VSYNC

Bit[5]:	HREF

Bit[4]:	PCLK Bit[3:0]: D[9:6]

GPIO Output Value 02 Bit[7:2]: D[5:0]

Bit[1:0]: Not used

Pad Selection Control Bit[7:3]: Not used Bit[2]:	IO FSIN select

Bit[1]:	 IO STROBE select Bit[0]:	Not used

Output Selection for GPIO Bit[7]:	Not used Bit[6]:	VSYNC select Bit[5]:	 HREF select Bit[4]:	PCLK select

Bit[3:0]: IO D[9:6] select

Pad select control

Bit[7:2]: IO D[5:0] select Bit[1:0]: Not used

Bit[7:4]: Process

0xB: BSI

Bit[3:0]: Chip revision

Pad Control

Bit[7:6]: Pad driving strength 00: 1x

01: 2x

10: 3x

11: 4x

Bit[5]:	pd_dato_en Bit[4:2]: Reserved

Bit[1]:	FSIN input enable Bit[0]:	 STROBE input enable

0x303A	SC PLLS CTRL0	0x00	RW	Bit[7]:	PLLS bypass

Bit[6:0]: Reserved

0x303B	SC PLLS CTRL1	0x1B	RW	Bit[7:5]: Not used

Bit[4:0]: PLLS multiplier

Bit[7]:	Not used

0x303C	SC PLLS CTRL2	0x11	RW

Bit[6:4]: PLL charge pump control Bit[3:0]: PLL system divider

##### table 7-1	system and IO pad control registers (sheet 3 of 3)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

0x303D	SC PLLS CTRL3	0x30	RW

0x3050	IO PAD VALUE	–	R

0x3051	IO PAD VALUE	–	R

0x3052	IO PAD VALUE	–	R

Bit[7:6]: Not used Bit[5:4]: PLLS predivider

00: /1

01: /1.5

10: /2

11: /3

Bit[3]:	Not used

Bit[2]:	PLLS root divider 0:	/1

1:	/2

Bit[1:0]: PLLS seld5

00: /1

01: /1

10: /2

11: /2.5

Read Pad Value Bit[7:4]: Not used Bit[3]:	PWDN

Bit[2]:	Not used

Bit[1]:	SIOC

Bit[0]:	Not used

Read Pad Value

Bit[7]:	OTP memory out Bit[6]:	VSYNC

Bit[5]:	HREF

Bit[4]:	PCLK Bit[3:0]: D[9:6]

Read Pad Value Bit[7:2]: D[5:0]

Bit[1:0]: Not used

  - SCCB control [0x3100 ~ 0x3108]

##### table 7-2	SCCB control registers

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

0x3100	SCCB_ID	0x78	RW	SCCB Slave ID

Bit[7:6]: Not used Bit[5]:	SRB reset

Bit[4]:	SCCB slave reset

0x3102	SCCB SYSTEM CTRL0	0x80	RW

0x3103	SCCB SYSTEM CTRL1	0x11	RW

0x3108	SYSTEM ROOT DIVIDER	0x16	RW

Bit[3]:	rst_pon_sccb Bit[2]:	Reserved

Bit[1]:	Not used

Bit[0]:	PLL reset

Bit[7:2]: Not used

Bit[1]:	Select PLL input clock

0:	From pad clock 1:	From pre divider

(clock modulator) Bit[0]:	Power up reset disable

Bit[7:6]: Not used

Bit[5:4]: PCLK root divider

00: PCLK = pll_clki 01: PCLK = pll_clki/2 10: PCLK = pll_clki/4 11: PCLK = pll_clki/8

Bit[3:2]: SCLK2x root divider

00: SCLK2x = pll_clki 01: SCLK2x = pll_clki/2 10: SCLK2x= pll_clki/4

11: SCLK2x= pll_clki/8 Bit[1:0]: SCLK root divider

00: SCLK = pll_clki 01: SCLK = pll_clki/2 10: SCLK= pll_clki/4

11: SCLK= pll_clki/8

  - SRB control [0x3200 ~ 0x3213]

##### table 7-3	SRB control registers

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

| 0x3200 | GROUP ADDR0 | 0x40 | RW | SRAM Group Address0 |
| --- | --- | --- | --- | --- |
| 0x3201 | GROUP ADDR1 | 0x4A | RW | SRAM Group Address1 |
| 0x3202 | GROUP ADDR2 | 0x54 | RW | SRAM Group Address2 |
| 0x3203 | GROUP ADDR3 | 0x5E | RW | SRAM Group Address3 |
|  |  |  |  | SRM Group Access |

0x3212	SRM GROUP ACCESS	–	W

0x3213	SRM GROUP STATUS	–	R

Bit[7]:	Group launch enable Bit[6]:	 Test mode access group Bit[5]:	Group launch

Bit[4]:	Group hold end Bit[3:0]: Group ID

00x: Group for register access

011: Group to hold register address of embedded line SOF

100: Group to hold register address of embedded line EOF

101: Test mode for store register value to memory

110: Test mode for restore register value from memory

111: Group for write mask address

SRM Group Status Bit[7]:	Store

Bit[6]:	 Restore Bit[5]:	 Group hold Bit[4]:	Group launch Bit[3]:	Group write Bit[2:0]: Group select

  - AWB gain control [0x3400 ~ 0x3406]

##### table 7-4	AWB gain control registers (sheet 1 of 2)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

| 0x3400 | AWB R GAIN | 0x04 | RW Bit[7:4]: Not used Bit[3:0]: AWB R gain[11:8] |
| --- | --- | --- | --- |
| 0x3401 | AWB R GAIN | 0x00 | RW Bit[7:0]: AWB R gain[7:0] |
| 0x3402 | AWB G GAIN | 0x04 | RW Bit[7:4]: Not used Bit[3:0]: AWB G gain[11:8] |

##### table 7-4	AWB gain control registers (sheet 2 of 2)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

0x3403	AWB G GAIN	0x00	RW	Bit[7:0]: AWB G gain[7:0]

0x3404	AWB B GAIN	0x04	RW	Bit[7:4]: Not used

Bit[3:0]: AWB B gain[11:8]

0x3405	AWB B GAIN	0x00	RW	Bit[7:0]: AWB B gain[7:0]

Bit[7:1]: Reserved

0x3406	AWB MANUAL CONTROL	0x00	RW

Bit[0]:	AWB gain manual enable 0:	Auto

1:	Manual

  - AEC/AGC control [0x3500 ~ 0x3513]

##### table 7-5	AEC/AGC control functions (sheet 1 of 2)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

0x3500	AEC PK EXPOSURE	0x00	RW

Exposure Output Bit[7:4]: Reserved

Bit[3:0]: Exposure[19:16]

0x3501	AEC PK EXPOSURE	0x02	RW	Exposure Output

Bit[7:0]: Exposure[15:8]

0x3502	AEC PK EXPOSURE	0x00	RW	Exposure Output

Bit[7:0]: Exposure[7:0]

AEC Manual Mode Control Bit[7:6]: Reserved

Bit[5]:  Gain delay option

Valid when 0x3503[4]=1’b0 0: Delay one frame latch 1:  One frame latch

0x3503	AEC PK MANUAL	0x00	RW

0x3504	AEC MANUAL SENSOR	0x00	RW GAIN

Bit[4:2]: Reserved Bit[1]:	AGC manual

0:	Auto enable

1:	Manual enable Bit[0]:	AEC manual

0:	Auto enable

1:	Manual enable

Bit[7:2]: Reserved

Bit[1:0]: AGC manual sensor gain[9:8] Valid when 0x3509[3]=1’b1

0x3505	AEC MANUAL SENSOR

GAIN

0x00	RW	Bit[7:0]: AEC manual sensor gain[7:0]

Valid when 0x3509[3]=1’b1

0x3512

AEC PK SENSOR GAIN OUTPUT

–

R

Bit[7:2]: Reserved

Bit[1:0]: AEC pk sensor gain[9:8]

0x3513

AEC PK SENSOR GAIN OUTPUT

–

R

Bit[7:0]: AEC pk sensor gain[7:0]

7.6  ANA control registers [0x3600 ~ 0x3634]

table 7-6

ANA control registers

0x3600~

0x3634

RSVD

–

–

Reserved

##### table 7-5	AEC/AGC control functions (sheet 2 of 2)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

| 0x3509 | AEC GAIN CONTROL MISC | 0x10 | RW | Bit[7:6]: Bit[5]: Bit[4]: Bit[3]: Bit[2]: Bit[1:0]: | Reserved Digital gain manual enable Reserved Gain manual enable Reserved Manual digital gain Valid when 0x3509[5]=1’b1 |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Real Gain |  |
| 0x350A | AEC PK REAL GAIN | 0x00 | RW Bit[7:2]: Reserved Bit[1:0]: Real gain[9:8] |  |  |
| 0x350B | AEC PK REAL GAIN | 0x10 | RW Real Gain Bit[7:0]: Real gain[7:0] |  |  |
| 0x350C | AEC PK ADD VTS | 0x00 | RW AEC Add VTS output Bit[7:0]: Add VTS[15:8] |  |  |
| 0x350D | AEC PK ADD VTS | 0x00 | RW AEC Add VTS output Bit[7:0]: Add VTS[7:0] |  |  |

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

  - sensor control [0x3700 ~ 0x373C]

##### table 7-7	sensor control registers

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

0x3700~

0x373C

RSVD	–	–	Reserved

  - timing control [0x3800 ~ 0x3836]

##### table 7-8	timing control registers (sheet 1 of 2)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

| 0x3800 | TIMING HS | 0x00 | RW Bit[7:4]: Not used Bit[3:0]: X address start[11:8] |
| --- | --- | --- | --- |
| 0x3801 | TIMING HS | 0x00 | RW Bit[7:0]: X address start[7:0] |
| 0x3802 | TIMING VS | 0x00 | RW Bit[7:4]: Not used Bit[3:0]: Y address start[11:8] |
| 0x3803 | TIMING VS | 0x00 | RW Bit[7:0]: Y address start[7:0] |
| 0x3804 | TIMING HW | 0x08 | RW Bit[7:4]: Not used Bit[3:0]: X address end[11:8] |
| 0x3805 | TIMING HW | 0x1F | RW Bit[7:0]: X address end[7:0] |
| 0x3806 | TIMING VH | 0x06 | RW Bit[7:4]: Not used Bit[3:0]: Y address end[11:8] |
| 0x3807 | TIMING VH | 0x0B | RW Bit[7:0]: Y address end[7:0] |
| 0x3808 | TIMING DVPHO | 0x08 | RW Bit[7:4]: Debug mode Bit[3:0]: DVP output horizontal width[11:8] |
| 0x3809 | TIMING DVPHO | 0x00 | RW Bit[7:0]: DVP output horizontal width[7:0] |
| 0x380A | TIMING DVPVO | 0x06 | RW Bit[7:4]: Not used Bit[3:0]: DVP output vertical height[11:8] |
| 0x380B | TIMING DVPVO | 0x00 | RW Bit[7:0]: DVP output vertical height[7:0] |
| 0x380C | TIMING HTS | 0x08 | RW Bit[7:4]: Not used Bit[3:0]: Total horizontal size[11:8] |
| 0x380D | TIMING HTS | 0xFC | RW Bit[7:0]: Total horizontal size[7:0] |
| 0x380E | TIMING VTS | 0x06 | RW Bit[7:0]: Total vertical size[15:8] |

##### table 7-8	timing control registers (sheet 2 of 2)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

| 0x380F | TIMING VTS | 0x1C | RW Bit[7:0]: Total vertical size[7:0] |  |  |
| --- | --- | --- | --- | --- | --- |
| 0x3810 | TIMING HOFFSET | 0x00 | RW Bit[7:4]: Not used Bit[3:0]: ISP horizontal offset[11:8] |  |  |
| 0x3811 | TIMING HOFFSET | 0x10 | RW Bit[7:0]: Horizontal offset[7:0] |  |  |
| 0x3812 | TIMING VOFFSET | 0x00 | RW Bit[7:3]: Not used Bit[2:0]: Vertical offset[10:8] |  |  |
| 0x3813 | TIMING VOFFSET | 0x06 | RW Bit[7:0]: Vertical offset[7:0] |  |  |
| 0x3814 | TIMING X INC | 0x11 | RW Bit[7:4]: Horizontal odd subsample increment Bit[3:0]: Horizontal even subsample increment |  |  |
| 0x3815 | TIMING Y INC | 0x11 | RW Bit[7:4]: Vertical odd subsample increment Bit[3:0]: Vertical even subsample increment |  |  |
| 0x3816 | HSYNC START | 0x00 | RW Bit[7:4]: Not used Bit[3:0]: HSYNC start point[11:8] |  |  |
| 0x3817 | HSYNC START | 0x00 | RW Bit[7:0]: HSYNC start point[7:0] |  |  |
| 0x3818 | HSYNC WIDTH | 0x00 | RW Bit[7:4]: Not used Bit[3:0]: HSYNC width[11:8] |  |  |
| 0x3819 | HSYNC WIDTH | 0x00 | RW Bit[7:0]: HSYNC width[7:0] |  |  |
|  |  |  | Timing Control |  |  |
| 0x3820 | TIMING TC REG20 | 0x40 | RW | Bit[7:5]: Bit[4]: Bit[3]: Bit[2]: Bit[1]: Bit[0]: | Reserved Blackline vflip Reserved ISP vflip Sensor vflip Vertical binning enable |
| Timing Control Bit[7:6]: Reserved Bit[5]: Compression enable |  |  |  |  |  |
| 0x3821 | TIMING TC REG21 | 0x00 | RW Bit[4:3]: Reserved Bit[2]: ISP mirror Bit[1]: Sensor mirror Bit[0]: Horizontal binning enable |  |  |
| 0x3835 | TIMING TC REG35 | 0x00 | Bit[7:2]: Not used RW Bit[1]: FSIN reverse Bit[0]: FSIN enable |  |  |
| 0x3836 | TIMING TC REG36 | 0x00 | RW Bit[7:4]: Not used |  |  |

Bit[3:0]  FSIN output width

  - AEC/AGC power down domain control [0x3A00 ~ 0x3A25]

##### table 7-9	AEC/AGC power down domain control registers (sheet 1 of 3)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

0x3A00	AEC CTRL00	0x78	RW

AEC System Control (0: disable; 1: enable) Bit[7:6]: Reserved

Bit[5]:	Band function enable Bit[4]:	Less 1 band enable Bit[3]:	Start selection

Bit[2]:	Night mode

Bit[1]:	New balance function Bit[0]:	 Freeze

0x3A01	AEC MIN EXPOSURE	0x04	RW	Minimum Exposure Output Limit

Bit[7:0]: Minimum exposure

0x3A02	AEC MAX EXPO

(60HZ)

0x3A03	AEC MAX EXPO

(60HZ)

0x30	RW	60Hz Maximum Exposure Output Limit

Bit[7:0]: Maximum exposure[15:8]

0xC0	RW	60Hz Maximum Exposure Output Limit

Bit[7:0]: Maximum exposure[7:0]

0x3A04	RSVD	–	–	Reserved

AEC System Control 2 Bit[7]:	Reserved Bit[6]:	frame insert

0:	In night mode, insert frame disable

1:	In night mode, insert frame

0x3A05	AEC CTRL05	0x30	RW

0x3A06	AEC CTRL06	0x10	RW

0x3A07	AEC CTRL07	0x18	RW

enable Bit[5]:	Step auto enable

0:	Step manual mode 1:	Step auto mode

Bit[4:0]: Step auto ratio

In step auto mode, step ratio setting to adjust speed

AEC System Control 3 Bit[7:5]: Reserved

Bit[4:0]: Step manual setting 1 Step manual

Increase mode fast step

AEC Manual Step Register Bit[7:4]: Step manual setting 2

Step manual, slow step Bit[3:0]: Step manual setting 3

Step manual, decrease mode fast step

##### table 7-9	AEC/AGC power down domain control registers (sheet 2 of 3)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

0x3A08	AEC B50 STEP	0x00	RW

50Hz Band Width Bit[7:2]: Reserved Bit[1:0]: B50 step[9:8]

0x3A09	AEC B50 STEP	0xEA	RW	50Hz Band Width

Bit[7:0]: B50 step[7:0]

60Hz Band Width

0x3A0A	AEC B60 STEP	0x00	RW

Bit[7:2]: Reserved Bit[1:0]: B60 step[13:8]

0x3A0B	AEC B60 STEP	0xC3	RW	60Hz Band Width

Bit[7:0]: B60 step[7:0]

Bit[7:4]: E1 max

0x3A0C	AEC CTRL0C	0xE4	RW

0x3A0D	AEC CTRL0D	0x08	RW

0x3A0E	AEC CTRL0E	0x06	RW

Decimal line high limit zone Bit[3:0]: E1 min

Decimal line low limit zone

60Hz Max Bands in One Frame Bit[7:6]: Reserved

Bit[5:0]: B60 max

50Hz Max Bands in One Frame Bit[7:6]: Debug mode Bit[5:0]: B50 max

0x3A0F	AEC CTRL0F	0x78	RW	Stable Range High Limit (Enter)

Bit[7:0]: WPT

0x3A10	AEC CTRL10	0x68	RW	Stable Range Low Limit (Enter)

Bit[7:0]: BPT

0x3A11	AEC CTRL11	0xD0	RW	Step Manual Mode, Fast Zone High Limit

Bit[7:0]: VPT high

0x3A12	RSVD	–	–	Reserved

Bit[7]:	Pre-gain enable

0x3A13	AEC CTRL13	0x90	RW

0x3A14	AEC MAX EXPO	0x30	RW (50HZ)

Bit[6:0]: Pre-gain enable 0x10: 1x

50Hz Maximum Exposure Output Limit Bit[7:4]: Reserved

Bit[3:0]: Max exposure[15:8]

0x3A15	AEC MAX EXPO

(50HZ)

0x72	RW	50Hz Maximum Exposure Output Limit

Bit[7:0]: Max exposure[7:0]

0x3A16	RSVD	–	–	Reserved

##### table 7-9	AEC/AGC power down domain control registers (sheet 3 of 3)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

Gain Base When in Night Mode

|  |  |  |  | Bit[7:2]: Bit[1:0]: | Reserved Gain night threshold |
| --- | --- | --- | --- | --- | --- |
| 0x3A17 | AEC CTRL17 | 0x01 | RW |  | 00: 0x00 01: 0x10 10: 0x30 11: 0x70 |
| Gain Output Top Limit |  |  |  |  |  |
| 0x3A18 | AEC GAIN CEILING | 0x07 | RW Bit[7:2]: Reserved Bit[1:0]: AEC gain ceiling[9:8] Real gain format |  |  |
| 0x3A19 | AEC GAIN CEILING | 0xC0 | Gain Output Top Limit RW Bit[7:0]: AEC gain ceiling[7:0] Real gain format |  |  |
| 0x3A1A | AEC DIFF MIN | 0x00 | RW Reserved Default Value for This Register Bit[7:0]: Difference minimal |  |  |
| 0x3A1B | AEC CTRL1B | 0x78 | RW Stable Range High Limit (Go Out) Bit[7:0]: WPT2 |  |  |
| 0x3A1C | LED ADD ROW | 0x06 | RW Exposure Values Added When Strobe is On Bit[7:0]: AEC LED add row[15:8] |  |  |
| 0x3A1D | LED ADD ROW | 0x18 | RW Exposure Values Added When Strobe is On Bit[7:0]: AEC LED add row[7:0] |  |  |
| 0x3A1E | AEC CTRL1E | 0x68 | RW Stable Range Low Limit (Go Out) Bit[7:0]: BPT2 |  |  |
| 0x3A1F | AEC CTRL1F | 0x40 | RW Step Manual Mode, Fast Zone Low Limit Bit[7:0]: VPT low |  |  |
| 0x3A20 | AEC CTRL20 | 0x20 | Bit[7:3]: Reserved RW Bit[2]: Strobe option |  |  |
|  |  |  |  | Bit[1:0]: | Reserved |
| 0x3A21 | AEC CTRL21 | 0x78 | RW | Bit[7]: Bit[6:4]: Bit[3:0]: | Reserved Insert frame number Reserved |
| 0x3A25 | AEC CTRL25 | 0x00 | RW | Bit[7:5]: Bit[4:2]: Bit[1:0]: | Reserved Freeze count Reserved |

  - strobe control [0x3B00]

##### table 7-10	strobe registers

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

0x3B00	STROBE CTRL	0x00	RW

Bit[7]:	Strobe request on/off 0:	Off

1:	On

Bit[6]:	Strobe pulse reverse Bit[3:2]: width_in_xenon Bit[1:0]: Strobe mode

00: Xenon

01: LED1

10: LED2

11: LED3

  - sigmadelta/5060Hz detector [0x3C00 ~ 0x3C1E]

##### table 7-11	sigmadelta/5060HZ detector registers (sheet 1 of 3)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

0x3C00	SIGMADELTA CTRL00	0x00	RW

Bit[7:6]: Reserved

Bit[5]:	Time counter threshold divisor enable Bit[4]:	 Low limit enable

Bit[3]:	Reserve sigmaq Bit[2]:	Band50 default value

Bit[1:0]: Time counter threshold 00: 1s

01: 2s

10: 4s

11: 8s

Bit[7]:	Band manual enable Bit[6]:	Band begin reset enable

| 0x3C01 | SIGMADELTA CTRL01 | 0x00 | RW | Bit[5]: Bit[4]: Bit[3:0]: | Sum auto mode enable Band counter enable Band counter Counter threshold for band change |
| --- | --- | --- | --- | --- | --- |
| 0x3C02 | SIGMADELTA CTRL02 | 0x00 | RW | Bit[7:6]: Bit[5:0]: | Low light limit mode Low light threshold No detection under low light |
| 0x3C03 | SIGMADELTA CTRL03 | 0x00 | RW | Bit[7:0]: | Counter threshold for low light |
| 0x3C04 | SIGMADELTA CTRL04 | 0x20 | RW | Bit[7:0]: | Threshold for low sum |

##### table 7-11	sigmadelta/5060HZ detector registers (sheet 2 of 3)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

THRESHOLD

THRESHOLD

| 0x3C05 SIGMADELTA CTRL05 0x70 |  |  | RW | Bit[7:0]: | Threshold for high sum |
| --- | --- | --- | --- | --- | --- |
| 0x3C06 LIGHT METER1 0x00 |  |  | RW | Bit[7:0]: | Lightmeter1 threshold[15:8] |
| 0x3C07 LIGHT METER1 0x00 |  |  | RW | Bit[7:0]: | Lightmeter1 threshold[7:0] |
| 0x3C08 LIGHT METER2 0x01 |  |  | RW | Bit[7:0]: | Lightmeter2 threshold[15:8] |
| 0x3C09 LIGHT METER2 0x2C |  |  | RW | Bit[7:0]: | Lightmeter2 threshold[7:0] |
| 0x3C0A SAMPLE NUMBER 0x4E |  |  | RW | Bit[7:0]: | Sample number[15:8] |
| 0x3C0B SAMPLE NUMBER 0x1F |  |  | RW | Bit[7:0]: | Sample number[7:0] |
|  |  |  |  | Bit[7:1]: | Reserved |
|  |  |  |  | Bit[0]: | Band50/60 |
| 0x3C0C SIGMADELTA CTRL0C – 0x3C0D SUM 50 – |  |  | R R | Bit[7:5]: | 0: 60Hz light 1: 50Hz light Not used |
|  |  |  |  | Bit[4:0]: | Sum50[28:24] |
| 0x3C0E | SUM 50 | – R |  | Bit[7:0]: | Sum50[23:16] |
| 0x3C0F | SUM 50 | – R |  | Bit[7:0]: | Sum50[15:8] |
| 0x3C10 | SUM 50 | – R |  | Bit[7:0]: | Sum50[7:0] |
| 0x3C11 | SUM 60 | – | R | Bit[7:5]: | Not used |
|  |  |  |  | Bit[4:0]: | Sum60[28:24] |
| 0x3C12 | SUM 60 | – R |  | Bit[7:0]: | Sum60[23:16] |
| 0x3C13 | SUM 60 | – R |  | Bit[7:0]: | Sum60[15:8] |
| 0x3C14 | SUM 60 | – R |  | Bit[7:0]: | Sum60[7:0] |
| 0x3C15 | SUM 50 60 | – R |  | Bit[7:0]: | Sum50/60[15:8] |
| 0x3C16 | SUM 50 60 | – R |  | Bit[7:0]: | Sum50/60[7:0] |
| 0x3C17 | BLOCK COUNTER | – R |  | Bit[7:0]: | Block counter[15:8] |
| 0x3C18 | BLOCK COUNTER | – R |  | Bit[7:0]: | Block counter[7:0] |
| 0x3C19 | B6 | – R |  | Bit[7:0]: | B6[15:8] |
| 0x3C1A | B6 | – R |  | Bit[7:0]: | B6[7:0] |
| 0x3C1B | LIGHTMETER OUTPUT | – | R | Bit[7:4]: | Not used |
|  |  |  |  | Bit[3:0]: | Light meter output[19:16] |
| 0x3C1C | LIGHTMETER OUTPUT | – R |  | Bit[7:0]: | Light meter output[15:8] |

THRESHOLD THRESHOLD

##### table 7-11	sigmadelta/5060HZ detector registers (sheet 3 of 3)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

0x3C1D	LIGHTMETER OUTPUT	 –	 R		Bit[7:0]: Light meter output[7:0] 0x3C1E	SUM THRESHOLD	–	R	Delta Sum Threshold

table 7-12	OTP control functions (sheet 1 of 2)

OTP control [0x3D00 ~ 0x3D21]

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

| 0x3D00 | OTP DATA00 | 0x00 | RW | OTP Dump/Load Data00 |
| --- | --- | --- | --- | --- |
| 0x3D01 | OTP DATA01 | 0x00 | RW | OTP Dump/Load Data01 |
| 0x3D02 | OTP DATA02 | 0x00 | RW | OTP Dump/Load Data02 |
| 0x3D03 | OTP DATA03 | 0x00 | RW | OTP Dump/Load Data03 |
| 0x3D04 | OTP DATA04 | 0x00 | RW | OTP Dump/Load Data04 |
| 0x3D05 | OTP DATA05 | 0x00 | RW | OTP Dump/Load Data05 |
| 0x3D06 | OTP DATA06 | 0x00 | RW | OTP Dump/Load Data06 |
| 0x3D07 | OTP DATA07 | 0x00 | RW | OTP Dump/Load Data07 |
| 0x3D08 | OTP DATA08 | 0x00 | RW | OTP Dump/Load Data08 |
| 0x3D09 | OTP DATA09 | 0x00 | RW | OTP Dump/Load Data09 |
| 0x3D0A | OTP DATA0A | 0x00 | RW | OTP Dump/Load Data0a |
| 0x3D0B | OTP DATA0B | 0x00 | RW | OTP Dump/Load Data0b |
| 0x3D0C | OTP DATA0C | 0x00 | RW | OTP Dump/Load Data0c |
| 0x3D0D | OTP DATA0D | 0x00 | RW | OTP Dump/Load Data0d |
| 0x3D0E | OTP DATA0E | 0x00 | RW | OTP Dump/Load Data0e |
| 0x3D0F | OTP DATA0F | 0x00 | RW | OTP Dump/Load Data0f |
| 0x3D10 | OTP DATA10 | 0x00 | RW | OTP Dump/Load Data10 |
| 0x3D11 | OTP DATA11 | 0x00 | RW | OTP Dump/Load Data11 |
| 0x3D12 | OTP DATA12 | 0x00 | RW | OTP Dump/Load Data12 |
| 0x3D13 | OTP DATA13 | 0x00 | RW | OTP Dump/Load Data13 |
| 0x3D14 | OTP DATA14 | 0x00 | RW | OTP Dump/Load Data14 |

##### table 7-12	OTP control functions (sheet 2 of 2)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

| 0x3D15 | OTP DATA15 | 0x00 | RW | OTP Dump/Load Data15 |
| --- | --- | --- | --- | --- |
| 0x3D16 | OTP DATA16 | 0x00 | RW | OTP Dump/Load Data16 |
| 0x3D17 | OTP DATA17 | 0x00 | RW | OTP Dump/Load Data17 |
| 0x3D18 | OTP DATA18 | 0x00 | RW | OTP Dump/Load Data18 |
| 0x3D19 | OTP DATA19 | 0x00 | RW | OTP Dump/Load Data19 |
| 0x3D1A | OTP DATA1A | 0x00 | RW | OTP Dump/Load Data1A |
| 0x3D1B | OTP DATA1B | 0x00 | RW | OTP Dump/Load Data1B |
| 0x3D1C | OTP DATA1C | 0x00 | RW | OTP Dump/Load Data1C |
| 0x3D1D | OTP DATA1D | 0x00 | RW | OTP Dump/Load Data1D |
| 0x3D1E | OTP DATA1E | 0x00 | RW | OTP Dump/Load Data1E |
| 0x3D1F | OTP DATA1F | 0x00 | RW | OTP Dump/Load Data1F |
|  |  |  |  | Bit[7]: OTP program busy Bit[6:2]: Not used |
| 0x3D20 | OTP PROGRAM CTRL | 0x00 | RW Bit[1]: OTP program speed 0: Fast 1: Slow Bit[0]: OTP program enable |  |
| 0x3D21 | OTP READ CTRL | 0x00 | Bit[7]: OTP read busy Bit[6:2]: Not used RW Bit[1]: OTP read speed |  |
|  |  |  |  | 0: Fast 1: Slow |

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

0x4000	BLC CTRL00	0x0D	RW

BLC Control 00 (0: disable; 1: enable) Bit[7:4]: Reserved

Bit[0]:	OTP read enable

7.13 BLC control [0x4000 ~ 0x4033]

table 7-13	BLC registers (sheet 1 of 3)

Bit[3]:	BGR comp enable Bit[2]:	 Apply selection Bit[1]:	Mid filter enable Bit[0]:	 BLC enable

##### table 7-13	BLC registers (sheet 2 of 3)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

0x4001	BLC CTRL01	0x04	RW	Bit[7:6]: Reserved

Bit[5:0]: BLC start line

Bit[7]:	Format change enable

| BLC update when format changes Bit[6]: BLC auto enable |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x4002 | BLC CTRL02 | 0x45 | RW 0: Manual 1: Auto Bit[5:0]: Reset frame number Frame number BLC do after reset |  |  |  |
| 0x4003 | BLC CTRL03 | 0x01 | Bit[7]: BLC redo enable Write 1 to this bit will trigger a BLC RW redo N frames begin, where N is 0x4003[5:0] Bit[6]: BLC freeze Bit[5:0]: Manual frame number |  |  |  |
| 0x4004 | BLC CTRL04 | 0x08 | RW Bit[7:0]: BLC line number Specify the line number BLC process |  |  |  |
|  |  |  | Bit[7:2]: Reserved Bit[1]: BLC always update |  |  |  |
| 0x4005 | BLC CTRL05 | 0x10 | RW | 0: |  | Normal freeze |
|  |  |  |  | 1: |  | BLC always update |
|  |  |  |  | Bit[0]: | Reserved |  |
|  |  |  |  | Bit[7]: | Reserved |  |
| 0x4006 | BLC CTRL06 | 0x08 | RW | Bit[6]: | Black line number manual enable |  |
|  |  |  |  | Bit[5:0]: | Black number manual |  |
|  |  |  |  | Bit[7:5]: | Reserved |  |
|  |  |  |  | Bit[4:3]: | Window selection |  |
|  |  |  |  | 00: |  | Full image |
|  |  |  |  | 01: |  | A windows not contain the first |
|  |  |  |  |  |  | 16 pixels and the end 16 pixels |
| 0x4007 | BLC CTRL07 | 0x00 | RW | 10: |  | A windows not contain the first 1/16 image and the end 1/16 |
|  |  |  |  |  |  | image |
|  |  |  |  | 11: |  | A windows not contain the first |
|  |  |  |  |  |  | 1/8 image and the end 1/8 |
|  |  |  |  |  |  | image |
|  |  |  |  | Bit[2:0]: | Reserved |  |
| 0x4009 | BLACK LEVEL | 0x10 | RW | Bit[7:0]: | BLC black level target at 10-bit range |  |

0x400A~

0x402B

RSVD	–	–	Reserved

0x402C	BLACK LEVEL00	0x00	RW	Bit[7:0]: Blacklevel00[15:8]

With 3 decimal

##### table 7-13	BLC registers (sheet 3 of 3)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

| 0x402D | BLACK LEVEL00 | 0x00 | RW Bit[7:0]: Blacklevel00[7:0] With 3 decimal |
| --- | --- | --- | --- |
| 0x402E | BLACK LEVEL01 | 0x00 | RW Bit[7:0]: Blacklevel01[15:8] With 3 decimal |
| 0x402F | BLACK LEVEL01 | 0x00 | RW Bit[7:0]: Blacklevel01[7:0] With 3 decimal |
| 0x4030 | BLACK LEVEL10 | 0x00 | RW Bit[7:0]: Blacklevel10[15:8] With 3 decimal |
| 0x4031 | BLACK LEVEL10 | 0x00 | RW Bit[7:0]: Blacklevel10[7:0] With 3 decimal |
| 0x4032 | BLACK LEVEL11 | 0x00 | RW Bit[7:0]: Blacklevel11[15:8] With 3 decimal |
| 0x4033 | BLACK LEVEL11 | 0x00 | RW Bit[7:0]: Blacklevel11[7:0] With 3 decimal |

  - frame control [0x4201 ~ 0x4202]

##### table 7-14	frame control registers

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

0x4201	FRAME CTRL01	0x00	R/W

0x4202	FRAME CTRL02	0x00	R/W

Control Passed Frame Number

When both ON and OFF number set to 0x00, frame control is in bypass mode

Bit[7:4]: Not used

Bit[3:0]: Frame ON number

Control Masked Frame Number

When both ON and OFF number set to 0x00, frame control is in bypass mode

Bit[7:4]: Not used

BIT[3:0]: Frame OFF number

  - format control [0x4300 ~ 0x430D]

##### table 7-15	format control registers (sheet 1 of 4)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

Format Control 00

Bit[7:4]: Output format selection 0x0: RAW

Bit[3:0]: Output sequence

0x0: BGBG... / GRGR...

0x1: GBGB... / RGRG...

0x2: GRGR... / BGBG...

0x3: RGRG... / GBGB...

0x4~0xF: Not allowed

0x4300	FORMAT CTRL 00	0xF8	RW

0x1: Y8

Bit[3:0]: Does not matter 0x2: Not used

0x3: YUV422

Bit[3:0]: Output sequence

0x0: YUYV...

0x1: YVYU...

0x2: UYVY...

0x3: VYUY...

0x4~0xE: Not allowed 0xF: UYVY...

0x4~0x5: Not used 0x6: RGB565

Bit[3:0]: Output sequence

0x0: {b[4:0],g[5:3]},

{g[2:0],r[4:0]}

0x1: {r[4:0],g[5:3]},

{g[2:0],b[4:0]}

0x2: {g[4:0],r[5:3]},

{r[2:0],b[4:0]}

0x3: {b[4:0],r[5:3]},

{r[2:0],g[4:0]}

0x4: {g[4:0],b[5:3]},

{b[2:0],r[4:0]}

0x5: {r[4:0],b[5:3]},

{b[2:0],g[4:0]}

0x6~0xE: Not allowed 0xF: {g[2:0],b[4:0]},

{r[4:0],g[5:3]}

0x7: RGB555 format 1

Bit[3:0]: Output sequence

0x0: {b[4:0],g[4:2]},

{g[1:0],1'b0,r[4:0]}

0x1: {r[4:0],g[4:2]},

{g[1:0],1'b0,b[4:0]}

##### table 7-15	format control registers (sheet 2 of 4)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

0x2: {g[4:0],r[4:2]},

{r[1:0],1'b0,b[4:0]}

0x3: {b[4:0],r[4:2]},

{r[1:0],1'b0,g[4:0]}

0x4: {r[4:0],b[4:2]},

{b[1:0],1'b0,g[4:0]}

0x5: {g[4:0],b[4:2]},

{b[1:0],1'b0,r[4:0]}

0x6~0xF: Not allowed 0x8: RGB555 format 2

Bit[3:0]: Output sequence

0x0: {1'b0,b[4:0],g[4:3]},

{g[2:0],r[4:0]}

0x1: {1'b0,r[4:0],g[4:2]},

{g[2:0],b[4:0]}

0x2: {1'b0,g[4:0],r[4:2]},

{r[2:0],b[4:0]}

0x3: {1'b0,b[4:0],r[4:2]},

{r[2:0],g[4:0]}

0x4: {1'b0,r[4:0],b[4:2]},

{b[2:0],g[4:0]}

0x5: {1'b0,g[4:0],b[4:2]},

{b[2:0],r[4:0]}

0x6: {b[4:0],1'b0,g[4:3]},

{g[2:0],r[4:0]}

0x8: {g[4:0],1'b0,r[4:2]},

{r[2:0],b[4:0]}

0x9: {b[4:0],1'b0,r[4:2]},

{r[2:0],g[4:0]}

0xA: {r[4:0],1'b0,b[4:2]},

{b[2:0],g[4:0]}

0xB: {g[4:0],1'b0,b[4:2]},

{b[2:0],r[4:0]}

0xC~0xF: Not allowed 0x9: RGB444 format 1

Bit[3:0]: Output sequence

0x0: {1'b0,b[3:0],2'h0,g[3]},

{g[2:0],1'b0,r[3:0]}

0x1: {1'b0,r[3:0],2'h0,g[3]},

{g[2:0],1'b0,b[3:0]}

0x2: {1'b0,g[3:0],2'h0,r[3]},

{r[2:0],1'b0,b[3:0]}

0x3: {1'b0,b[3:0],2'h0,r[3]},

{r[2:0],1'b0,g[3:0]}

0x4: {1'b0,r[3:0],2'h0,b[3]},

{b[2:0],1'b0,g[3:0]}

0x5: {1'b0,g[3:0],2'h0,b[3]},

{b[2:0],1'h0,r[3:0]}

0x6: {b[3:0],1'b0,g[3:1]},

{g[0],2'h0,r[3:0],1'b0}

##### table 7-15	format control registers (sheet 3 of 4)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

0x7: {r[3:0],1'b0,g[3:1]},

{g[0],2'h0,b[3:0],1'b0}

0x8: {g[3:0],1'b0,r[3:1]},

{r[0],2'h0,b[3:0],1'b0}

0x9: {b[3:0],1'b0,r[3:1]},

{r[0],2'h0,g[3:0],1'b0}

0xA: {r[3:0],1'b0,b[3:1]},

{b[0],2'h0,g[3:0],1'b0}

0xB: {g[3:0],1'b0,b[3:1]},

{b[0],2'h0,r[3:0],1'b0}

0xC~0xF: Not allowed 0xA: RGB444 format 2

Bit[3:0]: Output sequence

0x0: {4'b0,b[3:0]},

{g[3:0],r[3:0]}

0x1: {4'b0,r[3:0]},

{g[3:0],b[3:0]}

0x2: {4'b0,b[3:0]},

{r[3:0],g[3:0]}

0x3: {4'b0,r[3:0]},

{b[3:0],g[3:0]}

0x4: {4'b0,g[3:0]},

{b[3:0],r[3:0]}

0x5: {4'b0,g[3:0]},

{r[3:0],b[3:0]}

0x6: {b[3:0],g[3:0],2'h0},

{r[3:0],b[3:0],2'h0,g[3: 0],r[3:0],2'h0}

0x7: {r[3:0],g[3:0],2'h0},

{b[3:0],r[3:0],2'h0,g[3: 0],b[3:0],2'h0}

0x8: {b[3:0],r[3:0],2'h0},

{g[3:0],b[3:0],2'h0,r[3: 0],g[3:0],2'h0}

0x9: {r[3:0],b[3:0],2'h0},

{g[3:0],r[3:0],2'h0,b[3: 0],g[3:0],2'h0}

0xA: {g[3:0],b[3:0],2'h0},

{r[3:0],g[3:0],2'h0,b[3: 0],r[3:0],2'h0}

0xB: {g[3:0],r[3:0],2'h0},

{b[3:0],g[3:0],2'h0,r[3: 0],b[3:0],2'h0}

0xC~0xF: Not allowed 0xB~0xE: Not allowed

0xF: Bypass formatter module Bit[3:0]: Output format

0x8: Raw 0x9: YUV422

##### table 7-15	format control registers (sheet 4 of 4)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

|  |  |  |  | Bit[1:0]: | YUV422 UV control |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 00: U/V generated from average |
| 0x4301 | FORMAT CTRL 01 | 0x00 | RW |  | 01: U/V generated from first pixel |
|  |  |  |  |  | 10: Not valid |
|  |  |  |  |  | 11: U/V generated from second pixel |
| 0x4302 | YMAX VALUE | 0x03 | RW Bit[7:2]: Not used Bit[1:0]: Set y max clip value[9:8] |  |  |
| 0x4303 | YMAX VALUE | 0xFF | RW Bit[7:0]: Set y max clip value[7:0] |  |  |
| 0x4304 | YMIN VALUE | 0x00 | RW Bit[7:2]: Not used Bit[1:0]: Set y min clip value[9:8] |  |  |
| 0x4305 | YMIN VALUE | 0x00 | RW Bit[7:0]: Set y min clip value[7:0] |  |  |
| 0x4306 | UMAX VALUE | 0x03 | RW Bit[7:2]: Not used Bit[1:0]: Set u max clip value[9:8] |  |  |
| 0x4307 | UMAX VALUE | 0xFF | RW Bit[7:0]: Set u max clip value[7:0] |  |  |
| 0x4308 | UMIN VALUE | 0x00 | RW Bit[7:2]: Not used Bit[1:0]: Set u min clip value[9:8] |  |  |
| 0x4309 | UMIN VALUE | 0x00 | RW Bit[7:0]: Set u min clip value[7:0] |  |  |
| 0x430A | VMAX VALUE | 0x03 | RW Bit[7:2]: Not used Bit[1:0]: Set v max clip value[9:8] |  |  |
| 0x430B | VMAX VALUE | 0xFF | RW Bit[7:0]: Set v max clip value[7:0] |  |  |
| 0x430C | VMIN VALUE | 0x00 | RW Bit[7:2]: Not used Bit[1:0]: Set v min clip value[9:8] |  |  |
| 0x430D | VMIN VALUE | 0x00 | RW Bit[7:0]: Set v min clip value[7:0] |  |  |

  - compression control [0x4400 ~ 0x4458]

##### table 7-16	compression control registers (sheet 1 of 3)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

0x4400	COMPRESSION

CTRL00

0x4401	COMPRESSION

CTRL01

COMPRESSION

0x81	RW

0x01	RW

Bit[7]:	input_format

0:	YUV420

1:	YUV422

Bit[6:0]: JFIFO read speed control

Bit[7:4]: SFIFO output buffer speed control Bit[3]:	Read SRAM enable when blanking

0:	Disable

1:	Enable

Bit[2]:	Read SRAM at first blanking 0:	Disable

1:	Enable

Bit[1:0]: SFIFO read speed control

Bit[7]:	SFIFO output control mode

0:	Control by HREF and valid which before scale down

1:	Control by input HREF and valid

0x4402

CTRL02	0x10	RW

Bit[6:4]: SOF control

001: Start at the first valid HREF 010: Start at the eighth valid HREF

Bit[3:0]: SFIFO output buffer speed control at last stripe

Bit[7]:	Memory select

0:	Select ROM QT 1:	Select SRAM QT

Bit[6]:	MPEG enable

0x4403	COMPRESSION

CTRL03

COMPRESSION

0x33	RW

Bit[5]:	Enable zero stuff

Bit[4]:	 Enable Huffman table output Bit[3]:	Rounding enable for C Bit[2]:	Rounding enable for Y Bit[1]:	Input shift 128 select for C Bit[0]:	Input shift 128 select for Y

Bit[7]:	jfifo_pwrdn

Bit[6]:	SFIFO power down Bit[5]:	Header output enable Bit[4]:	Enable gated clock

0x4404

CTRL04	0x24	RW

0:	Disable gated clock 1:	Enable gated clock

|  |  | Bit[3]: Bit[2:0]: | Substitute 0xFF to 0xFE in QT Quantization rounding bias: Set value = Bias/8 |
| --- | --- | --- | --- |
| 0x4405 COMPRESSION 0x40 | RW | Bit[7:0]: | QZ out truncate for Y |

CTRL05

##### table 7-16	compression control registers (sheet 2 of 3)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

| 0x4406 COMPRESSION 0x40 | RW | Bit[7:0]: | QZ out truncate for C |
| --- | --- | --- | --- |
| 0x4407 COMPRESSION 0x0C CTRL07 | RW | Bit[7]: Bit[5:0]: | Enable read QTA auto increment QS Quantization scale |
|  |  | Bit[7]: | Scalado mode enable |

CTRL06

0:	Normal

1:	Insert 0xFF after EOB

0x4408

COMPRESSION ISI CTRL

0x00

RW

Bit[6]:

Bit[5:4]:

Bit[3]:

Bit[2]:

Bit[1]:

Bit[0]:

Compression size manual enable Reserved

Replace 0xFF to 0xFE in comment data

Cut 0xD9 at the end of frame EOI generation enable

ISI insert

0x4409

COMPRESSION CTRL09

0x00

RW

Bit[7:0]: D9 data

0x440A

COMPRESSION CTRL0A

0x4E

RW

Bit[7:0]: JFIFO output delay

0x440B

COMPRESSION CTRL0B

0x16

RW

Bit[7:6]: Not used

Bit[5]:	Dummy read speed manual mode Bit[4:0]: SFIFO SOF delay

0x440C

COMPRESSION CTRL0C

0x00

RW

Bit[7:0]: Dummy read speed

0x440D

COMPRESSION CTRL0D

0x0A

RW

Bit[7:5]: Not used Bit[4]:	JFIFO test1 Bit[3:0]: JFIFO rm

0x440E

COMPRESSION CTRL0E

0x09

RW

Bit[7:5]: Not used Bit[4]:	SFIFO test1 Bit[3:0]: SFIFO rm

0x4410

COMPRESSION QT DATA

0x00

RW

Bit[7:0]: QT data

0x4411

COMPRESSION QT ADDR

0x00

RW

Bit[7:0]: QT address

0x4412	COMPRESSION ISI DATA

0x00	RW	Bit[7:0]: ISI data

Bit[7:4]: Not used

Bit[3]:	D9 odd (read only)

–	R

0x4413	COMPRESSION ISI CTRL

Bit[2]:	Reset counter write

0:	Not valid

1:	Reset counter

Bit[1]:	ISO EOF

##### table 7-16	compression control registers (sheet 3 of 3)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

0x4414	COMPRESSION

LENGTH

0x4415	COMPRESSION

LENGTH

0x4416	COMPRESSION

0x4417

JFIFO OVERFLOW

–

R

Bit[7:1]: Not used

Bit[0]:	JFIFO overflow indicator

0x4420~

0x442F

COMPRESSION COMMENT

0x00

RW

Compression Comment Data Embedded in Compression Data

COMMENT

H SIZE MANUAL H SIZE MANUAL V SIZE MANUAL V SIZE MANUAL

LENGTH

- R	Bit[7:0]: Compression length[23:16]

- R	Bit[7:0]: Compression length[15:8]

- R	Bit[7:0]: Compression length[7:0]

| 0x4430 COMPRESSION 0x00 COMMENT | RW | Comment Length Two bytes align |
| --- | --- | --- |
| 0x4431 COMPRESSION 0xFE | RW | Comment Data Marker |
| 0x4440 COMPRESSION 0x02 | RW | Compression Header Horizontal Size Manual |
| 0x4441 COMPRESSION 0x80 | RW | Compression Header Horizontal Size Manual |
| 0x4442 COMPRESSION 0x01 | RW | Compression Header Vertical Size Manual |
| 0x4443 COMPRESSION 0xE0 | RW | Compression Header Vertical Size Manual |
| 0x4457 DRI HIGH 0x00 | RW | Compression Dri High |
| 0x4458 DRI LOW 0x02 | RW | Compression Dri Low |

  - VFIFO control [0x4600 ~ 0x460D]

##### table 7-17	VFIFO registers

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

0x4600	VFIFO CTRL00	0x80	RW

Bit[7:6]: Not used

Bit[5]: Compression output fixed height enable

0: In compression mode2, Compression height is different in each frame

1: in compression mode2, Compression height is fixed in each frame

Bit[4]:  Test pattern enable,

incremental pattern Bit[3:0]: Debug mode

0x4602	VFIFO HSIZE	0x04	RW	Compression Output Width High Byte

0x4603	VFIFO HSIZE	0x00	RW	Compression Output Width Low Byte

0x4604	VFIFO VSIZE	0x03	RW	Compression Output Height High Byte

0x4605	VFIFO HSIZE	0x00	RW	Compression Output Height Low Byte Bit[7:4]: Compression dummy data

pad speed Bit[2]:	Footer disable

Compression footer disable 0:	In compression mode2

footer will be added in the last six bytes of

0x460C	VFIFO CTRL0C	0x20	RW

each frame

1:	Disable footer Bit[1]:	PCLK manual enable

0:	DVP PCLK divider control by auto mode

1:	DVP PCLK divider control by 0x3824[4:0]

Bit[0]:	Not used

0x460D	VFIFO CTRL0D	0x00	RW	Dummy Data

  - DVP control [0x4709 ~ 0x4745]

##### table 7-18	DVP control registers (sheet 1 of 3)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

| 0x4709 | DVP VSYNC WIDTH0 | 0x02 | RW | VSYNC Width Line Unit |
| --- | --- | --- | --- | --- |
| 0x470A | DVP VSYNC WIDTH1 | 0x00 | RW | VSYNC Width PCLK Unit High Byte |
| 0x470B | DVP VSYNC WIDTH2 | 0x01 | RW | VSYNC Width PCLK Unit Low Byte |
| 0x4711 | PAD LEFT CTRL | 0x00 | RW HSYNC Mode Left Padding Pixel Count Add padding data at start of a line |  |
| 0x4712 | PAD RIGHT CTRL | 0x00 | RW HSYNC Mode Right Padding Pixel Count Add padding data at end of a line |  |

0x4713	COMPRESSION MODE SELECT

0x02	RW

Bit[7:3]: Not used

Bit[2:0]: Compression mode select 001: Compression mode 1

010: Compression mode 2

011: Compression mode 3

100: Compression mode 4

101: Compression mode 5

110: Compression mode 6

Bit[7:4]: Not used

0x4715	656 DUMMY LINE	0x00	RW

Bit[3:0]: CCIR656 dummy line number Control dummy line number at beginning of the frame

| 0x4719 | CCIR656 CTRL | 0x01 | RW Bit[7:2]: Not used Bit[1:0]: CCIR656 EAV/SAV option |  |  |
| --- | --- | --- | --- | --- | --- |
| 0x471B | HSYNC CTRL00 | 0x02 | RW Bit[7:1]: Not used Bit[0]: HSYNC mode enable |  |  |
|  |  |  | Bit[7:3]: Not used Bit[2]: vsync_sel1_clr Bit[1:0]: vsync_mode |  |  |
|  |  |  |  | 00: | VSYNC positive edge |
|  |  |  |  |  | trigger by end of field, |
|  |  |  |  |  | negative edge trigger by |
|  |  |  |  |  | start of frame |
| 0x471D | DVP VSYNC CTRL | 0x01 | RW | 01: | VSYNC positive edge |
|  |  |  |  |  | trigger by end of frame, |
|  |  |  |  |  | the width define by |
|  |  |  |  |  | register |
|  |  |  |  | 10: | VSYNC positive edge |
|  |  |  |  |  | trigger by start of field, |
|  |  |  |  |  | the width defined by |
|  |  |  |  |  | register |

##### table 7-18	DVP control registers (sheet 2 of 3)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

0x471F	DVP HREF CTRL	0x40	RW	HREF Minimum Blanking in Compression

Mode23

0x4721

VERTICAL START	0x01	RW OFFSET

Bit[7:4]: Debug mode

Bit[3:0]: Vertical start delay between video output and video input

Bit[7:4]: Not used

0x4722	VERTICAL END OFFSET	0x00	RW

Bit[3:0]: Vertical end delay between video output and video input

0x4723	DVP CTRL23	0x00	RW	DVP Compression Mode456 Skip Line

Number

Bit[7]:	SYNC code selection

0:	Auto generate sync code

1:	Sync code from register setting 0x4732~4735

|  |  |  |  | Bit[6]: Bit[5]: | F value in CCIR656 SYNC code when fixed f value Fixed f value |
| --- | --- | --- | --- | --- | --- |
| 0x4730 | CCIR656 CTRL00 | 0x00 | RW | Bit[4:3]: Bit[2]: | Blank toggle data options 00: Toggle data is 1'h040/1'h200 01: Use register setting 0x4736~0x4738 10: Blanking data always keep 0 Debug mode |
|  |  |  |  | Bit[1]: Bit[0]: | Clip data disable CCIR656 mode enable |
| 0x4731 | CCIR656 CTRL01 | 0x01 | RW | Bit[7:1]: Bit[0]: | Not used Blanking toggle data order option |
| 0x4732 | CCIR656 FS | 0x01 | RW | CCIR656 SYNC Code Frame Start |  |
| 0x4733 | CCIR656 FE | 0x0F | RW | CCIR656 SYNC Code Frame End |  |
| 0x4734 | CCIR656 LS | 0x00 | RW | CCIR6656 SYNC Code Line Start |  |
| 0x4735 | CCIR656 LE | 0x00 | RW | CCIR656 SYNC Code Line End |  |
| 0x4736 | CCIR656 CTRL6 | 0x00 | RW | Bit[7:4]: Not used Bit[3:2]: Toggle data0[9:8] Bit[1:0]: Toggle data1[9:8] |  |
| 0x4737 | CCIR656 CTRL7 | 0x00 | RW | Bit[7:0]: Toggle data0[7:0] |  |
| 0x4738 | CCIR656 CTRL8 | 0x00 | RW | Bit[7:0]: Toggle data1[7:0] |  |

##### table 7-18	DVP control registers (sheet 3 of 3)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

0x4740	POLARITY CTRL00	0x20	RW

0x4741	TEST PATTERN	0x00	RW

0x4745	DATA ORDER	0x00	RW

Bit[7:6]: Not used Bit[5]:	PCLK polarity

0:	Active low

1:	Active high

Bit[4]:	Reserved

Bit[3]:	Gate PCLK under VSYNC Bit[2]:	Gate PCLK under HREF Bit[1]:	HREF polarity

0:	Active low

1:	Active high Bit[0]:	VSYNC polarity

0:	Active low

1:	Active high

Bit[7:3]: Not used

Bit[2]:	Test pattern enable Bit[1]:	Test pattern select

0:	Output test pattern 0

1:	Output test pattern 1 Bit[0]:	Test pattern 8-bit/10-bit

0:	10-bit test pattern 1:	8-bit test pattern

Bit[7:3]: Not used

Bit[2:1]: DVP order option for debug 00: D[9:0]

10: {D[7:0],D[9:8]}

x1:  {D[1:0],D[9:2]}

Bit[0]:	Output data order

0:	Normal output

1:	Reverse output data bit order

  - ISP frame control [0x4901 ~ 0x4902]

##### table 7-19	ISP frame control registers

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

0x4901	FRAME CTRL01	0x00	RW

0x4902	FRAME CTRL02	0x00	RW

Control Passed Frame Number

When both ON and OFF number set to 0x00, frame control is in bypass mode

Bit[7:4]: Not used

Bit[3:0]: Frame ON number

Control Masked Frame Number

When both ON and OFF number set to 0x00, frame control is in bypass mode

Bit[7:4]: Not used

Bit[3:0]: Frame OFF number

  - ISP top control [0x5000 ~ 0x5063]

##### table 7-20	ISP top control registers (sheet 1 of 4)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

0x5000	ISP CONTROL 00	0x06	RW

Bit[7]:	LENC correction enable 0:	Disable

1:	Enable

Bit[6]:	Reserved Bit[5]:	Gamma enable

0:	Disable

1:	Enable Bit[4:3]: Reserved

Bit[2]:	Black pixel cancellation enable 0:	Disable

1:	Enable

Bit[1]:	White pixel cancellation enable 0:	Disable

1:	Enable

Bit[0]:	Color interpolation enable 0:	Disable

1:	Enable

##### table 7-20	ISP top control registers (sheet 2 of 4)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

0x5001	ISP CONTROL 01	0x01	RW

0x5003	ISP CONTROL 03	0x08	RW

0x5004	ISP CONTROL 04	0x08	RW

Bit[7]:	Special digital effect enable 0:	Disable

1:	Enable

Bit[6]:	Reserved Bit[5]:	 Scale enable

0:	Disable

1:	Enable Bit[4:3]: Reserved

Bit[2]:	UV average enable

0:	Disable

1:	Enable

Bit[1]:	Color matrix enable

0:	Disable

1:	Enable

Bit[0]:	Auto white balance enable 0:	Disable

1:	Enable

Bit[7:3]: Reserved Bit[2]:	Bin enable

0:	Disable

1:	Enable

Bit[1]:	Reserved Bit[0]:	Solarize enable

0:	Disable

1:	Enable

Bit[7:4]: Reserved

Bit[3]:	Size auto control enable 0:	Manual

1:	Automatic Bit[2:0]: Reserved

##### table 7-20	ISP top control registers (sheet 3 of 4)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

0x5005	ISP CONTROL 05	0x36	RW

Bit[7]:	Reserved

Bit[6]:	AWB bias manual enable 0:	Disable

1:	Enable

Bit[5]:	AWB bias ON enable 0:	Disable

1:	Enable

Bit[4]:	AWB bias plus enable 0:	Disable

1:	Enable

Bit[3]:	Reserved

Bit[2]:	LENC bias ON enable 0:	Disable

1:	Enable

Bit[1]:	GMA bias ON enable 0:	Disable

1:	Enable

Bit[0]:	LENC bias manual enable 0:	Disable

1:	Enable

0x5006~

0x501C

RSVD	–	–	Reserved

Bit[7]:	Reserved

Bit[6]:	SDE AVG manual enable

0x501D	ISP MISC	0x00	RW

Bit[5]:	 AWB YUV2CBCR enable Bit[4]:	Average size manual enable Bit[3:0]: Reserved

0x501E	RSVD	–	–	Reserved Format Control

Bit[7:6]: Not used

Bit[5]:	Enable option Bit[4]:	Reserved Bit[3]:	 Format vfirst

0x501F	FORMAT CTRL	0x03	RW

Bit[2:0]: Format select

000: YUV422

001: RGB

010: Dither

011: RAW after DPC 101: RAW after CIP

|  |  |  |  | Bit[7]: | Not used |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Bit[6]: | Dither MUX |
| 0x5020 | DITHER CTRL 0 | 0x00 | RW | Bit[5:4]: | R dithering |
|  |  |  |  | Bit[3:2]: | G dithering |
|  |  |  |  | Bit[1:0]: | B dithering |

##### table 7-20	ISP top control registers (sheet 4 of 4)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

0x503D	PRE ISP TEST

SETTING 1

0x00	RW

Bit[7]:	Test enable

0:	Test disable

1:	Color bar enable

Bit[6]:	Rolling Bit[5]:	 Transparent

Bit[4]:	Square black and white Bit[3:2]: Color bar style

00: Standard 8 color bar

01: Gradual change at vertical mode 1 10: Gradual change at horizontal

11: Gradual change at vertical mode 2 Bit[1:0]: Test select

00: Color bar

01: Random data

10: Square data

11: Black image

| 0x503E~ 0x5060 | RSVD | – – | Reserved |
| --- | --- | --- | --- |
| 0x5061 | ISP SENSOR BIAS I | – R | ISP Sensor Bias Input |
| 0x5062 | ISP SENSOR GAIN I | – R | ISP Real Gain Input High Byte |
| 0x5063 | ISP SENSOR GAIN I | – R | ISP Real Gain Input Low Byte |

  - AWB control [0x5180 ~ 0x51D0]

##### table 7-21	AWB registers (sheet 1 of 3)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

| 0x5180 | AWB CONTROL 00 | 0xFF | RW Bit[7:0]: AWB B block |
| --- | --- | --- | --- |
| 0x5181 | AWB CONTROL 01 | 0x58 | Bit[7:6]: Step local Bit[5:4]: Step fast RW Bit[3]: Slop 8x Bit[2]: Slop 4x Bit[1]: One zone Bit[0]: AVG all |
| 0x5182 | AWB CONTROL 02 | 0x11 | RW Bit[7:4]: Max local counter Bit[3:0]: Max fast counter |

##### table 7-21	AWB registers (sheet 2 of 3)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

0x5183	AWB CONTROL 03	0x90	RW

0x5184	AWB CONTROL 04	0x25	RW

0x5185	AWB CONTROL 05	0x24	RW

Bit[7]:	AWB simple enable

0:	AWB advance

1:	AWB simple Bit[6]:	YUV enable

0:	YUV enable

1:	Simple YUV enable Bit[5]:	AWB preset

Bit[4]:	AWB SIMF

Bit[3:2]: AWB win Bit[1:0]: Reserved

Bit[7:6]: Count area selection Bit[5]:	G enable

Bit[4:2]: Count limit control Bit[1:0]: Counter threshold

Bit[7:4]: Stable range unstable

Threshold for unstable to stable change Bit[3:0]: Stable range stable

Threshold for stable to unstable change

0x5186~

0x5190

AWB CONTROL	–	RW	Advanced AWB Control

0x5191	AWB CONTROL 17	0xFF	RW	Bit[7:0]: AWB top limit

0x5192	AWB CONTROL 18	0x00	RW	Bit[7:0]: AWB bottom limit 0x5193	AWB CONTROL 19	0xF0	RW	Bit[7:0]: Red limit

0x5194	AWB CONTROL 20	 0xF0	 RW	 Bit[7:0]: Green limit 0x5195	AWB CONTROL 21	0xF0	RW	Bit[7:0]: Blue limit

Bit[7:6]: Reserved Bit[5]:	AWB freeze Bit[4]:	Reserved

Bit[3:2]: AWB simple selection

0x5196	AWB CONTROL 22	0x03	RW

00: AWB simple from after AWB gain 01: AWB simple from after GMA

10: AWB simple from after GMA

11: AWB simple from after AWB gain Bit[1]:	Fast enable

Bit[0]:	AWB bias stat

0x5197	AWB CONTROL 23	0x02	RW	Bit[7:0]: Local limit

0x5198~

0x519D

RSVD	–	–	Reserved

Bit[7:4]: Reserved

0x519E	AWB CONTROL 30	0x30	RW

Bit[3]:	Local limit select Bit[2]:	Simple stable select Bit[1:0]: Reserved

##### table 7-21	AWB registers (sheet 3 of 3)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

0x519F~

0x51D0

AWB G SUM	–	R	AWB Debug Information

  - CIP control [0x5300 ~ 0x530F]

##### table 7-22	CIP control registers (sheet 1 of 2)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

0x5300	CIP SHARPENMT THRESHOLD 1

0x08	RW	Color Interpolation Sharpen MT

Threshold 1

| 0x5301 | CIP SHARPENMT THRESHOLD 2 | 0x48 | RW | Color Interpolation Sharpen MT Threshold 2 |
| --- | --- | --- | --- | --- |
| 0x5302 | CIP SHARPENMT OFFSET1 | 0x18 | RW | CIP Sharpen MT Offset1 (Y edge mt manual setting when |
|  |  |  |  | 0x5308[6]=1) |
| 0x5303 | CIP SHARPENMT OFFSET2 | 0x0E | RW | CIP Sharpen MT Offset2 |
| 0x5304 | CIP DNS THRESHOLD 1 | 0x08 | RW | CIP DNS Threshold 1 |
| 0x5305 | CIP DNS THRESHOLD 2 | 0x48 | RW | CIP DNS Threshold 2 |
| 0x5306 | CIP DNS OFFSET1 | 0x09 | RW | CIP DNS Offset1 (DNS threshold manual setting when |
|  |  |  |  | 0x5308[4]=1) |
| 0x5307 | CIP DNS OFFSET2 | 0x16 | RW | CIP DNS Offset2 |
| 0x5308 | CIP CTRL | 0x25 | RW | Bit[7]: Not used Bit[6]: CIP edge MT manual enable Bit[4]: CIP DNS manual enable |
|  |  |  |  | Bit[3]: Not used Bit[2:0]: CIP threshold for BR sharpen |

0x5309	CIP SHARPENTH THRESHOLD 1

0x08	RW	CIP Sharpen TH Threshold 1

| 0x530A | CIP SHARPENTH THRESHOLD 2 | 0x48 | RW | CIP Sharpen TH Threshold 2 |
| --- | --- | --- | --- | --- |
| 0x530B | CIP SHARPENTH OFFSET1 | 0x04 | RW | CIP Sharpen TH Offset1 (Sharpen threshold manual setting when |
|  |  |  |  | 0x5308[6]=1) |
| 0x530C | CIP SHARPENTH OFFSET2 | 0x06 | RW | CIP Sharpen TH Offset2 |
| 0x530D | CIP EDGE MT AUTO | – | R | CIP Edge MT Auto Read |

##### table 7-22	CIP control registers (sheet 2 of 2)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

0x530E	CIP DNS THRESHOLD

AUTO

0x530F	CIP SHARPEN THRESHOLD

AUTO

- R	CIP DNS Threshold Auto Read

- R	CIP Sharpen Threshold Auto Read

table 7-23	CMX control registers

CMX control [0x5380 ~ 0x538B]

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

| 0x5380 | CMX0 | 0x00 | RW | Bit[7:1]: Bit[0]: | Reserved gb_cbcr |
| --- | --- | --- | --- | --- | --- |
| 0x5381 | CMX1 | 0x20 | RW | Bit[7:0]: | CMX1 |
| 0x5382 | CMX2 | 0x64 | RW | Bit[7:0]: | CMX2 |
| 0x5383 | CMX3 | 0x08 | RW | Bit[7:0]: | CMX3 |
| 0x5384 | CMX4 | 0x30 | RW | Bit[7:0]: | CMX4 |
| 0x5385 | CMX5 | 0x90 | RW | Bit[7:0]: | CMX5 |
| 0x5386 | CMX6 | 0xC0 | RW | Bit[7:0]: | CMX6 |
| 0x5387 | CMX7 | 0xA0 | RW | Bit[7:0]: | CMX7 |
| 0x5388 | CMX8 | 0x98 | RW | Bit[7:0]: | CMX8 |
| 0x5389 | CMX9 | 0x08 | RW | Bit[7:0]: | CMX9 |
| 0x538A | CMXSIGN | 0x01 | RW | Cmxsign Bit[7:1]: | Reserved |
|  |  |  |  | Bit[0]: | CMX9 sign |
|  |  |  |  | Cmxsign Bit[7]: | CMX8 sign |
| 0x538B | CMXSIGN | 0x98 | RW | Bit[6]: Bit[5]: Bit[4]: Bit[3]: Bit[2]: Bit[1]: Bit[0]: | CMX7 sign CMX6 sign CMX5 sign CMX4 sign CMX3 sign CMX2 sign CMX1 sign |

  - gamma control [0x5480 ~ 0x5490]

##### table 7-24	gamma control registers

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

| 0x5480 | GAMMA CONTROL00 | 0x01 | RW | Bit[7:2]: Bit[1]: Bit[0]: | Reserved YSLP15 manual enable BIAS plus on |
| --- | --- | --- | --- | --- | --- |
| 0x5481 | GAMMA YST00 | 0x26 | RW | Bit[7:0]: | Y yst1 |
| 0x5482 | GAMMA YST01 | 0x35 | RW | Bit[7:0]: | Y yst2 |
| 0x5483 | GAMMA YST02 | 0x48 | RW | Bit[7:0]: | Y yst3 |
| 0x5484 | GAMMA YST03 | 0x63 | RW | Bit[7:0]: | Y yst4 |
| 0x5485 | GAMMA YST04 | 0x6E | RW | Bit[7:0]: | Y yst5 |
| 0x5486 | GAMMA YST05 | 0x77 | RW | Bit[7:0]: | Y yst6 |
| 0x5487 | GAMMA YST06 | 0x80 | RW | Bit[7:0]: | Y yst7 |
| 0x5488 | GAMMA YST07 | 0x88 | RW | Bit[7:0]: | Y yst8 |
| 0x5489 | GAMMA YST08 | 0x8F | RW | Bit[7:0]: | Y yst9 |
| 0x548A | GAMMA YST09 | 0x96 | RW | Bit[7:0]: | Y yst10 |
| 0x548B | GAMMA YST0A | 0xA3 | RW | Bit[7:0]: | Y yst11 |
| 0x548C | GAMMA YST0B | 0xAF | RW | Bit[7:0]: | Y yst12 |
| 0x548D | GAMMA YST0C | 0xC5 | RW | Bit[7:0]: | Y yst13 |
| 0x548E | GAMMA YST0D | 0xD7 | RW | Bit[7:0]: | Y yst14 |
| 0x548F | GAMMA YST0E | 0xE8 | RW | Bit[7:0]: | Y yst15 |
| 0x5490 | GAMMA YSLP15 | 0x0F | RW | Bit[7:0]: | Y yslp15 Slope of yst15 |

  - SDE control [0x5580 ~ 0x558C]

##### table 7-25	SDE control registers (sheet 1 of 2)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

Bit[7]:	Fixed Y enable

0:	Disable

1:	Enable Bit[6]:	Negative enable

0x5580

SDE CTRL0

0x00

RW

0:	Disable

1:	Enable Bit[5]:	Gray enable

0:	Disable

1:	Enable Bit[4]:	Fixed V enable

0:	Disable

1:	Enable Bit[3]:	Fixed U enable

0:	Disable

1:	Enable Bit[2]:	Contrast enable

0:	Disable

1:	Enable Bit[1]:	Saturation enable

0:	Disable

1:	Enable Bit[0]:	Hue enable

0:	Disable

1:	Enable

| 0x5581 | SDE CTRL1 | 0x80 | RW | Bit[7:0]: | Hue cos coefficient |
| --- | --- | --- | --- | --- | --- |
| 0x5582 | SDE CTRL2 | 0x00 | RW | Bit[7:0]: | Hue sin coefficient |
| 0x5583 | SDE CTRL3 | 0x40 | RW | Bit[7:0]: | Saturation U when 0x5580[1]=1 and 0x5588[6]=1, max value for auto color saturation adjust when 0x5580[1]=1 and 0x5588[6]=0; or fixed U when 0x5580[3]=1 |
| 0x5584 | SDE CTRL4 | 0x00 | RW | Bit[7:0]: | Saturation V when 0x5580[1]=1 and 0x5588[6]=1, min value for auto color saturation adjust when 0x5580[1]=1 and 0x5588[6]=0; or Vreg when 0x5580[4]=1 |
| 0x5585 | SDE CTRL5 | 0x00 | RW | Bit[7:0]: Y offset for contrast when 0x5044[3]=1; or fixed Y when 0x5580[7]=1 |  |
| 0x5586 | SDE CTRL6 | 0x20 | RW | Bit[7:0]: Y gain for contrast |  |
| 0x5587 | SDE CTRL7 | 0x00 | RW | Bit[7:0]: Y bright for contrast |  |

##### table 7-25	SDE control registers (sheet 2 of 2)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

|  |  |  |  | Bit[7]: | Reserved |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Bit[6]: | Auto color saturation adjust manual enable |
|  |  |  |  | Bit[5]: | Sign5 for hue V, cos |
|  |  |  |  | Bit[4]: | Sign4 for hue U, cos |
|  |  |  |  | Bit[3]: | Sign3 Y bright sign for contrast |
|  |  |  |  |  | 0: Keep Y bright sign |
| 0x5588 | SDE CTRL8 | 0x00 | RW |  | 1: Negative Y bright sign |
|  |  |  |  | Bit[2]: | Sign2 |
|  |  |  |  |  | Y offset sign for contrast when 0x5044[3]=1 |
|  |  |  |  | Bit[1]: Bit[0]: | 0: Keep Y offset sign 1: Negative Y offset sign Sign1 for hue V, sin Sign0 for hue U, sin |
| 0x5589 | SDE CTRL9 | 0x08 | RW Bit[7:0]: Auto color saturation adjust threshold 1 Valid when 0x5580[1]=1 |  |  |
| 0x558A | SDE CTRL10 | 0x80 | Bit[7:1]: Reserved RW Bit[0]: Auto color saturation adjust threshold 2[8] Valid when 0x5580[1]=1 |  |  |
| 0x558B | SDE CTRL11 | 0x00 | RW Bit[7:0]: Auto color saturation adjust threshold 2[7:0] Valid when 0x5580[1]=1 |  |  |
| 0x558C | SDE CTRL12 | – | R Bit[7:0]: Auto color saturation adjust value read out |  |  |

  - scale control [0x5600 ~ 0x5606]

##### table 7-26	scale control registers (sheet 1 of 2)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

0x5600	SCALE CTRL 0	0x10	RW

0x5601	SCALE CTRL 1	0x00	RW

Bit[7]:	Reserved

Bit[6]:	V first

YUV444 to 422 output U or V first 0:	U first

1:	V first

Bit[5]:	UV drop

YUV444 to 422 drop mode versus AVG mode selection

0:	AVG mode

1:	Drop mode Bit[4]:	Auto mode

Scale auto mode versus manual mode selection

0:	Manual mode

1:	Auto mode Bit[3]:	Horizontal round

DCW horizontal rounding

0:	No horizontal rounding 1:	Horizontal rounding

Bit[2]:	Horizontal drop

DCW horizontal drop mode

0:	Horizontal average mode 1:	Horizontal drop mode

Bit[1]:	Vertical round

DCW vertical rounding

0:	No vertical rounding 1:	Vertical rounding

Bit[0]:	Vertical drop

DCW vertical drop mode

0:	Vertical average mode 1:	Vertical drop mode

Bit[7]:	Reserved Bit[6:4]: Horizontal div

DCW scale times 000: DCW 1 time

001: DCW 2 time

010: DCW 4 time

011: DCW 8 time 1xx: DCW 16 time

Bit[2:0]: Vertical div

DCW scale times 000: DCW 1 time

001: DCW 2 time

010: DCW 4 time

011: DCW 8 time 1xx: DCW 16 time

##### table 7-26	scale control registers (sheet 2 of 2)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

| 0x5602 | SCALE CTRL 2 | 0x02 | RW | Bit[7:0]: | XSC[15:8] |
| --- | --- | --- | --- | --- | --- |
| 0x5603 | SCALE CTRL 3 | 0x00 | RW | Bit[7:0]: | XSC[7:0] |
| 0x5604 | SCALE CTRL 4 | 0x02 | RW | Bit[7:0]: | YSC[15:8] |
| 0x5605 | SCALE CTRL 5 | 0x00 | RW | Bit[7:0]: | YSC[7:0] |
| 0x5606 | SCALE CTRL 6 | 0x00 | RW | Bit[7:4]: Bit[3:0]: | Not used Vertical offset |

  - average control [0x5680 ~ 0x56A2]

##### table 7-27	AVG registers (sheet 1 of 3)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

0x5680	X START	0x00	RW

0x5681	X START	0x00	RW

0x5682	Y START	0x00	RW

0x5683	Y START	0x00	RW

0x5684	X WINDOW	0x10	RW

0x5685	X WINDOW	0xA0	RW

0x5686	Y WINDOW	0x0C	RW

Bit[7:4]: Reserved Bit[3:0]: X start[11:8]

Horizontal start position for average window high byte, valid when 0x501D[4]=1

Bit[7:0]: X start[7:0]

Horizontal start position for average window low byte, valid when 0x501D[4]=1

Bit[7:3]: Reserved Bit[2:0]: Y start[10:8]

Vertical start position for average window high byte, valid when 0x501D[4]=1

Bit[7:0]: Y start[7:0]

Vertical start position for average window low byte, valid when 0x501D[4]=1

Bit[7:4]: Reserved Bit[3:0]: Window X[11:8]

Horizontal end position for average window high byte, valid when 0x501D[4]=1

Bit[7:0]: Window X[7:0]

Horizontal end position for average window low byte, valid when 0x501D[4]=1

Bit[7:3]: Reserved Bit[2:0]: Window Y[10:8]

Vertical end position for average window high byte, valid when 0x501D[4]=1

##### table 7-27	AVG registers (sheet 2 of 3)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

|  |  |  |  | Bit[7:0]: | Window Y[7:0] |
| --- | --- | --- | --- | --- | --- |
| 0x5687 | Y WINDOW | 0x78 | RW Vertical end position for average window low byte, valid when 0x501D[4]=1 |  |  |
| 0x5688 | WEIGHT00 | 0x11 | RW Bit[7:4]: Window 01 weight Bit[3:0]: Window 00 weight |  |  |
| 0x5689 | WEIGHT01 | 0x11 | RW Bit[7:4]: Window 03 weight Bit[3:0]: Window 02 weight |  |  |
| 0x568A | WEIGHT02 | 0x11 | RW Bit[7:4]: Window 11 weight Bit[3:0]: Window 10 weight |  |  |
| 0x568B | WEIGHT03 | 0x11 | RW Bit[7:4]: Window 13 weight Bit[3:0]: Window 12 weight |  |  |
| 0x568C | WEIGHT04 | 0x11 | RW Bit[7:4]: Window 21 weight Bit[3:0]: Window 20 weight |  |  |
| 0x568D | WEIGHT05 | 0x11 | RW Bit[7:4]: Window 23 weight Bit[3:0]: Window 22 weight |  |  |
| 0x568E | WEIGHT06 | 0x11 | RW Bit[7:4]: Window 31 weight Bit[3:0]: Window 30 weight |  |  |
| 0x568F | WEIGHT07 | 0x11 | RW Bit[7:4]: Window 33 weight Bit[3:0]: Window 32 weight |  |  |
| 0x5690 | AVG CTRL10 | 0x01 | Bit[7:1]: Reserved RW Bit[0]: AVG option 0: Sum=(4*B+9*G*2+10*R)/8 1: Sum=(B+G*2+R)/4 |  |  |
| 0x5691 | AVG WIN 00 | – R |  | Bit[7:0]: | Average of win 00 |
| 0x5692 | AVG WIN 01 | – R |  | Bit[7:0]: | Average of win 01 |
| 0x5693 | AVG WIN 02 | – R |  | Bit[7:0]: | Average of win 02 |
| 0x5694 | AVG WIN 03 | – R |  | Bit[7:0]: | Average of win 03 |
| 0x5695 | AVG WIN 10 | – R |  | Bit[7:0]: | Average of win 10 |
| 0x5696 | AVG WIN 11 | – R |  | Bit[7:0]: | Average of win 11 |
| 0x5697 | AVG WIN 12 | – R |  | Bit[7:0]: | Average of win 12 |
| 0x5698 | AVG WIN 13 | – R |  | Bit[7:0]: | Average of win 13 |
| 0x5699 | AVG WIN 20 | – R |  | Bit[7:0]: | Average of win 20 |
| 0x569A | AVG WIN 21 | – R |  | Bit[7:0]: | Average of win 21 |
| 0x569B | AVG WIN 22 | – R |  | Bit[7:0]: | Average of win 22 |
| 0x569C | AVG WIN 23 | – R |  | Bit[7:0]: | Average of win 23 |

##### table 7-27	AVG registers (sheet 3 of 3)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

| 0x569D | AVG WIN 30 | – R | Bit[7:0]: | Average of win 30 |
| --- | --- | --- | --- | --- |
| 0x569E | AVG WIN 31 | – R | Bit[7:0]: | Average of win 31 |
| 0x569F | AVG WIN 32 | – R | Bit[7:0]: | Average of win 32 |
| 0x56A0 | AVG WIN 33 | – R | Bit[7:0]: | Average of win 33 |
| 0x56A1 | AVG READOUT | – R | Bit[7:0]: | Average value output |

0x56A2	AVG WEIGHT

SUM

–	R	Bit[7:0]: Average weight sum

  - DPC control [0x5780 ~ 0x5794]

##### table 7-28	DPC control registers (sheet 1 of 2)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

| 0x5780 | DPC CTRL00 | 0x14 | RW | DPC Control Register |
| --- | --- | --- | --- | --- |
| 0x5781 | DPC CTRL01 | 0x0F | RW | DPC Control Register |
| 0x5782 | DPC CTRL02 | 0x04 | RW | DPC Control Register |
| 0x5783 | DPC CTRL03 | 0x02 | RW | DPC Control Register |
| 0x5784 | DPC CTRL04 | 0x01 | RW | DPC Control Register |
| 0x5785 | DPC CTRL05 | 0x01 | RW | DPC Control Register |
| 0x5786 | DPC CTRL06 | 0x00 | RW | DPC Control Register |
| 0x5787 | DPC CTRL07 | 0x04 | RW | DPC Control Register |
| 0x5788 | DPC CTRL08 | 0x0C | RW | DPC Control Register |
| 0x5789 | DPC CTRL09 | 0x00 | RW | DPC Control Register |
| 0x578A | DPC CTRL0A | 0x01 | RW | DPC Control Register |
| 0x578B | DPC CTRL0B | 0x02 | RW | DPC Control Register |
| 0x578C | DPC CTRL0C | 0x03 | RW | DPC Control Register |
| 0x578D | DPC CTRL0D | 0x03 | RW | DPC Control Register |
| 0x578E | DPC CTRL0E | 0x0F | RW | DPC Control Register |
| 0x578F | DPC CTRL0F | 0x3F | RW | DPC Control Register |
| 0x5790 | DPC CTRL10 | 0x08 | RW | DPC Control Register |

##### table 7-28	DPC control registers (sheet 2 of 2)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

| 0x5791 | DPC CTRL11 | 0x04 | RW | DPC Control Register |
| --- | --- | --- | --- | --- |
| 0x5792 | DPC CTRL12 | 0x04 | RW | DPC Control Register |
| 0x5793 | DPC CTRL13 | 0x00 | RW | DPC Control Register |
| 0x5794 | DPC CTRL14 | 0x03 | RW | DPC Control Register |

7.29 LENC control [0x5800 ~ 0x5849]

table 7-29	LENC control registers (sheet 1

| 0x5800 | GMTRX00 | 0x10 | RW Bit[7:6]: Reserved Bit[5:0]: Green matrix 00 |
| --- | --- | --- | --- |
| 0x5801 | GMTRX01 | 0x10 | RW Bit[7:6]: Reserved Bit[5:0]: Green matrix 01 |
| 0x5802 | GMTRX02 | 0x10 | RW Bit[7:6]: Reserved Bit[5:0]: Green matrix 02 |
| 0x5803 | GMTRX03 | 0x10 | RW Bit[7:6]: Reserved Bit[5:0]: Green matrix 03 |
| 0x5804 | GMTRX04 | 0x10 | RW Bit[7:6]: Reserved Bit[5:0]: Green matrix 04 |
| 0x5805 | GMTRX05 | 0x10 | RW Bit[7:6]: Reserved Bit[5:0]: Green matrix 05 |
| 0x5806 | GMTRX10 | 0x10 | RW Bit[7:6]: Reserved Bit[5:0]: Green matrix 06 |
| 0x5807 | GMTRX11 | 0x08 | RW Bit[7:6]: Reserved Bit[5:0]: Green matrix 07 |
| 0x5808 | GMTRX12 | 0x08 | RW Bit[7:6]: Reserved Bit[5:0]: Green matrix 08 |
| 0x5809 | GMTRX13 | 0x08 | RW Bit[7:6]: Reserved Bit[5:0]: Green matrix 09 |
| 0x580A | GMTRX14 | 0x08 | RW Bit[7:6]: Reserved Bit[5:0]: Green matrix 0A |
| 0x580B | GMTRX15 | 0x10 | RW Bit[7:6]: Reserved |

of 6)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

Bit[5:0]: Green matrix 0B

##### table 7-29	LENC control registers (sheet 2 of 6)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

| 0x580C | GMTRX20 | 0x10 | RW Bit[7:6]: Reserved Bit[5:0]: Green matrix 0C |
| --- | --- | --- | --- |
| 0x580D | GMTRX21 | 0x08 | RW Bit[7:6]: Reserved Bit[5:0]: Green matrix 0D |
| 0x580E | GMTRX22 | 0x00 | RW Bit[7:6]: Reserved Bit[5:0]: Green matrix 0E |
| 0x580F | GMTRX23 | 0x00 | RW Bit[7:6]: Reserved Bit[5:0]: Green matrix 0F |
| 0x5810 | GMTRX24 | 0x08 | RW Bit[7:6]: Reserved Bit[5:0]: Green matrix 10 |
| 0x5811 | GMTRX25 | 0x10 | RW Bit[7:6]: Reserved Bit[5:0]: Green matrix 11 |
| 0x5812 | GMTRX30 | 0x10 | R/W Bit[7:6]: Reserved Bit[5:0]: Green matrix 12 |
| 0x5813 | GMTRX31 | 0x08 | R/W Bit[7:6]: Reserved Bit[5:0]: Green matrix 13 |
| 0x5814 | GMTRX32 | 0x00 | R/W Bit[7:6]: Reserved Bit[5:0]: Green matrix 14 |
| 0x5815 | GMTRX33 | 0x00 | R/W Bit[7:6]: Reserved Bit[5:0]: Green matrix 15 |
| 0x5816 | GMTRX34 | 0x08 | R/W Bit[7:6]: Reserved Bit[5:0]: Green matrix 16 |
| 0x5817 | GMTRX35 | 0x10 | R/W Bit[7:6]: Reserved Bit[5:0]: Green matrix 17 |
| 0x5818 | GMTRX40 | 0x10 | R/W Bit[7:6]: Reserved Bit[5:0]: Green matrix 18 |
| 0x5819 | GMTRX41 | 0x08 | R/W Bit[7:6]: Reserved Bit[5:0]: Green matrix 19 |
| 0x581A | GMTRX42 | 0x08 | R/W Bit[7:6]: Reserved Bit[5:0]: Green matrix 1A |
| 0x581B | GMTRX43 | 0x08 | R/W Bit[7:6]: Reserved Bit[5:0]: Green matrix 1B |
| 0x581C | GMTRX44 | 0x08 | R/W Bit[7:6]: Reserved Bit[5:0]: Green matrix 1C |
| 0x581D | GMTRX45 | 0x10 | R/W Bit[7:6]: Reserved Bit[5:0]: Green matrix 1D |
| 0x581E | GMTRX50 | 0x10 | R/W Bit[7:6]: Reserved |

Bit[5:0]: Green matrix 1E

##### table 7-29	LENC control registers (sheet 3 of 6)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

| 0x581F | GMTRX51 | 0x10 | R/W Bit[7:6]: Reserved Bit[5:0]: Green matrix 1F |
| --- | --- | --- | --- |
| 0x5820 | GMTRX52 | 0x10 | R/W Bit[7:6]: Reserved Bit[5:0]: Green matrix 20 |
| 0x5821 | GMTRX53 | 0x10 | R/W Bit[7:6]: Reserved Bit[5:0]: Green matrix 21 |
| 0x5822 | GMTRX54 | 0x10 | R/W Bit[7:6]: Reserved Bit[5:0]: Green matrix 22 |
| 0x5823 | GMTRX55 | 0x10 | R/W Bit[7:6]: Reserved Bit[5:0]: Green matrix 23 |
| 0x5824 | BRMATRX00 | 0xAA | R/W Bit[7:4]: Blue matrix 00 Bit[3:0]: Red matrix 00 |
| 0x5825 | BRMATRX01 | 0xAA | RW Bit[7:4]: Blue matrix 01 Bit[3:0]: Red matrix 01 |
| 0x5826 | BRMATRX02 | 0xAA | RW Bit[7:4]: Blue matrix 02 Bit[3:0]: Red matrix 02 |
| 0x5827 | BRMATRX03 | 0xAA | RW Bit[7:4]: Blue matrix 03 Bit[3:0]: Red matrix 03 |
| 0x5828 | BRMATRX04 | 0xAA | RW Bit[7:4]: Blue matrix 04 Bit[3:0]: Red matrix 04 |
| 0x5829 | BRMATRX05 | 0xAA | RW Bit[7:4]: Blue matrix 05 Bit[3:0]: Red matrix 05 |
| 0x582A | BRMATRX06 | 0x99 | RW Bit[7:4]: Blue matrix 06 Bit[3:0]: Red matrix 06 |
| 0x582B | BRMATRX07 | 0x99 | RW Bit[7:4]: Blue matrix 07 Bit[3:0]: Red matrix 07 |
| 0x582C | BRMATRX08 | 0x99 | RW Bit[7:4]: Blue matrix 08 Bit[3:0]: Red matrix 08 |
| 0x582D | BRMATRX09 | 0xAA | RW Bit[7:4]: Blue matrix 09 Bit[3:0]: Red matrix 09 |
| 0x582E | BRMATRX20 | 0xAA | RW Bit[7:4]: Blue matrix 20 Bit[3:0]: Red matrix 20 |
| 0x582F | BRMATRX21 | 0x99 | RW Bit[7:4]: Blue matrix 21 Bit[3:0]: Red matrix 21 |
| 0x5830 | BRMATRX22 | 0x88 | RW Bit[7:4]: Blue matrix 22 Bit[3:0]: Red matrix 22 |
| 0x5831 | BRMATRX23 | 0x99 | RW Bit[7:4]: Blue matrix 23 |

Bit[3:0]: Red matrix 23

##### table 7-29	LENC control registers (sheet 4 of 6)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

| 0x5832 | BRMATRX24 | 0xAA | RW Bit[7:4]: Blue matrix 24 Bit[3:0]: Red matrix 24 |  |  |
| --- | --- | --- | --- | --- | --- |
| 0x5833 | BRMATRX30 | 0xAA | RW Bit[7:4]: Blue matrix 30 Bit[3:0]: Red matrix 30 |  |  |
| 0x5834 | BRMATRX31 | 0x99 | RW Bit[7:4]: Blue matrix 31 Bit[3:0]: Red matrix 31 |  |  |
| 0x5835 | BRMATRX32 | 0x99 | RW Bit[7:4]: Blue matrix 32 Bit[3:0]: Red matrix 32 |  |  |
| 0x5836 | BRMATRX33 | 0x99 | RW Bit[7:4]: Blue matrix 33 Bit[3:0]: Red matrix 33 |  |  |
| 0x5837 | BRMATRX34 | 0xAA | RW Bit[7:4]: Blue matrix 34 Bit[3:0]: Red matrix 34 |  |  |
| 0x5838 | BRMATRX40 | 0xAA | R/W Bit[7:4]: Blue matrix 40 Bit[3:0]: Red matrix 40 |  |  |
| 0x5839 | BRMATRX41 | 0xAA | R/W Bit[7:4]: Blue matrix 41 Bit[3:0]: Red matrix 41 |  |  |
| 0x583A | BRMATRX42 | 0xAA | R/W Bit[7:4]: Blue matrix 42 Bit[3:0]: Red matrix 42 |  |  |
| 0x583B | BRMATRX43 | 0xAA | R/W Bit[7:4]: Blue matrix 43 Bit[3:0]: Red matrix 43 |  |  |
| 0x583C | BRMATRX44 | 0xAA | R/W Bit[7:4]: Blue matrix 44 Bit[3:0]: Red matrix 44 |  |  |
| 0x583D | LENC BR OFFSET | 0x88 | R/W Bit[7:4]: LENC b offset Bit[3:0]: LENC r offset |  |  |
| 0x583E | MAX GAIN | 0x40 | R/W Bit[7:0]: Maximum gain |  |  |
| 0x583F | MIN GAIN | 0x20 | R/W Bit[7:0]: Minimum gain |  |  |
| 0x5840 | MIN Q | 0x18 | R/W Bit[7]: Reserved Bit[6:0]: Minimum Q |  |  |
| 0x5841 | LENC CTRL59 | 0x0D | R/W | Bit[7:4]: Bit[3]: Bit[2]: | Reserved Add BLC enable 0: Disable BLC add back function 1: Enable BLC add back function BLC enable 0: Disable BLC function 1: Enable BLC function |

Bit[1]:	Reserved Bit[0]:	 Auto Q enable

0:	Used constant Q (0x40) 1:	Used calculated Q

##### table 7-29	LENC control registers (sheet 5 of 6)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

|  |  |  |  | Bit[7:3]: Bit[2:0]: | Reserved BR H scale[10:8] Reciprocal of horizontal step for BR |
| --- | --- | --- | --- | --- | --- |
| 0x5842 | BR HSCALE | 0x00 | RW |  | channel. BR channel in whole image |
|  |  |  |  |  | is divided into 5x5 blocks. The step |
|  |  |  |  |  | is used to point to the border of the |
|  |  |  |  |  | adjacent block |
|  |  |  |  | Bit[7:0]: | BR H scale[7:0] |
|  |  |  |  |  | Reciprocal of horizontal step for BR |
| 0x5843 | BR HSCAL | 0xBD | RW |  | channel. BR channel in whole image is divided into 5x5 blocks. The step |
|  |  |  |  |  | is used to point to the border of the |
|  |  |  |  |  | adjacent block |
|  |  |  |  | Bit[7:3]: | Reserved |
|  |  |  |  | Bit[2:0]: | BR V scale[10:8] |
|  |  |  |  |  | Reciprocal of vertical step for BR |
| 0x5844 | BR VSCALE | 0x00 | RW |  | channel. BR channel in whole image |
|  |  |  |  |  | is divided into 5x5 blocks. The step |
|  |  |  |  |  | is used to point to the border of the |
|  |  |  |  |  | adjacent block |
|  |  |  |  | Bit[7:0]: | BR V scale[7:0] |
|  |  |  |  |  | Reciprocal of vertical step for BR |
| 0x5845 | BR VSCALE | 0xFE | RW |  | channel. BR channel in whole image is divided into 5x5 blocks. The step |
|  |  |  |  |  | is used to point to the border of the |
|  |  |  |  |  | adjacent block |
|  |  |  |  | Bit[7:3]: | Reserved |
|  |  |  |  | Bit[2:0]: | G H scale[10:8] |
|  |  |  |  |  | Reciprocal of horizontal step for G |
| 0x5846 | G HSCALE | 0x00 | RW |  | channel. G channel in whole image |
|  |  |  |  |  | is divided into 6x6 blocks. The step |
|  |  |  |  |  | is used to point to the border of the |
|  |  |  |  |  | adjacent block |
|  |  |  |  | Bit[7:0]: | G H scale[7:0] |
|  |  |  |  |  | Reciprocal of horizontal step for G |
| 0x5847 | G HSCAL | 0xFC | RW |  | channel. G channel in whole image is divided into 6x6 blocks. The step |
|  |  |  |  |  | is used to point to the border of the |
|  |  |  |  |  | adjacent block |
|  |  |  |  | Bit[7:3]: | Reserved |
|  |  |  |  | Bit[2:0]: | G V scale[10:8] |
|  |  |  |  |  | Reciprocal of vertical step for G |
| 0x5848 | G VSCALE | 0x00 | RW |  | channel. G channel in whole image |
|  |  |  |  |  | is divided into 6x6 blocks. The step |
|  |  |  |  |  | is used to point to the border of the |
|  |  |  |  |  | adjacent block |

##### table 7-29	LENC control registers (sheet 6 of 6)

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

0x5849	G VSCALE	0xA9	RW

Bit[7:0]: G V scale[7:0]

7.30  temperature sensor control [0x6700 ~ 0x6721]

table 7-30	TPM control registers

0x6700~

0x6705

TPM CTRL00~05

–

–

Temperature Sensor Control Registers

Bit[7:4]:

Bit[3:0]:

0x6706	TPM CTRL06

0x78

RW

Reserved

Sample clock must be around 3MHz

If the user has 24 MHz XVCLK, sample clock = XVCLK/clock divisor => 3MHz = 24MHz / 8 (these 4 bits must be 4’b1000) If the user has 12 MHz XVCLK, sample clock = XVCLK/clock divisor => 3MHz = 12MHz / 4 (these 4 bits must be 4’b0100)

0x6707~

0x6718

TPM CTRL09~18

–

–

Temperature Sensor Control Registers

0x6719	TPM CTRL19

–

R

Bit[7:0]: Measured temperature

0x671A~

0x6721

TPM CTRL1A~21

–

–

Temperature Sensor Control Registers

Reciprocal of vertical step for G channel. G channel in whole image is divided into 6x6 blocks. The step is used to point to the border of the adjacent block

| address | register name | default value | R/W | description |
| --- | --- | --- | --- | --- |

- operating specifications

## absolute maximum ratings

- exceeding the absolute maximum ratings shown above invalidates all AC and DC electrical specifications and may result in permanent damage to the device. Exposure to absolute maximum rated conditions for extended periods may affect device reliability.

- for negative voltage with respect to ground, VDD-A (-4.5V), VDD-C (-3V), VDD-IO (-4.5V)

  - functional temperature

table 8-2

functional temperature

operating temperaturea

stable image temperatureb

-20°C to +70°C junction temperature

0°C to +50°C junction temperature

a.	sensor functions but image quality may be noticeably different at temperatures outside of stable image range

range

parameter

table 8-1	absolute maximum ratings

| parameter |  | absolute maximum ratinga |
| --- | --- | --- |
| ambient storage temperature |  | -40°C to +95°C |
|  | VDD-A | 4.5V |
| supply voltage (with respect to ground)b | VDD-D | 3V |
|  | VDD-IO | 4.5V |
| electro-static discharge (ESD) | human body model machine model | 2000V 200V |
| all input/output voltages (with respect to ground) |  | -0.3V to VDD-IO + 1V |
| I/O current on any input or output pin |  | ±200 mA |
| peak solder temperature (10 second dwell time) |  | 245°C |

b.	image quality remains stable throughout this temperature range

## DC characteristics

IDD-A

28

mA

active (operating) current

DDS-SCCB

PDDS-SCCB

PDDS-PWDN

standby power consumption

TBD

TBD

µW

µW

IDD-A IDD-Db, c IDD-IO

IDDS-SCCB

active (operating) current

28

64

6

40

mA mA mA

µA

PDDS-SCCB

PDDS-PWDN

standby power consumption

TBD

TBD

µW

µW

IDD-A

IDD-Db, c

active (operating) current

28

64

mA

mA

external DVDD, DOVDD=1.8V

external DVDD, DOVDD=2.8V

internal DVDD, DOVDD=1.8V

table 8-3	DC characteristics (-20°C < TJ < 70°C) (sheet 1 of 2)

| symbol | parameter | min | typ | max | unit |
| --- | --- | --- | --- | --- | --- |
| power supply |  |  |  |  |  |

| VDD-A supply voltage (analog) | 2.6 | 2.8 | 3.0 | V |
| --- | --- | --- | --- | --- |
| VDD-D a supply voltage (digital core) | 1.425 | 1.5 | 1.575 | V |
| VDD-IO supply voltage (digital I/O) | 1.71 | 1.8 | 3.0 | V |

| IDD-IOb, c | 70 |  | mA |  |
| --- | --- | --- | --- | --- |
| I d | 20 standby current |  | µA |  |
| IDDS-PWDNd |  | 20 |  | µA |
| PO | active (operating) power consumption |  | TBD | mW |

|  | standby current |  |  |  |
| --- | --- | --- | --- | --- |
| IDDS-PWDN |  | 40 |  | µA |
| PO | active (operating) power consumption |  | TBD | mW |

IDD-IO	4	mA

|  | standby current |  |  |  |
| --- | --- | --- | --- | --- |
| IDDS-PWDN |  | 40 |  | µA |
| PO | active (operating) power consumption |  | TBD | mW |

IDDS-SCCB

40	µA

PDDS-SCCB	standby power consumptione	TBD	µW

PDDS-PWDN	TBD	µW

##### table 8-3	DC characteristics (-20°C < TJ < 70°C) (sheet 2 of 2)

| symbol | parameter | min | typ | max | unit |
| --- | --- | --- | --- | --- | --- |
| digital inputs (typical conditions: AVDD = 2.8V, DVDD = 1.5V, DOVDD = 1.8V) |  |  |  |  |  |

| VIL | input voltage LOW |  | 0.54 |  | V |
| --- | --- | --- | --- | --- | --- |
| VIH | input voltage HIGH | 1.26 |  |  | V |
| CIN | input capacitor |  | 10 |  | pF |
| digital outputs (standard loading 25 pF) |  |  |  |  |  |
| VOH | output voltage HIGH | 1.62 |  |  | V |
| VOL | output voltage LOW |  | 0.18 |  | V |
| serial interface inputsf |  |  |  |  |  |
| VIL | SIOC and SIOD | -0.5 | 0 | 0.54 | V |
| VIH | SIOC and SIOD | 1.26 | 1.8 | 3.0 | V |

- using the internal DVDD regulator is strongly recommended for minimum power down current

- active current is based on sensor resolution at full size and at full speed in compression format. For smaller sizes such as 720p or below preview, the total active current will be about half.

- DOVDD active current is based on loading of 10pF and typical compression format output PCLK (48MHz). For YUV output with higher PCLK, or higher loading, DOVDD current can go up.

- at room temperature and typical supply voltages

- standby current is measured at room temperature

- based on DOVDD = 1.8V.

## timing characteristics

##### table 8-4	timing characteristics

| symbol | parameter | min | typ | max | unit |
| --- | --- | --- | --- | --- | --- |
| oscillator and clock input |  |  |  |  |  |

| fOSC | frequency (XVCLK)a | 6 | 24 | 54 | MHz |
| --- | --- | --- | --- | --- | --- |
| tr, tf | clock input rise/fall timeb |  |  | 5 (10c) | ns |
| fPCLK | parallel port output pixel clock |  | 54 | 108 | MHz |

    - for input clock range 6~27MHz, the OV3660 can tolerate input clock jitter up to 1ns, for input clock range to 54MHz, the OV3660 can tolerate input clock jitter up to 500ps

    - if the PLL is bypassed, the delay from input clock to output clock is approximately 4~5ns

    - if using the internal PLL

# mechanical specifications

## physical specifications

S2

1 2 3 4 5 6 7 8

8 7 6 5 4 3 2 1

A B C D E F G H

J2

B

WX Y Z A B CD

A B

- center of BGA (die) =

- center of the package E

F G H

D

C2

top view

(bumps down)

glass

A

J1

die

C3

S1

bottom view

(bumps up)

C1

C4

C

side view

3660_CSP_DS_9_1

table 9-1

package dimensions

figure 9-1	package specifications

| parameter | symbol | min | typ | max | unit |
| --- | --- | --- | --- | --- | --- |

| package body dimension x | A | 4985 | 5010 | 5035 | µm |
| --- | --- | --- | --- | --- | --- |
| package body dimension y | B | 4935 | 4960 | 4985 | µm |
| package height | C | 690 | 750 | 810 | µm |
| ball height | C1 | 100 | 130 | 160 | µm |
| package body thickness | C2 | 575 | 620 | 665 | µm |
| cover glass thickness | C3 | 425 | 445 | 465 | µm |
| ball diameter | D | 220 | 250 | 280 | µm |
| total pin count | N |  | 51 (7 NC) |  |  |
| pin count x-axis | N1 |  | 8 |  |  |
| pin count y-axis | N2 |  | 8 |  |  |
| pins pitch x-axis | J1 |  | 580 |  | µm |
| pins pitch y-axis | J2 |  | 600 |  | µm |
| edge-to-pin center distance along x | S1 | 445 | 475 | 505 | µm |
| edge-to-pin center distance along y | S2 | 350 | 380 | 410 | µm |

-22

-2

18

38

58

78

98

118

138

158

178

198

218

238

258

278

298

318

338

358

369

## IR reflow specifications

note

The OV3660 uses a lead-free package.

temperature (°C)

figure 9-2	IR reflow ramp rate requirements

300.0

280.0

260.0

240.0

220.0

200.0

180.0

160.0

140.0

120.0

100.0

80.0

60.0

40.0

20.0

0.0

Z1

Z2

Z3

Z4

Z5	Z6

Z7

end

time (sec)

3660_DS_9_2

table 9-2

reflow conditions

average ramp-up rate (30°C to 217°C)

less than 3°C per second

> 100°C

between 330 - 600 seconds

> 150°C

at least 210 seconds

> 217°C

at least 30 seconds (30 ~ 120 seconds)

peak temperature

245°C

cool-down rate (peak to 50°C)

less than 6°C per second

exposure

condition

time from 30°C to 245°C	no greater than 390 seconds

# optical specifications

## sensor array center

##### figure 10-1	sensor array center

A1	A2	A3	A4	A5	A6	A7	A8

2167.2 μm

sensor array

OV3660

top view

note 1 this drawing is not to scale and is for reference only.

note 2 as most optical assemblies invert and mirror the image, the chip is typically mounted with pin A1 to A8 oriented down on the PCB.

2912 μm

first pixel readout

(-1448 μm, 1222.6 μm)

array center

(8 μm, 139 μm)

package center (0 μm, 0 μm)

3660_CSP_DS_10_1

0.000

0.090

0.180

0.270

0.360

0.450

0.540

0.630

0.720

0.810

0.900

0.990

1.080

1.160

1.250

1.340

1.430

1.520

1.610

1.700

1.790

  - lens chief ray angle (CRA)

30.0

25.0

20.0

15.0

10.0

5.0

CRA

0.0

image height (mm)

3660_DS_10_2

table 10-1	CRA versus image height plot (sheet 1 of 2)

CRA (degrees)

figure 10-2	chief ray angle (CRA)

| field (%) |  | image height (mm) | CRA (degrees) |
| --- | --- | --- | --- |
| 0 | 0 |  | 0 |
| 0.05 | 0.090 |  | 2.4 |
| 0.1 | 0.180 |  | 4.7 |
| 0.15 | 0.270 |  | 7.0 |
| 0.2 | 0.360 |  | 9.3 |
| 0.25 | 0.450 |  | 11.5 |
| 0.3 | 0.540 |  | 13.7 |
| 0.35 | 0.630 |  | 15.7 |
| 0.4 | 0.720 |  | 17.7 |
| 0.45 | 0.810 |  | 19.6 |
| 0.5 | 0.900 |  | 21.3 |
| 0.55 | 0.990 |  | 22.8 |

10.3 spectrum response

figure 10-3	spectrum response graph

R

Gr Gb B

##### table 10-1	CRA versus image height plot (sheet 2 of 2)

| field (%) | image height (mm) | CRA (degrees) |
| --- | --- | --- |

| 0.6 | 1.080 | 24.2 |
| --- | --- | --- |
| 0.65 | 1.160 | 25.3 |
| 0.7 | 1.250 | 26.2 |
| 0.75 | 1.340 | 26.8 |
| 0.8 | 1.430 | 27.3 |
| 0.85 | 1.520 | 27.5 |
| 0.9 | 1.610 | 27.6 |
| 0.95 | 1.700 | 27.6 |
| 1 | 1.790 | 27.6 |

| 300 | 400 | 500 | 600 | 700 | 800 | 900 | 1000 | 1100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | wavelength (nm) |  |  |  | 3660_DS_10_3 |

revision history

###### version 1.0	09.23.2010

- initial release

###### version 1.1	12.16.2010

- on page iii, under key specifications, added "junction temperature" to temperature range and removed well capacity

- on page iii, under key specifications, changed image area from "2080 µm x 1548 µm" to "2912 µm x 2167.2 µm"

- in section 3, updated figure 3-1

- in table 8-2, added "junction temperature" to range

- in section 10, updated figure 10-1

###### version 1.2	01.31.2011

- in features section on page i, removed "support for video or snapshot operations", "support for internal and external frame synchronization for frame exposure mode", and "support for minimizing artifacts on binned image"

- in features section on page i, added "support 2x2 binning with binning filter to remove binning artifacts"

- in key specifications section on page i, added "(1.8V recommended)" to I/O power supply specification

- in key specifications section on page i, removed "/ frame exposure" from shutter specification

- moved section 3, pixel array structure to section 2.3

- moved section 2.3, format and frame rate to section 2.7

- moved section 2.4, I/O control to section 2.8

- moved section 2.5, system clock control to section 2.9

- moved section 2.6, SCCB interface to chapter 6 and added subsections for data transfer protocol, message format, read/write operation, and SCCB timing

- moved table 2-6, group write registers to new section 2.10, group write

- changed chapter 4, image sensor core digital functions (with all subsections) to chapter 3

- in new chapter 3, added section 3.2.2, scaling

- in new chapter 3, renamed AEC/AGC algorithms section to section 3.4, exposure/gain control and completely re-wrote the section

- removed AEC/AGC steps section (previously section 4.5)

- moved section 4.6, black level calibration to section 3.5

- removed light frequency selection (previously section 4.7) and merged content into new section 3.4

- removed digital gain (previously section 4.8) and merged content into new section 3.4

- moved section 4.9, strobe flash to section 3.6

  - moved section 4.10, one time programmable (OTP) memory to section 3.7 and completely re-wrote the section

  - moved section 4.11, temperature sensor to section 3.8 and completely re-wrote the section

  - changed chapter 5, image sensor processor digital functions (with all subsections) to chapter 4

  - moved section 5.6 to section 4.6, renamed section to "color interpolation (CIP) and sharpening", and re-wrote first paragraph

  - removed (previously numbered) section 5.8, UV average and (previously numbered) section 5.9, scaling

  - moved section 5.10, UV adjust to section 4.8 and renamed section to "auto color saturation adjust"

  - moved section 5.11, special digital effects (SDE) to section 4.9

  - removed (previously numbered) section 5.12, ISP format and (previously numbered) section 5.13, average

  - changed chapter 6, image sensor output interface digital functions (with all subsections) to chapter 5

  - removed (previously numbered) section 6.2, system control and (previously numbered) section 6.3, format description

  - in chapter 7, added subsection number and title for each register table

  - renamed section 8.4 from "AC characteristics" to "timing characteristics" and removed AC characteristics table and SCCB timing figure and table

  - in chapter 10, added section 10.3, spectrum response

  - on page iii, under key specifications, added "junction temperature" to temperature range and removed well capacity

  - on page iii, under key specifications, changed image area from "2080 µm x 1548 µm" to "2912 µm x 2167.2 µm"

  - in section 3, updated figure 3-1

  - in table 8-2, added "junction temperature" to range

  - in section 10, updated figure 10-1

###### version 1.21	04.06.2011

  - in chapter 8, updated table 8-3 title to DC characteristics (-20°C < TJ < 70°C)

  - in chapter 8, table 8-3 added a table footnote to standby current

###### version 1.3	05.13.2011

  - re-wrote section 4.3, white balance; section 4.4, gamma; section 4.6, sharpness and de-noise; section 4.7, color matrix (CMX)

the clear advantage™

OmniVision Technologies, Inc.

UNITED STATES

4275 Burton Drive Santa Clara, CA 95054

tel: + 1 408 567 3000

fax: + 1 408 567 3001

email: salesamerican@ovt.com

UNITED KINGDOM

Hampshire + 44 1256 744 610

FINLAND

Mouhijärvi + 358 3 341 1898

GERMANY

Munich +49 89 63 81 99 88

CHINA

Beijing + 86 10 6580 1690

Shanghai + 86 21 6175 9888

Shenzhen + 86 755 8384 9733

Hong Kong + 852 2403 4011

JAPAN

Tokyo + 81 3 5765 6321

KOREA

Seoul + 82 2 3478 2812

SINGAPORE + 65 6562 8250

TAIWAN

Taipei + 886 2 2657 9800 -

ext.#100

website: www.ovt.com